- AIChE/DIPPR® Project ESP

- Environmental and Safety Properties

- Policy and Procedures Guidelines

June 2008

- Rogers, T.N.; Zei, D.A.

Table of contents

1.0  Introduction	5

1.1  Project Approach	5

2.0  Project ESP Parameters	6

2.1  List of Parameters	6

2.2  Definitions of Parameters	6

2.2.1  Block 1 Oxygen Demand	6

2.2.2  Block 2 Partitioning Parameters	7

2.2.3  Block 3 General Physical Properties	8

2.2.4  Block 4 Vapor-Liquid Equilibrium	10

2.2.5  Block 5 Fire and Explosion Parameters	11

2.2.6  Block 6 Sensory, Health and Toxicity Impacts	11

2.2.7  Units	12

3.0  Description of Chemical List	13

4.0  Literature Search	13

4.1  General Approach	13

4.2  Project 801 Release	14

4.3  Principal Literature Search	14

4.3.1  Blocks 1 and 2 Oxygen Demand and Soil/Water Partitioning	14

4.3.2  Block 3 General Physical Properties	16

4.3.3  Block 4 Vapor-Liquid Equilibrium	16

4.3.4  Block 5 Fire and Explosions	17

4.3.5  Block 6 Sensory, Health & Toxicity Impacts	17

5.0  Evaluation Criteria	17

5.1  Quality Assurance	17

5.2  Qualitative Review	18

5.3  Quantitative Reviews	19

5.3.1  Statistical Quality Control (SQC)	19

5.3.2  Classification of Errors Using SQC	20

6.0  Data Tracking and Management	21

6.1  General Overview	21

6.2  Datafiles Description	21

6.3  Data Entry System	22

7.0  Temperature Dependent Properties	22

8.0  Estimation Methods	23

8.1  Biochemical Oxygen Demand	23

8.2  Chemical Oxygen Demand	23

8.3  Theoretical Oxygen Demand	24

8.4  Partitioning/Equilibrium Parameters	24

8.4.1  Solubility of a Chemical in Water	25

8.4.2  Solubility of Water in a Chemical	25

8.4.3  1-Octanol/Water Partition Coefficient (Kow)	25

8.4.4  Soil/Water Partitioning	26

8.4.5  Bioconcentration Factor	26

8.4.6  Activity Coefficients	26

8.4.7  Henry's Law Constant	26

9.0  Database Software	26

9.1  Description	26

9.2  Software Tables	27

Table 1:  Property List	28

Table 2:  Default Units and Mathematical Expressions to Convert to Unit Options	31

Table 3 :  Chemical List	35

Table 4:  Principal Literature Sources	58

Table 5:  BOD Rating Criteria	66

Table 6:  Evaluation Criteria for Soil/Water Partition Coefficient	67

Table 7:  Some Reliable References and Criteria for Soil/Water Partitioning	68

Table 8:  Criteria for Flagging Data Entry Errors and Significant Outliers	69

Table 9:  Evaluation Criteria for Block 3	70

Table 10:  Evaluation Criteria for Block 6	70

Table 11:  Translation of DIPPR® 801 Evaluation Codes to DIPPR® ESP Evaluation

Codes	71

Table 12: Results of SQC Review	72

Table 13:  Structure for Values Table	73

Table 14:  Structure of Citation Table	74

Table 15:  Structure of Property Table	74

Table 16:  Structure of Chemical Table	75

Table 17:  Structure of Convunit Table	75

Table 18:  Project 912 Estimation Methods 	76

Table 19:  Family Grouping Codes	77

Figure 1:  DIPPR® ESP Tracking Form	78

Figure 2:  Citation Formats	79

Figure 3:  Data Entry Form	80

Figure 4:  Acute Aquatic Toxicity Entry Format	81

- Policy and Procedures Guideline

- 1.0  Introduction

Design Institute for Physical Property Data (DIPPR)® Project ESP entitled Environmental and Safety Properties is responding to a need for a critically evaluated, internally consistent, documented and computerized database covering the title properties for a select list of pure chemicals with known hazards.  The project is anticipated to satisfy industrial needs for more accurate and complete property data in order to respond effectively to regulatory requirements.  The project scope was developed around the 275 chemicals in the Clean Air Act Amendments of 1990 (CAAA) and the Occupational Safety and Health (OSHA) Act of 1970, Federal Register No. 1910.119 with six blocks of properties covering specific areas of interest.  In 1994, the 84 chemicals of the Environmental Protection Agency's Risk Management Program (EPA-RMP) were added to the project chemical list.  The updates to the chemical list from 1994 to the present have included sponsor-selected chemicals.

Michigan Technological University was chosen as the prime contractor to develop the Project ESP database with assistance from software and database consultants.  This document describes the policy and guidelines which Project ESP staff have used to develop the Environmental and Safety Properties.  Each section describes in detail the methods, assumptions, and procedures used by Project ESP staff to complete the tasks set forth.

1.1  Project Approach

The team assembled by Michigan Technological University consists of chemical and environmental engineers, chemists, biologists, and software and database consultants.  This team has set the following objectives:

1.  Develop a data compilation for each property and chemical identified by the AIChE/DIPPR® Technical Steering Committee.  This compilation would include the values and units for each of the properties and for each of the chemicals as well as the quality rating, literature citation, appropriate comments, and experimental conditions where available.

2.  Provide a comprehensive search of all available, reliable data resources both domestic and foreign.  These sources include standard library literature as well as industrial reports, commercially available computerized databases, government reports, and Project 801 data.

3.  Evaluate each piece of data on the basis of consistency with other properties of that chemical, fundamental thermodynamics, statistical correlations, and the documentable quality assurance/quality control (QA/QC) behind each study.

4.  Develop a DOS software product incorporating features such as CAS number indexing, literature citations, quality codes, temperature dependent graphing capabilities, and built-in conversion of units.  The final DOS software product (sponsor version 6.0) was released to the Project ESP Steering Committee in July 1998.  Future public dissemination of Project ESP data in electronic form will be done through a third party vendor, EPCON International (www.epcon.com).  In 1998, the steering committee granted EPCON the rights to license the ESP database to third party users.  Sponsor companies continue to receive the Project ESP data files from MTU prior to public release.

5.  In a companion project, Project 912, MTU evaluated existing and developed new estimation methods for many of the properties in Project ESP.  These Project 912 methods are used to estimate data not available in the literature and to test available data for consistency.

In order to efficiently manage and track the number of individual pieces of data as well as the amount of data source material, the Project ESP team chose to develop, with the assistance of software and database consultants, a computerized data management and tracking system.  A detailed discussion of each of the components including properties and chemicals of interest, literature searches, quality screening methods, data tracking system, and demonstration software in the Project ESP effort is provided in separate sections below.

- 2.0  Project ESP Parameters

- 2.1  List of Parameters

The Technical Steering Committee has identified six blocks of parameters or properties to be included in the Project ESP effort.  These are:

| Block Number | Block Title |
| --- | --- |
| 1 | Oxygen Demand |
| 2 | Partitioning Parameters |
| 3 | General Physical Properties |
| 4 | Vapor-Liquid Equilibrium |
| 5 | Fire and Explosion Parameters |
| 6 | Sensory, Health and Toxicity Impacts |

Each block contains three or more parameters.  The properties requested by the Technical Steering Committee have been incorporated into the data management and tracking system.  The list of property names and property codes as used by Project ESP and as established by the Technical Steering Committee can be found in Table 1.  In order to incorporate this list in the data management and tracking system, the list was revised slightly from the original layout.  In some cases additional properties have been added to provide unique descriptors of each property such as for Flammability Limits.  It was also necessary to shorten some names to provide more room on the final software product screens.

- 2.2  Definitions of Parameters

The following is a listing of the definitions of each of the properties as used in Project ESP.  A discussion of the units used for each definition is contained in Section 2.2.7.

- 2.2.1  Block 1 Oxygen Demand

1a.  Biochemical oxygen demand (BOD) is a measure of the oxygen consumed by microbial life while assimilating and oxidizing the organic matter present in a wastewater.  The standard BOD tests are typically run for 5 days at 20C in the dark.

1b.  Chemical oxygen demand (COD) is another method of obtaining the equivalent of oxygen demand of the organic matter by oxidizing the sample using a strong chemical oxidant such as potassium dichromate or permanganate.  Since COD tests are very severe and interference from chloride ions can be a problem if not accounted for during testing, it should be noted that COD is not a true measure of biodegradable organic carbon.  Two property codes have been created for COD to reflect the two most commonly used test procedures.  The property code for COD using potassium dichromate as the oxidant is 1b, and the property code for COD using potassium permanganate as the oxidant is 1bp.

1c. Theoretical Oxygen Demand (ThOD) is estimated for specific organic compounds based on the stoichiometry of a balanced chemical reaction which describes the reaction of the organic chemical with oxygen to produce theoretical inorganic end products.  The following guidelines were used to determine the reaction's inorganic end products.  Organic chemicals which contain only carbon and hydrogen were assumed to be converted to CO2 and H2O; organic chemicals which contain carbon, hydrogen, and sulfur were assumed to be converted to CO2, H2O, and SO42-; organic chemicals which contain carbon, hydrogen, and phosphorus were assumed to be converted to CO2, H2O and orthophosphate forms (i.e., H2PO4- or HPO42-); organic chemicals which contain carbon, hydrogen, and halogens (e.g., chlorine, bromine) were assumed to be converted to CO2,, H2O and the respective halogen ion (e.g., Cl-, Br-); and, organic chemicals which contain carbon, hydrogen, and nitrogen were assumed to be converted to CO2, H2O, and ammonia.  In the later case the oxygen demand associated with nitrogenous theoretical oxygen demand was found by showing the stoichiometric amount of oxygen required to oxidize the ammonia to nitrate.

An exception to these rules is chemicals with nitro (NO2) or nitroso (NO) groups.  The nitrogen in a nitro or nitroso group will be converted to ammonia under anaerobic, reducing conditions.  Under aerobic conditions the nitro or nitroso group will go directly to nitrite and subsequently to nitrate.  In order to calculate the carbonaceous ThOD for chemicals with nitro or nitroso groups, the stoichiometry was drawn out by hand assuming nitro or nitroso groups go to nitrate and all other nitrogen goes to ammonia.  For combined carbonaceous and nitrogenous oxygen demand all the nitrogen was assumed to go to nitrate.

Both ThOD based on oxidation of C and H (carbonaceous) and ThOD based on oxidation of all elements (including N) to their highest oxidation state in the environment (combined) are provided for the Project ESP chemicals.  The property code for carbonaceous ThOD is 1cc, and the property code for combined carbonaceous and nitrogenous ThOD is 1cn.

- 2.2.2 Block 2 Partitioning Parameters

2a. 1-Octanol/water partition coefficient (Kow) is defined as the ratio of the equilibrium concentrations of a dissolved chemical in a 1-octanol-rich phase and a water-rich phase.  The 1-octanol and water phases are nearly immiscible.  It is dimensionless and is expressed as the coefficient (Kow) or its base 10 logarithm (log Kow).  All partitioning parameters are measured at 298.15 K unless stated otherwise.

2b.  Soil/water partition coefficients relate the concentration of an organic solute associated with the aqueous and solid phases at equilibrium.  It is defined by the relationship S = Kp C1/n where S is the sorbed chemical concentration, Kp is the soil/water partition coefficient, C is the aqueous concentration and n is the Freundlich exponent.  If the aqueous concentration of the pollutant is less than 10-5 moles/mole solution or one half its aqueous solubility (whichever is lower), the Freundlich isotherm has been found to be linear (i.e.  n=1).  Because Kp's can vary dramatically for different soil types and are typically developed from knowledge of a chemical's Koc (Koc = Kp/OC, where Koc is the soil/water partition normalized for organic carbon on a unit basis and OC is the fraction of organic carbon in the soil) and the fraction of organic carbon in a soil, Project ESP has not incorporated Kp data into the data management system.  In the software, the equation Kp = Koc x (% OC/100) will be displayed if no comment is available.  The user can then use the percentage of organic carbon in their soil sample to calculate the soil/water partition coefficient.

2c.  Organic carbon/water partition coefficients are used in many environmental situations where the amount of organic carbon present in the sorbent dominates sorption (organic carbon >0.1%).  Thus, partition coefficients are normalized with respect to organic carbon or organic matter content.  Although the value given in the Project ESP is the log of the coefficient, units of cm3/g OC are assigned to the value to indicate the antilog units applied to the value.

2d.  Bioconcentration factor (BCF) is defined as concentration of the chemical in the organism at equilibrium divided by the concentration of the chemical in water.  This parameter is dimensionless and can be determined experimentally by dosing water containing the organism and dividing the concentration in the organism by the concentration in water once equilibrium is reached.  If equilibration is slow, the rate of uptake and loss can be used to calculate the bioconcentration factor at equilibrium.  The BCFs are collected for freshwater fish species.  BCF values in the Project ESP database may not necessarily be at 25C, as BCF tests are conducted at temperatures that are dependent on the species.   For example, experiments using a cold-water species such as trout are usually conducted at 16C,  as most trout would die during a BCF test at a higher temperature.  Temperatures for specific BCF data values are found as part of the reference information in the Project ESP database.

- 2.2.3 Block 3 General Physical Properties

Unless otherwise noted, properties of liquids below the normal boiling point are at 1 atm, and those above the normal boiling point are at saturation conditions; solid and vapor properties are at 1 atm or low pressure.  For property values, quality codes, indicating source and accuracy, and references are included.  For temperature dependent properties, discrete data at or near 25C ( 5C) is collected.  The temperature associated with the discrete data is also included. In addition to the discrete data, temperature dependent correlations have also been developed, concentrating on the temperature range of environmental interest, 250 to 500 K.  For these properties, equation-fit statistics and the upper and lower temperature limits of applicability and values computed at these limits are included.  Temperature dependent data are calculated from a temperature dependent correlation and displayed at the default temperature of 25C  (298.15 K) or can be viewed in a graphical form in the Project ESP software.  See Table 2 for units and unit conversions.

3a.  Molecular Weight is defined as the sum of the atomic weights of all the atoms in the molecule based on the IUPAC list of 1993 (J. Phys. Chem Ref. Data 24, 1561 (1995)).  This is expressed in units of kg/kmol.   The  molecular weight data  is used as a means to cross check the  accuracy of  articles using  SQC. The highest rated molecular weight values in the Project ESP datafiles will be the IUPAC calculation.

3b and 3bt.  Liquid Density () is the ratio of the mass of a substance to its volume.  Density can also be determined by dividing the molecular weight by the molar volume or by correcting for the density of water where molar volume or specific gravity are given in the source material.

Specific gravity is frequently given in data collections and is written in the form displayed in Equations (1) and (2).

Example:  0.668204					   Eqn 2

Density of liquid at t1Density of water at t2=ρ1ρw                                                                 Eqn 1

In equation 2, the superscript number is the temperature of the liquid, and the subscript denotes the temperature of the water.  The density of water is 1.00000 g/mL at 277.15 K.  In Project ESP specific gravities are converted to densities by multiplying the density of water and the specific gravity.  Data are presented at or near 25C (3b) or calculated from a temperature dependent correlation (3bt) and expressed in units of kg/m3.  If the substance is a solid at 298.15 K, the density  may be given with a note that it is for the solid.

3c.  Solubility in Water (sa) is defined as the relative mass of the chemical dissolved in aqueous solution which is in equilibrium with an excess of the substance.  The dissolved chemical may be a solid, liquid, or gas.  For any binary solute-solvent system which has limited miscibility, there is a corresponding solubility limit of the solvent in the solute.  These two endpoints of the equilibrium "tie-line" are related through equal-fugacity expressions at a given temperature and pressure and are not independent.  Under Project ESP, we are attempting to tabulate (or estimate) both chemical-in-water and water-in-chemical solubilities.  The water-in-chemical solubilities, if given, are recorded in the comments (literature section) of the chemical-in-water solubility.  Values listed are at 298.15 K unless noted otherwise and in units of ppm(wt).  When the literature reports a chemical as “infinitely soluble”, a value of 1,000,000 ppm (wt) has been placed in the Project ESP database in the corresponding “value” and “unit” fields.

For environmental purposes (temperature range of 20 to 30C), solubilities of gases in water in the Project ESP database are assumed to be at 1 atm total pressure, unless otherwise noted in the reference information or comments.    For applications outside an environmental range, the partial pressure of the chemical must be considered.

3d.  Melting Point (TMP) is defined as the temperature at which both liquid and solid exist in equilibrium at a pressure of 1 atmosphere.  These are presented in units of K.

3e.  Normal Boiling Point (TBP) is defined as the temperature at which the liquid exerts an equilibrium vapor pressure of 1 atm.  These are presented in units of K.

3f and 3g.  The Vapor Pressure (Pvp) of a pure chemical is the pressure which the chemical will exert at a given temperature; i.e., it is the pressure of the vapor above the chemical at equilibrium.  In a closed vessel, a chemical is in a dynamic state of equilibrium in which the molecules of the chemical are constantly shifting between chemical and vapor states.  The presence of this vapor creates a pressure termed the vapor pressure.  Vapor pressure is temperature dependent and is presented in Project ESP at or near 25C (temperature stated if not 298.15 K) (3f) or calculated from a temperature dependent correlation (3g).  Both 3f and 3g are presented in units of Pa.

3h and 3i.  Molecular Diffusivity (DAB) is a measure of the mass transfer through a homogeneous gas,

liquid, or solid phase as defined by Fick's Law: N=DABdCAdx, where N is the molar flux and dCAdxis the concentration or potential gradient for species A in a phase B.  This mass transfer results from gradients in chemical potential and the diffusion coefficient relates the potential difference (usually represented by concentration or partial pressure) to the rate of diffusion.  Diffusivity values are presented in Project ESP in air (3h) and in water (3i) at or near 25C unless otherwise noted and in cm2/s.

3j.  Vapor Viscosity (V) is analogous to the viscosity of a liquid.  If atoms in a fast moving layer travel into a slower moving layer, they transfer their excess momentum to the atoms in the slow moving layer.  If the opposite occurs, the atoms from the slow layer retard the motion of the faster layer.  The rate at which these atom transfers occur is proportional to the vapor density and to the average velocity gradient.  Data are calculated from a temperature dependent empirical correlation in Pas or can be viewed in a graphical form in the Project ESP software product.  The viscosity used here is the dynamic viscosity.

3k.  Liquid Viscosity (L) is a measure of the resistance of a fluid to dynamic change.  When a force is applied to a fluid, it begins to flow at a velocity which is proportional to the force.  This flow is not uniform because of the fluid's resistance to motion.  Data are calculated from a temperature dependent empirical correlation in Pas. The viscosity used here is the dynamic viscosity.

3l and 3lt.  Surface Tension () is a contracting force parallel to a liquid surface which forms a boundary between the liquid and gas phases or liquid/liquid interface.  In mathematical terms it is the force per unit length required to create a new unit area of the gas-liquid interface.  The surface tension cited is the interfacial tension between the pure component and air at or near 298.15 K.  Data in Project ESP are presented at or near 25C (temperature stated if not 298.15 K; 3l) and calculated from a temperature dependent correlation (3lt) in N/m.

3m.  Thermal Conductivity (l and v) is defined as the time rate of heat transfer by conduction through unit thickness, across unit area for a unit difference in temperature.  It is typically measured as Watts per meter times K (W/mK).  Thermal conductivity is calculated in Project ESP for the liquid (l) (3ml) and vapor (v) (3mv) from a temperature dependent property.   Information for solids is also included when available.

3n.  Standard Enthalpy of Formation (Hof,298.15), also known as the Ideal Gas Heat of Formation, is defined as the change in enthalpy associated with the reaction of forming one mole of the given compound in the ideal gas state from the elements in their standard state at 298.15 K.  The standard state of the element is the form which is the most stable at 298.15 K.  Enthalpy of formation is given in joules per kilomole (J/kmol).  When ideal gas values are not available for inorganic compounds solid at 298.15 K, the values for the solid are given in the comments.  If both the ideal gas and the solid values are available, the solid value is placed in the comments.

If values are not located in the literature, they are calculated by the method of Rhodes (1984).  This method uses Benson (1976) groups, by the ASTM's CHETAH (1974) program which uses known heat of formation values for designated compounds as well as Benson groups, or from Benson groups available in Danner and Daubert (1987).

3o.  Liquid Heat Capacity (Cp) is the energy required to change the temperature of a liquid by a specified thermal quantity.  It can be defined in at least three ways: heat capacity at constant pressure, heat capacity at constant volume, and saturated liquid heat capacity.  Unless otherwise stated in the comments, the  Project ESP values cited are for heat capacity at constant pressure.  Data are calculated from a temperature dependent correlation and presented in J/kmolK.  Information for solids may be included when it is available.

3p.  Vapor Heat Capacity (Cp) , also known as the Ideal Gas Heat Capacity, is the energy required to change the temperature of a vapor by specified thermal quantity.  It can be defined in at least four ways: heat capacity at constant pressure, heat capacity at constant volume, saturated vapor heat capacity, and ideal gas heat capacity.  Unless otherwise stated in the comments, the Project ESP values cited are for ideal gas heat capacity.  Data are calculated from a temperature dependent correlation and presented in J/kmolK.

3q.  Critical Temperature (Tc) is the highest temperature at which a pure substance can exist in vapor-liquid equilibrium. This critical point has the coordinates critical pressure (Pc) and critical temperature (Tc).  The critical temperature is where density of the liquid equals the density of the saturated vapor and the heat of vaporization becomes zero.  The pressure at this temperature is the critical pressure.  Critical temperature is presented in units of K.

3r.  Critical Pressure (Pc) is the pressure at the highest temperature at which a pure substance can exist in vapor-liquid equilibrium.  This critical point has the coordinates critical pressure (Pc) and critical temperature (Tc).  Critical pressure is presented in Pa.

3s.  Critical Volume (Vc) is the molar volume at the critical point and is presented in m3/kmol.

3t, 3tt, and 3tz.  The Latent enthalpy of Vaporization (Hvb) is defined as the enthalpy required in the phase change from a liquid to a gas.  This value is usually obtained for the normal boiling point.  The nomenclature for latent enthalpy of vaporization at the normal boiling point is Hvb.  Data are collected and presented for three forms: Hvb at or near 298.15 K (3t), calculated from a temperature dependent correlation (3tt), and at the normal boiling point for that chemical (3tz) all in J/kmol.

- 2.2.4 Block 4 Vapor-Liquid Equilibrium

4a and 4b.  Activity Coefficients () are a measure of the nonideal behavior of a chemical mixture relative to ideal behavior (Lewis-Randall or Henry's law) equilibrium calculations.  The Lewis-Randall standard state for a chemical is commonly chosen to be the pure liquid at the system temperature and pressure where the activity coefficient has a value of one.  If >1.0, there are said to be positive deviations from ideal behavior in which the molecules of the different components in solution are dissimilar and have no preferential interaction between the species.  If <1, there are negative deviations from ideal behavior in which the molecules of the different components exhibit preferential attractive forces (such as hydrogen bonding, etc.) that do not occur for either species alone.  In aqueous systems activity coefficients for hydrophobic chemicals are often expressed as infinite dilution activity coefficients i based on the Lewis-Randall standard state.  The Project ESP group is compiling activity coefficients at infinite dilution of the chemical in water (4a) and of water in the chemical (4b).  Values are for 298.15 K unless stated otherwise and are dimensionless.

4c.  Aqueous Henry's Law Constant (H) is defined as the ratio of a chemical's concentration in the gas phase to its concentration in the water phase at equilibrium, and is often called an air/water partitioning coefficient.  This form of Henry's Law constant is dimensionless [mole of chemical (gas phase)/mole of chemical (liquid phase)].  Two other forms of Henry's constant with units also exist: one relates gas-phase partial pressure to liquid-phase molar concentration (units of Pam3/mol), and the other relates gas-phase partial pressure to liquid-phase mole fraction (units of Pa).  This last form of Henry's constant, by definition, is the product of the Lewis-Randall infinite dilution activity coefficient and the pure component vapor pressure.  The temperature is 298.15 K unless stated otherwise.  Henry's constants are presented in units of kPamol/mol.

- 2.2.5 Block 5 Fire and Explosion Parameters

5a.  Flammability Limits in Air are defined as the boundary line mixtures of gas with air, which if ignited will just propagate flame.  They are presented in terms of percentage by volume of gas in the air.  Both the upper (5au) and lower (5al) flammability limits are presented in Project ESP in units of volume% in air.

5b.  The Flash Point is the lowest temperature (presented here in Kelvin) at which a liquid gives off sufficient vapor to form an ignitable mixture with air near the surface of the liquid or within the vessel used.  The preferred experimental method is closed cup.  In Project ESP, the experimental method is noted with the value where it has been explicitly given. The value for closed cup is in  the “values’ field.  Any value for open cup will be in the “comments” field.  Sataflash is a closed cup method and Cleveland is an open cup method.

5c.  Autoignition Temperature is the minimum temperature for a substance to initiate self-combustion in air in the absence of a spark or flame.

5d.  Standard net enthalpy (heat) of combustion -   The standard net enthalpy of combustion is the increase in enthalpy when a substance in its standard state at 298.15 K and 101325 Pa undergoes oxidation to products obtained in complete combustion.  Unless otherwise stated the value is for the combustion of the gaseous substance to produce H2O(g), CO2(g), F2(g), Cl2(g), Br2(g), I2(g), SO2(g), N2(g), H3PO4(s), and SiO2(crystobalite).  It is given in joules per kilomole (J/kmol) at 298.15 K.   Heat of combustion values in the Project ESP database are defined as negative values for purposes of sign convention.  The standard net enthalpy heat of combustion is also known as the Net Heating Value or the Lower Heating Value (LHV).    For Project ESP chemicals that are the gaseous combustion products given above, the value field in the database has been left blank, and “property inappropriate for this compound” has been placed in the comment field.

For compounds containing C, H, and O if a gross heat of combustion (also known as the Higher Heating Value (HHV)), (H2O(liquid)) value is available, the net value is calculated by adding the enthalpy of vaporization for the appropriate amount of water.  Again, the negative sign convention for heats of combustion is used here.   The user is cautioned that the absolute value of the net heat of combustion should always be less than the absolute value of the gross heat of combustion, by an amount equal to the enthalpy of vaporization of the appropriate amount of water, as stated earlier.  In addition, the enthalpy of combustion is calculated from the accepted enthalpies of formation (in the ideal gas state) to check for consistency with the enthalpy of combustion obtained from the gross enthalpy of combustion.  This calculation involves subtracting the enthalpy of vaporization of the compound at 298.15 K, so that the enthalpy of formation is for the standard state.  The enthalpy of vaporization value used is that calculated from the regression equation.  The agreement is usually excellent; if it is not, the enthalpy of combustion calculated from the enthalpies of formation is accepted.  For some compounds, no experimental values are available but a value for enthalpy of combustion can be predicted by Benson's method and the CHETAH program.  These values are entered based on the ideal gas state and are so noted.

To convert net enthalpy of combustion values for any compound to gross enthalpy of combustion values where the end products are HCl(g), HF(g), P4O10(g), and H2O(liquid), rather than Cl2(g), F2(g), H3PO4(s), and H2O(g), respectively, use the following for H(a), Cl(b), F(c), P(d):

- Hc(gross) = Hc(net) + (b)Hf(HCl)(g) - b2Hf(Cl2)(g) + (c)Hf(HF)(g) - c2Hf(F2)(g) +

d4Hf(P4O10)(g) - (d)Hf(H3PO4)(s) + (a-b-c)/2Hf(H2O)(liquid) - (a-3d)/2Hf(H2O)(g)	Eqn 3

- 2.2.6 Block 6 Sensory, Health and Toxicity Impacts

6a-6i.  Acute Aquatic Toxicity is defined as the potential a chemical has to cause adverse effects in an              aquatic organism over four or fewer days.  The end-points used in Block 6a-i, Acute Aquatic Toxicity, are:

LC50:  the median lethal concentration of material which results in mortality of 50% of test organisms;

EC50:  the median effective concentration of material which results in a sublethal response, such as a behavioral change, in 50% of test organisms.

Pimephales promelas (fathead minnow), Daphnia magna (water flea), Salmonidae (trout and salmon family) Mysidacea (opossum shrimp order), two toxicity endpoints (EC50, LC50), and three exposure times (24, 48, 96 hours) are incorporated under this block.  Initially, the literature was searched for each combination of organism, endpoint, and time.  However, it became apparent that few data were available for some of these combinations, and a number of property codes were deleted as indicated by an asterisk (*) below:

Property		                                         		Test

Code		Name			Endpoint	Duration

6ab		fathead minnow		EC50		48 hours

6ac		fathead minnow		EC50		96 hours

6ba		fathead minnow		LC50		24 hours

6bb		fathead minnow		LC50		48 hours

6bc		fathead minnow		LC50		96 hours

6ca		Daphnia magna		EC50		24 hours

6cb		Daphnia magna		EC50		48 hours

*6cc		Daphnia magna		EC50		96 hours

6da		Daphnia magna		LC50		24 hours

6db		Daphnia magna		LC50		48 hours

*6dc		Daphnia magna		LC50		96 hours

*6ea		Salmonidae		EC50		24 hours

*6eb		Salmonidae		EC50		48 hours

*6ec		Salmonidae		EC50		96 hours

6fa		Salmonidae		LC50		24 hours

6fb		Salmonidae		LC50		48 hours

6fc		Salmonidae		LC50		96 hours

*6ga		Mysidacea		EC50		24 hours

*6gb		Mysidacea		EC50		48 hours

*6gc		Mysidacea		EC50		96 hours

*6ha		Mysidacea		LC50		24 hours

*6hb		Mysidacea		LC50		48 hours

6hc		Mysidacea		LC50		96 hours

6i			Other (Those property numbers designated with

an asterisk (*) are grouped into this general property code)

Bioconcentration factors are collected for the following species:  fathead minnow, Pimephales promelas; common water flea, Daphnia magna; Atlantic salmon, Salmo salar; brown trout, Salmo trutta; lake trout, Salmo trutta lacustris; arctic char, Salvelinus alpinus; brook trout, Salvelinus fontinalis; white spotted char, Salvelinus leucomaenis; Dolly Varden, Salvelinus malma; lake trout, siscowet, Salvelinus namaycush; golden trout, Oncorhynchus aguabonita; cutthroat trout, Oncorhynchus clarki; gila trout, Oncorhynchus gilae; pink salmon, Oncorhynchus gorbuscha; chum salmon, Oncorhynchus keta; coho salmon, silver salmon, Oncorhynchus kisutch; cherry salmon, yamame trout, Oncorhynchus masou; rainbow trout, Donaldson trout, Oncorhynchus mykiss; sockeye salmon, Oncorhynchus nerka; chinook salmon, Oncorhynchus tshawytscha; and the opossum shrimp order Mysidacea, Bowmaniella dissimilis, Leptomysis mediterranea, Mysidopsis bahia, Mysidopsis bigelowi, Mysidopsis almyra, Mysis relicta, Neomysis americana, Neomysis awatschensis, Neomysis integer, Taphromysis louisianae.

- 2.2.7 Units

Data are entered into the Project ESP database in the original units from the data source material.  In some cases the data are converted to simplify the conversion routine.  For example, solubility units given in log mmol/L are converted with inverse log to get mmol/L.  A unit conversion routine has been developed in which the data extracted for the Project ESP software product are converted into a default unit.  The routine then gives options of SI or CGS units.  Table 2 provides a list of the default units for each property, selection of unit options and the mathematical expression to convert to them.  The default unit is the unit to which all the values for that property have been converted and will be displayed on the software screen unless the user chooses a different unit from the list of unit options.  In all cases where it is necessary to incorporate the density of water, it was assumed to be 1 g/mL.

The conversion routines are based on a temperature of 298.15 K.  If there are values in the database at temperatures other than 298.15 K, the conversion is calculated by hand and entered into the database in the default units.  The original value and units as stated in the literature source is entered into the comments section.  This is used especially for Henry's constants (4c) and solubility.

This "hard-coding" is also employed for solubility (3c) values which exceed 10,000 ppm(wt).  The automatic conversion routines and relationships for solubility are only applicable for values < 10,000 ppm(wt).  Again, the original value and units are entered in the comments section of the database.

- 3.0 Description of Chemical List

The stated purpose of Project ESP data compilation effort is to focus on chemicals of environmental and safety concerns.  Initially the chemicals listed in the CAAA were chosen by the Project ESP Technical Steering Committee as the focus.  This list consists of 172 chemicals identified by the Chemical Abstract Service (CAS) number as well as an additional 17 chemicals, such as cadmium compounds, which can be listed with multiple CAS numbers.  The 1992 effort focused on those with single CAS numbers although information has been collected on the other chemicals as well.  In 1993, 109 chemicals from the OSHA list were added to the database.  The 1994 additions included the EPA-RMP list and a list of sponsor-selected chemicals.  The chemical list is currently at 1104 total chemicals with all additional chemicals not mentioned above sponsor-selected.

Several problems were discovered in regard to the CAAA list and have been discussed with EPA staff.  First, several chemicals identified by specific CAS number are for salts, esters or mixtures of species such as 2,4-D, Aroclors, 4,6-dinitro-o-cresol, and dibenzofurans.  The DIPPR® projects are interested in pure chemicals.  Unique CAS numbers exist for the salts and esters or for the different dibenzofurans.  Second, the CAS number given in the CAAA list for DDE is incorrect.  The correct CAS number is used in Project ESP. Third, CAS number 121697 for N,N,-Diethylaniline in the CAAA list is actually for the N,N-Dimethylaniline.  Project ESP has incorporated the CAS numbers for both chemicals until a decision from EPA is made regarding which chemical should be listed.  The Project ESP list has also distinguished between the isomers of 1,3-Dichloropropene.  There are three forms of phosphorus: white, yellow and red.  Priority for entry into the database has been given in the following order: white, yellow, red.  Data for yellow or red phosphorus are placed in the comments where appropriate.

In the OSHA list, incorrect CAS numbers were identified for Propyl Nitrate, Ammonium Permanganate, Oleum and Trichloro(dichlorophenyl)silane and the correct CAS numbers were incorporated into the database.  Another change made to the OSHA list was that Project ESP distinguishes between the isomers of Ethyl Methyl Ketone Peroxide in the database.  Additional chemical suggested by sponsors are also included in the database.

In order to conform to software constraints, each chemical was assigned an arbitrary number based on alphabetical order for the CAAA list and, starting with number 173, in numerical order based on CAS number for the OSHA list (Table 3).  The number continues for the EPA-RMP list and the sponsor-selected chemicals.

4.0 Literature Search

4.1 General Approach

There are a number of sources of data which are incorporated into the Project ESP database.  They include the following:  1) values from AIChE/DIPPR® Project 801 where Project 801 and Project ESP have common chemicals and parameters; 2) principal literature sources where data are readily available; 3) available literature compilations, commercially available electronic databases, and other electronic sources; and 4) international literature to obtain source materials to fill the additional parameters and data gaps.  The method followed for each of these steps is discussed below.

- 4.2 Project 801 Release

Communication among Brigham Young University, DIPPR Project 801, and MTU, DIPPR Project ESP, is ongoing in the exchange of new methods of prediction and new ideas.  This allows each team to keep abreast of general progress of each of the projects.  An example of this exchange has been in the prediction of Block 5 Fire and Explosion parameters.  Project 801 has been utilizing the new prediction methods developed and utilized by Project ESP.

AIChE/DIPPR® Project 801 is a data compilation project for physical, thermodynamic, and transport property data.  Some properties and chemicals are common to both Project 801 and Project ESP.  In order to avoid duplicating efforts, Project ESP receives and extracts the Project 801 data release on a CD  each year.  In the past, the data were provided on a set of magnetic tapes which were then extracted on the MTU mainframe.  The Project 801 data, references, supplementary coefficients, policies and procedures, and footnotes are contained in separate files on the CD.

To conduct the data extraction for the Project ESP chemical list, a series of queries using MS Access is used to pull the appropriate values with the associated references, units, quality codes, equation numbers, correlation coefficients, and footnotes, rearrange the field locations as necessary, and insert them into the Project ESP database.  This effort was developed by MTU Staff.  Only those values which were labeled as acceptable by Project 801 are extracted.

An update or extraction of Project 801 data is currently incorporated into the ESP database semiannually.  The updates are performed by tagging the code system which Project 801 uses.  For example, Project 801 labeled values entered during 8/1/91 to 1/31/92 with a L and so on alphabetically until the 2/1/96-7/31/96 which was marked with an X.  Changes made to Project 801 data which have already been downloaded by Project ESP is designated with a C or & or @ which permits the extraction of all the updates of the Project 801 database.  These updates and extractions have been occurring annually since August of 1991.

4.3 Principal Literature Search

Literature reviews, thorough searches of available journals, chemical supply catalogs, and published compilations are conducted at Michigan Technological University's Van Pelt Library.  Source materials not available from the Van Pelt Library are requested by interlibrary loan.  Certain source materials were found to be particularly productive sources of high quality data.  These principal literature sources are given in Table 4 by block number and the journals are reviewed regularly for additional data.  If data are not available from the principal sources a more detailed search is conducted.

- 4.3.1 Blocks 1 and 2 Oxygen Demand and Soil/Water Partitioning

A literature search was conducted in 1992, 1993, and 1995 primarily with a WilsonDisc CD-ROM information retrieval system developed by the H. W. Wilson Co. of Bronx, New York.  This is an electronic data base system that contains approximately 800 journals from four major indexes: Readers Guide, Biological and Agricultural, Applied Science and Technology, and Business Periodicals.  The majority of this search was spent in the Applied Science and Technology, and Biological and Agricultural indexes.  The system at the MTU J. Robert Van Pelt Library only covers back to approximately 1983 for most references, though information was obtained from sources dating back as far as 1967.  Any relevant titles and keywords were searched on this system by entering the words; partitioning, sorption, Koc and distribution coefficient to name just a few.  In addition, Environ. Sci. Technol., Water Res., Soil Sci. Soc. Am. J. and J. Agric. Food Chem. were individually searched back to 1965 because several articles on the subject were referenced to these journals.  The 1995 search generated many more first-hand data sources for BOD dating back to 1932.  These sources included Purdue waste conferences from 1946 to present, Sewage Works Journal, Sewage and Industrial Wastes Engineering, and several German sources.  One difficulty with BOD values is the lack of measured values in the past two decades.

Another search that turned out to be more productive came from searching through glossaries of two books that had previously compiled data on partitioning.  These books were K. Verschueren, Handbook of Environmental Data on Organic Chemicals (1983) and the Mackay, et al., Illustrated Handbook of Physical Chemical Properties and Environmental Fate for Organic Chemicals (1992).  An excellent source of experimental COD data was a German publication Chemische Oxiderbarkeit organischer Wasserinhaltsstoffe  which contains nearly 600 values for COD by both dichromate and permanganate methods.  A complete list of references is found in Table 4 (Blocks 1 & 2).

Oxygen demand data were compiled using two basic criteria:  1) only chemicals listed in the Technical Steering Committee list were considered; and 2) the study must have followed standard procedures in the obtaining of these data.  For example, COD values were obtained for methods using either a dichromate oxidant in the presence of a silver sulfate catalyst for 1b or permanganate oxidant for 1bp.  Experimental data were not found for all the chemicals.  Estimates were then used to fill in gaps in the data base.  This is described in more detail under Estimation Methods below.

Due to the nature of the Biochemical Oxygen Demand (BOD) test, the rating criteria used differed from the system used for other DIPPR properties.  A rating system specifically for BOD was created based on the conditions that critically affect the results of the BOD test.  For convenience a copy of the rating form is provided in Table 5.

The experimental technique used is of utmost importance in the BOD test as the results tend to be highly variable.  Those experiments not following the 1992, 18th Edition of Standard Methods for Examination of Wastewater and Water (APHA, AWWA, WEF) (Standard Methods) produce data that are not comparable to other BOD values.  For this reason, those papers following Standard Methods were given a higher ranking than those not following Standard Methods.

Just as the experimental technique is important, so too is the temperature at which the experiment is conducted.  Most biological processes are a function of temperature.  To insure that data from different sources are comparable with respect to temperature, Standard Methods recommends performing the BOD test 20C.  In some instances temperature is intentionally manipulated for the purpose of the experiment.  For example, an experiment may be designed to show the effect on BOD of seasonal changes in a waste water.  In such cases the rating system does not penalize a paper for running the experiment at a temperature other than 20C.  A paper is only penalized if the temperature is not reported.  The comment field will indicate the temperature of all experiments run at a temperature other than 20C.

The state of seed acclimation also plays an important role in the BOD test.  An organism that is not acclimated to a specific chemical may experience a lag phase during which the organism adapts itself to the compound of interest.  The length of this lag phase is highly variable.  If the lag phase is greater than the five days allowed for the test, the chemical may appear to be nonbiodegradable or perhaps less biodegradable than if an acclimated seed was used.  An acclimated seed receives a higher rating than a nonacclimated seed in the rating criteria, which in turn receives a higher rating than those paper in which acclimation is not reported.  Both the seed used and the state of acclimation will be reported in the comment field.

The initial chemical concentration can also affect the results of the BOD test.  It is important that the concentration is not toxic to the seed organisms and that an adequate amount of dissolved oxygen is present in the test container to degrade the compound of interest.  Standard Methods states that a dilution which results in a residual dissolved oxygen concentration of at least 1 mg/L and a dissolved oxygen uptake of at least 2 mg/L after five days incubation produces the most reliable results.  Papers were evaluated on whether or not the initial chemical concentration was reported to indicate the author's thorough understanding of the BOD test.

Finally, the BOD of a chemical must be less than or equal to the chemical's theoretical oxygen demand.  The comparison of the results of the BOD test to the chemical's theoretical oxygen demand was used to show internal consistency.

Based on the literature review conducted in 1992, 1993, and 1995 for soil/water partitioning, it appears that it will be very difficult to develop an extensive, good quality data base for most chemicals for a number of reasons. Three important reasons for this are:  1) many DIPPR chemicals are not strongly sorbed to soils/sediments; therefore, their sorptive properties have not been investigated intensively; 2) there is no standard method for determining soil/water partition coefficients or units of reporting; thus, much of the data reviewed for this project were judged unacceptable; and, 3) a major problem not accounted for by many researchers when measuring soil water partition coefficients is that the target chemical can be lost by biological degradation, volatilization, or sorption onto surfaces other than the soil (e.g., onto glassware).

Many values were judged unacceptable because the reported units could not be normalized to one common standardized unit.  Soil/water partition coefficients are typically normalized to some measurement of organic carbon.  This allows application of a partition coefficient to most circumstances, as long as the amount of organic carbon is known.  Unfortunately, researchers normalize the partition coefficient on either a basis of organic carbon (OC) or organic matter (OM).  OM is the organic fraction of the soil and includes organic carbon, nitrogen, phosphorus, and sulfur.  However, OC only accounts for the organic carbon fraction of soil.  While, OM is typically related to OC by multiplying OC by 1.724, a number of studies suggest that the conversion factor may actually range from 1.9 to 2.5 (Nelson, D.W. and L.E. Sommers, Total Carbon, Organic Carbon, and Organic Matter, American Society of Agronomy, Madison, WI, 1982).  Thus, values of a soil/water partition coefficient which are reported as Kom, instead of Koc, were not used unless the researcher reported a specific conversion ratio (OC/OM) for their test system.

Where possible, soil/water partition data were obtained according to the following conditions:  1) the isotherms were performed with several soils of varying organic carbon content and the data were fit to a straight line, or a Freundlich isotherm with 1/n=1.  When the data were fit to a Freundlich isotherm with 1/n between 0.9 and 1.1, these data were refit to a straight line to obtain Koc. Experiments have shown that if 1/n is between 0.9 and 1.1 it is statistically more valid to assume that the isotherm is linear (J.C. Crittenden, Dept. of Civil and Environmental Eng., Michigan Tech, personal communication with James Baker, September, 1993); 2) either the soils were sterilized to inhibit biological degradation of the chemical or other tests were performed to account for no biotic loss of the chemical; 3) sorption equilibrium was confirmed by previous results or sequential sampling; 4) the water was free of dissolved organics; and 5)  the pH of the solution was near neutral.  These criteria were developed to follow a proposed American Society of Testing and Materials method for activated carbon adsorption studies (J.C. Crittenden, Department of Civil and Environmental engineering, Michigan Technological University, personal communication with Dr. James Mihelcic, August 1992). The system for rating papers in the area of Soil/Water Partitioning is provided in Table 6.  When multiple sources of Koc that did not meet all the above criteria were available, the source that fit the most criteria was chosen.  Articles that provided data that did not fit any of the criteria were discarded.  For example the extensive data base of G.G. Briggs, J. Agric. Food Chem. 29, 1050 (1981) was discarded because the samples in this study were only equilibrated for 2 hours.

Table 7 provides the reliable sources obtained for Koc and the criteria used for evaluation.  The first row of Table 7 has the proposed ASTM method.  This table was used to decide which references were most reliable in the case where different values for the same chemical were in different papers.  Important criteria from above are listed here such as equilibration time, method of sterilization, source of water used, number of soils used and the range of organic carbon in the soil, the pH at which the isotherms were performed, and the specific method used.

- 4.3.2 Block 3 General Physical Properties

As stated earlier, the Project 801 data have served as a very important starting point for the compilation of general physical properties.  The properties not found in Project 801 have been the subject of an open literature search as described previously.  Examples of the sources scrutinized are listed in Table 4 (Block 3).  The search to date has included both properties at 298.15 K and those as a function of temperature.  Some of the temperature dependent data have not yet been correlated with parameterized equations, and may not yet appear in the DIPPR® ESP electronic database.  The results of the extractions are summarized in master database table (VALUES.MDB).  The coefficients of the temperature dependents are also contained in the VALUES table.

4.3.3 Block 4 Vapor-Liquid Equilibrium

In addition to data sources typically used by chemists, chemical engineers, and environmental scientists, the Project ESP team has broadened the search for partitioning/equilibrium data to medicinal and pharmacological journals.  The latter are particularly good sources for Quantitative Structure Activity Relationships (QSAR) predictive methods.  The team is also investigating collections of data compiled by U.S. EPA contractors, as well as the software data files associated with process simulator packages and wastewater fate and transport models. Table 4 summarizes the primary literature citations used to date.

- 4.3.4 Block 5 Fire and Explosions

A large amount of high quality data are available from international reference material.  Project ESP has obtained data from:  A.N. Baratov, A.Ya. Korol'chenko, G.I. Kravchukh, et al., The Handbook for Flammability and Explosability of Chemicals and Substances, and Means of Extinguishing, Khimia, Moscow, 1990; and K. Nabert and G. Schön, Sicherheitstechnische Kennzahlen brennbarer Gas and Dampfe, Deutscher Eichuerlag Gmbtl., Berlin, 1963.  The Russian translation was conducted by a native speaker, who translated the introductory material, a summary of methods used in the compilations, and the data.  Project ESP found this compilation to be a very rich source of Fire and Explosion data.  Fire and explosion data were also translated from the German handbook as well as data for General Physical Properties.  A list of principal sources is provided in Table 4, Block 5.

- 4.3.5 Block 6 Sensory, Health & Toxicity Impacts and 2d Bioconcentration Factors

In addition to the Wilsondisc library search described in Section 4.3.1, the reference sections of collected papers were also searched for relevant titles in the area of Aquatic Toxicity and bioconcentration factor.  Consequently, papers were collected from over 40 journals and other research compilations.  Journals checked regularly are listed in Table 4 (Block 6).  An on-line Current Contents search of the tables of contents of journals not received at MTU is performed periodically.

Another source of information, the U.S. Environmental Protection Agency's (EPA) Aquatic Toxicity Information Retrieval Data Base (AQUIRE) was purchased from the National Technical Information Service (NTIS).  This source contains ecotoxicology data, literature references, and EPA data quality codes.

The AQUIRE ecotoxicology data are distributed over nine files:  master record file with pointers to data in the  other eight files; test chemical purity; concentration of the chemical in water and/or bioconcentration factor and confidence interval; test temperature, hardness, dissolved oxygen, alkalinity, and pH; species name code; CAS number for each record; CAS numbers and corresponding chemical names; comments; references.  An additional nine files define the structure of the nine corresponding data files.

These 18 files were uploaded from magnetic tape to MTU's Sun network and converted to text files from VMS-VAX format.  A search program was developed to extract all the AQUIRE ecotoxicology information, EPA quality codes, and literature references for a specified species/chemical combination.  Each record has 64 fields.  The output may be viewed on screen on a Sun terminal, or it can be imported into a spreadsheet or database program as a text file.

A final program is used to aid in condensing the 64 AQUIRE fields into 20 Project ESP fields, reformatting the AQUIRE citations, and translating the EPA review codes into Project ESP rating codes.  Removal of key violations (where a paper has more than one data point per species/chemical/citation combination) and consolidation of the 7, 60-character AQUIRE comment fields into 1, 120-character ESP field must be performed manually.  Finally, when AQUIRE reports a range for the temperature, pH, or hardness field, the median value is reported and the range is manually entered as a ± value in the Project ESP Comments field.

- 5.0 Evaluation Criteria

- 5.1 Quality Assurance

Every effort is made to ensure that the thousands of pieces of data in the Project ESP database are recorded accurately.  The quality assurance procedures for the data entry process are threefold; 1) a screening by the reviewer, 2) a screening of the data entry, and item by item check where necessary, and 3) a review for consistency with data from other source materials.

The first review is conducted by the technical reviewer.  This initial evaluation consists of an examination of the data generation and collection procedures which are outlined in the literature.  The paper is then rated based on the experimental procedures followed.

The second review is conducted by the data entry personnel.  The data are transcribed from the literature source onto a data entry form and it is determined if all necessary information is present.  The data entry form is then entered directly into an Excel™ spreadsheet.  A printout of the spreadsheet is compared to the original publication prior to incorporation into the Project ESP database.

The third review is a quantitative assessment for consistency with other data, thermodynamic relationships, or other theoretical principles.  A Windows based error flagging program utilizing Statistical Quality Control (SQC) techniques has been developed which can perform an error analysis on data files for which the units have been converted to a common units system.  The program uses the highest rated value as the basis for the test, which is referred to as a “Level 1” test by Project ESP personnel.   Presently, any value exceeding the percent error defined for that particular property is labeled as an outlier and is reviewed in detail.  The percent error is set differently within each different physical property depending on the general quality of the data available.  Table 8 summarizes the percent errors used for the various properties.  The values that are flagged are double-checked with the literature for entry errors.  If the value is correctly entered but greatly disagrees with the other reported values at the same temperature, an “AF” code (Anomaly flagged) note is placed in the Anomaly column of the project ESP database.   If the current highest rated data value does not agree with other available data, the highest rated data point is labeled as “AD” (Anomaly Dropped) in the Anomaly column, the rating is lowered, and a new highest rated data point is determined.   The SQC software is then used to compare all available data to the new highest rated data point.  During the SQC review process, MTU investigators are asked for any adjustments to the evaluations or ratings based upon agreement with other work, past experience with a particular data source, or other factors.  All values which have undergone Level 1 scrutiny are then coded with a "Q1" in the Keywords column.   Additional information on the results of the SQC analysis has been made available to Project ESP sponsors at the steering committee meetings.  The use of the SQC software to do thermodynamic based comparisons (Level 2 check) across several physical properties (e.g., Henry’s Constant should be greater than or equal to the ratio of Vapor Pressure to Aqueous Solubility) is ongoing.  Those data points that have completed a Level 2 analysis are labeled with a “Q2” in the Keywords column.

- 5.2 Qualitative Review

A required component of Project ESP is to provide a critical evaluation and quality assessment of each of the data points in the data compilation.  The Project ESP investigator evaluates the data source material based on 5 basic criteria (Table 9) where each is ranked on a scale from 0 to 2.  This system provides that well documented primary sources receive a higher score than incompletely documented or secondary sources.  Separate evaluation criteria for BOD and Soil/Water Partition Coefficient as stated in Section 4.3.1 are shown in Table 5 and Table 7.  As mentioned previously, the data and data analysis for the Acute Aquatic Toxicity Block are managed differently from the engineering parameters and therefore also have separate evaluation criteria as illustrated in Table 10.

The data from Project 801 automatically receive a rating higher than the data gathered by the Project ESP team because of the rigorous data entry, testing and quality checking procedures.   The quality codes used by Project 801 have been incorporated into the Project ESP data management and tracking system. However, they are not completely compatible.  To make them compatible, the Project 801 ratings were translated into the same scale as Project ESP ratings.  Project ESP ratings range from 0 to 10.  The Project 801 ratings were given ratings which ranged from 11 to 14 with 14 being the highest rating.  This was performed by allocating points to Project 801 coding as shown in Table 11.  This provided the ability for the software product to preferentially choose Project 801 values over Project ESP values.  In the cases where Project 801's type of data is predicted, defined, smoothed, unknown, or extrapolated, the rating is left blank as stated in Table 11.  In these cases, Project ESP data are chosen.  Project ESP data are also chosen preferentially in some Fire and Explosion data values because of past complications in Project 801 data.  Any changes to the Project 801 ratings are communicated to the Project 801 staff.

Data in AQUIRE are rated by the EPA on a 5-number scale.  The AQUIRE Technical Support Document defines these review codes as follows:

1 = Thorough methods and documentation;

2 = Documentation generally satisfactory, but one or more of the pieces of information are missing from either the methods or results section, such as control information or chemical concentrations are unmeasured;

3 = Insufficient methods and results documentation;

4 = Indicates data is available only in a limited format, such as conference proceeding abstracts or English language abstracts for untranslated international papers;

5 = This file contains laboratory data for acute toxicity of organic chemicals, using a single test species (30-day fathead minnows).  The information was transferred via computer files and was not subjected to the standard AQUIRE procedures.  All test results, including data not available on-line, are available in five volumes titled:  Acute Toxicities of Organic Chemicals to Fathead Minnows (Pimephales promelas), (Center for Lake Superior Environmental Studies; University of Wisconsin-Superior; 1984, 1985, 1986, 1988, and 1990).

Review codes are converted to Project ESP's ten-point scale rating codes as follows:  1 = 9; 2 = 5; 3 = 1; 4 = 0; 5 = 10.

Ratings for AQUIRE in general are not based on an author’s reputation.  Rather, they are based on subjective impressions of Project ESP investigators about the data set based on the completeness of detail in the experimental methods.  A data digest is attached to each paper reviewed.  If key experiment details are not available (such as: test temperature; whether values reported are nominal or measured, especially for volatile organics; whether system was static or flow through; whether dosing was constant or intermittent; whether controls were missing; whether pH was reported, especially for metals; etc.) the rating will be low because the reliability to the end user is reduced.  Lower rated values should always be used only with caution.

- 5.3 Quantitative Reviews

The Qualitative Review rating system provides an initial screen of data but is not intended to replace more quantitative analysis by each investigator.  Once enough data have been received and reviewed, a more quantitative evaluation for physical and thermodynamic consistency is conducted.  The first evaluation is a visual inspection for "smoothness" and consistency.  Consistent values from differing data sources are considered more trustworthy than values which are scattered.  Tests for accuracy include investigator examination of the methods by which the data were obtained to see if acceptable experimental and quality assurance procedures were followed.  The investigators look particularly for analytical methods which can be traced back to quantitative primary standards.

The data sets are also examined for their consistency with theoretical relationships and empirical predictive methods of Project 912.  Since the Project ESP effort concentrates on collecting data on many interrelated properties, data from more than one property code can be checked to confirm their consistency with simple theoretical relationships or more complex interactions.  After such quantitative analysis, the investigators assign a final quality code to the data which then can be drawn into the software product.

- 5.3.1 Statistical Quality Control (SQC)

Prior to  December of 1996, the date at which time SQC was applied to the Project ESP database, a manually intensive program using  QBASIC language  performed similar tasks.  The QBASIC Program was a quantitative assessment for consistency with other data, thermodynamic relationships, or other theoretical principals.  It consisted of two steps: conversion of units and percent error analysis, both of which utilize computer programs generated for the Project ESP project.  The original data collected from the literature are reported in many different units and entered into the database in the original units.  An automated unit conversion routine was employed to put the data into consistent units (SI, or user-defined).

A more automated system is now employed using a Windows software utility to review the Project ESP database.  This software applies SQC techniques to compare data values in some of the same ways as the older QBASIC routines.  The SQC program uses the highest rated value as the basis for the test. The internal comparison to the highest rated data value for a particular chemical and property code combination is referred to as a “Level 1” SQC review.  Presently, any value exceeding a defined percent error tolerance value is identified as an outlier in the output file.  The percent error tolerance value is set differently for each property within the different property blocks depending on the general quality of the data available, e.g. 10% error for most of Block 3 and 50% error for Block 6.  The outlying values are double checked with the literature to identify entry errors.  If the value has been correctly entered, but greatly disagrees with the other reported values at the same temperature, a note is placed in the comments indicating such.  The project investigators are asked for any adjustments to the evaluations or ratings of that particular entry based upon "agreement with other work."

All values which have undergone a quantitative review are coded with a Q character in the Keywords column of the VALUES table.  The data analyzed using the older QBASIC routines received a code of “Q” when they passed the review criteria.  The newer SQC system uses codes of “Q1” for data values passing the Level 1 check, and “Q2” for passing a Level 2 review, as defined below.

The Level 2 data evaluation is a comparison of physical properties using additional SQC criteria. It involves the comparison of data values for a given property code to another property, an algebraic calculation involving one or more properties, or comparison to a specified range.  Examples of Level 2 data evaluations are confirming that all melting points are less than their corresponding normal boiling points, or that the Henry’s Law Constant is greater than or equal to the ratio of the vapor pressure to solubility.  Data values successfully completing a Level 2 review are designated in the Project ESP software with a “Q2” quality code.

- 5.3.2 Classification of Errors Using SQC

When performing an analysis using the SQC system, the "actual error" rate and "flag" rate must be carefully  separated.  Each data value flagged by the SQC software is examined and analyzed to determine the classification or type of error.  All data that successfully pass the SQC check will have a "Q1" or "Q2" entered into the database column which will be seen in the Project ESP software product.

For those data values that do not pass the SQC criteria, the original article is reviewed so that an error code can be attached to the data value.  Those values receiving an error code of "AF" (Anomaly Flagged) or "AD" (Anomaly, Rating Dropped) are reviewed by an MTU investigator to determine their accuracy, and a recommendation made as to their disposition.  Data values that are determined to be data entry mistakes will be flagged as "DE" (Data Entry Error) and corrected in the Project ESP data files.

Values that have been flagged with an "AF" code are values that are outside the tolerance range when compared to the highest qualitatively rated data value for a particular chemical and physical property, but the value has been correctly transcribed from the original literature reference.  An anomaly flag of "AD" is used when a number of literature sources report data values that are in agreement with each other, but are outside the tolerance range when compared to the highest qualitatively rated data value.  The original literature references are reviewed, and a determination is made as to whether a transcription error has occurred with the highest rated value, or whether different experimental conditions or techniques were used for the various literature sources.  A reliability assessment is also made about the authors of the literature source, based on the experience of MTU investigators.  If an error in transcription has occurred, the highest rated value is corrected and labeled "DE."  If it is found that the highest rated value uses a less reliable experimental technique, or there are questions about the quality of work based on the authors, the highest rated value is labeled "AD", the qualitative numeric rating is lowered, and the SQC system is run again to check the data against the new highest rated value.  The "DE", "AF", and "AD" codes are not displayed within the Project ESP software product, but they are documented by the SQC tracking system and are available in the Access format sponsor release datafile.

Data values that are flagged with an error code of "DE" are rechecked by the SQC system during the next SQC review of the Project ESP database.  At that time, it is anticipated that the error will not be repeated, and the values will receive a rating of "Q1" or "Q2".  An Anomaly Tracking Form is kept with the database reference article so that MTU staff have a complete record of any changes made to data values from a particular reference.  The output files from the SQC system, which are a compilation of all errors identified for the criteria checks run, are logged and dated in a notebook and maintained in a file according to the physical property on which the SQC analysis was completed.  The SQC output files can be cross referenced to the individual data errors on the Anomaly Tracking Forms attached to each Project ESP database reference paper.  Table 12 contains the current  results from  the SQC Review  System.

- 6.0 Data Tracking and Management

- 6.1 General Overview

An integral part of the compilation effort is to organize into an efficient software system the thousands of reference materials and tens of thousands of pieces of data.  Ideally the organizational system would allow the Project ESP team quick and easy access to the data as well as data source materials and provide a mechanism to download the data for quantitative review and for the final software product.  To accomplish this task the Project ESP team, with the assistance of a software and database consultant, developed the data management and tracking system based on a relational database approach.  This tracking database is designed around a MS Access™ driver and customized according to the needs of Project ESP.

The purpose of the tracking database is to provide a method to identify, track, and store information about the Project ESP chemical and property lists, data source materials, data values, units, and qualitative rating.  In order to efficiently design the database, these types of information were separated into tables and key relationships among them were established.  For an efficient database design, relationships are made between categories by assigning a key field to the categories one wishes to relate and by assigning an arbitrary numbering or coding system to that key field.  In the tracking database, the chemical, property, and source materials lists are separate tables, each with an assigned number and/or code.  The values for each chemical and property combination are kept in a separate table and related to the chemical, property, and citation lists by the appropriate numbers and codes.  In this manner, the data source material can be sorted, tagged and downloaded independently of the data associated with it.  Conversely, each piece of data is identified by its source.

- 6.2 Datafiles Description

The data source tables are distributed to project sponsors on an annual basis in the Access format.  A brief description of each table and its purpose and structure is given below.

The VALUES Table is the primary data table which contains the chemical number, property code, quality code, citation number, data value, unit, comment, and rating.  The structure of the table is provided in Table 13 with a brief description of what each field contains.  The data contained in VALUES is quantitatively assessed for errors and is in the original units given in the source material.  Since this table is the primary data Table a multitude of forms, scripts, and reports are generated from it.  These are described in greater detail below.

The CITATION Table contains the data about the source material, i.e. the author, title, journal or book, and publication date.  The Project ESP data entry personnel have developed a standard format for entering data into this table from different data sources.  This format is not completely compatible with the references which are obtained from Project 801 data files.  The Project 801 reference numbers have been given paper numbers starting from 80,000 to distinguish them from Project ESP data sources.  In addition to these Project 801 datafiles, a number of calculations have been performed over the course of Projects ESP and 912 by different researchers  which must be documented.  Therefore it has become necessary to develop a system of identifying these with distinct citation numbers.  These citation numbers are identified by project number, year, and sequence.  For example, the first set of calculations performed by Project ESP in the year 1995 are given the number ESP951.  The second set would be designated ESP952, and so on.  The year 1995 follows the project year:  June 1, 1995 - May 31, 1996.  The structure of the CITATION Table is given in Table 14.

The PROPERTY Table contains information about the Project ESP properties.  This includes the property code name, reviewer name, and default units for that property.  A "Y" indicates that the property is temperature dependent.  This Table is used in the Project ESP commercial software and the structure is given in Table 15.

The CHEMICAL Table contains information about the chemical list in Project ESP.  The structure and information in the CHEMICAL Table are provided in Table 16.  This provides a valuable link between the software and the VALUES Table because they contain the CAS number and VALUES Table does not.

The CONVUNIT Table contains the unit conversion information.  It contains fields listing the default units and original literature units, and the multipliers and addends required to convert between the two. The structure is provided in Table 17.  This Table is used in the units conversion routine in the Project ESP software product.  The unit conversion data are defined by the Project ESP staff and conversions may be added or changed as necessary to satisfy user needs.

Another temporary table is the Completion Report.xls table.  Based on the data which are entered in the VALUES table, this completion report is an output of the chemicals and property codes for which there are data entered.  It is a good method of determining where there are holes in the database which need to be filled.  The completeness report identifies where at least one value has been entered for each chemical for each property with an "X".  A "  " is left when there is no value for that property.  A summary of the number of data points per property is generated at the end of the report.  The purpose of this report is to identify gaps in data, and set priorities for data entry and data searches.  This table is furnished upon request.

6.3 Data Entry System

In the initial organization of Project ESP, a data entry system was requested from the software consultants.  The purpose of this system was to provide a user friendly and "foolproof" method for data entry. Data entry template software and forms were custom designed in MS Access™ for Project ESP.  Access tables were set up to easily update the VALUES, CHEMICAL, CITATION, PROPERTY, and CONVUNIT tables.  These tables allow the data entry personnel to enter, delete or modify records which have been entered.

A system has been developed to track and handle data source material and to provide a uniform method for data entry.  Once a data source has been located, it is immediately attached to a DIPPR® Tracking Form (Figure

1).  A Citation Entry Format (Figure 2) is used to provide uniform entry for varied material such as journal articles, books, chapters from books, government or industrial reports, or staff-generated materials.  The original tracking form is forwarded to the appropriate reviewer.  The reviewer rates the paper, offers comments concerning the quality of the data, and notes the chemicals and properties contained in the paper.

The data entry personnel then check the assigned chemical numbers against the chemical list and copy the appropriate (highlighted) data from the publication to data entry forms (Figure 3).  The data are then entered directly into an Excel™ spreadsheet.  A printout of this spreadsheet is then double checked against the original publication for any transcription errors.  The data are then imported into Access™ and entered directly into the VALUES Table.  Once the data have been entered into the data base, the source material along with the data entry form and Excel™ printout of the exact data which have been entered into VALUES is filed by citation number in a central file "library."

The data for property Block 6, Acute Aquatic Toxicity, is very different from the engineering related data so the values template is used in a slightly different manner from the other property blocks.  This data entry format is shown in Figure 4.

7.0 Temperature Dependent Properties

Several of the properties to be evaluated in Project ESP show temperature dependency as noted in Table 1.  The tracking database handles these properties by accepting equation numbers and correlation coefficients as identified under the VALUES table structure (Table 13).  All temperature dependent coefficients are developed using temperature units of Kelvin.  The temperature range for which the correlation is developed is placed in the value and temperature fields for lower and upper limits (Tmin and Tmax), respectively.  Equations used in the Project ESP database are assigned the same numbers as used by Project 801.  All sets of coefficients are assigned a citation number which identifies the investigator(s) who developed the coefficients.

Although the source material for temperature dependent data is tracked in the tracking database, the database is not designed to accept multiple values for the same chemical, property, and citation.  Thus the tracking database cannot be used as the data repository for the data to determine new correlations.  This is done via an ExcelTM spreadsheet application for which a standard format has been developed.

Where it is appropriate, more than one correlation may be developed to accommodate different behavior in different temperature regimes.  In those cases, more than one record is added to the tracking database.

An ongoing effort is finding data for the temperature dependent properties to enlarge our data sets for developing temperature dependent correlations.  As a rule of thumb, the DIPPR® ESP project will employ the DIPPR® 801 format for all common properties.  For data sets of properties not in common with DIPPR® 801, or those covering a different range, we have developed two additional equation forms and have included them in the Project ESP database as equations 200 and 201.  Correlation coefficients for specific chemicals and properties are being continually added to the Project ESP database.  A minimum of ten data points is needed before we use curve fitting software to generate correlation coefficients.  We are continuing our effort to use Project 912 predictive methods to generate predicted data values to supplement experimental data sets that do not meet the minimum requirement of 10 data points.

As decided at the June 1994 DIPPR® ESP/912 meeting at MTU, and reconfirmed at the June 1999 meeting at MTU, we are concerned principally with a temperature range of 250 to 500 K.  Literature data that fall outside this temperature range are still used in developing the coefficient correlations, if the data fit the appropriate equation form.  MTU personnel review all temperature dependent data plots for expected curve shapes.  Values for Tmin and Tmax are selected based on curve shape, temperature range of available data, and to eliminate any inflection points on the plots of temperature dependent data. It is not recommended that a user of the Project ESP database extrapolate a set of coefficients to a temperature outside the range of Tmin to Tmax .

- 8.0 Estimation Methods

Even after a thorough literature search, gaps still remain for many properties and chemicals.  These gaps are identified by running a completion report as described previously.  Various estimation methods are utilized to fill these gaps.  Values developed from estimation methods are identified in color in the Project ESP software product and details on the method used are provided in the comments.  The estimation technique methods that have already been used in the Project ESP software product are summarized in Table 18 and some are described in more detail briefly below.  Some data gaps remain to be filled using appropriate, validated estimation techniques.  Full documentation of the methods used and their selection criteria have been reported in the 1994 DIPPR® Project 912 Progress Report.

- 8.1 Biochemical Oxygen Demand

The Biochemical Oxygen Demand (BOD) is estimated by substituting it for the Theoretical Oxygen Demand (ThOD) in the equation below.

COD = ThOD						Eqn 4

This provides a potentially overly optimistic value for BOD.  This is especially true because even if a chemical is readily biodegradable, its ThOD and ultimate BOD may be similar, but its 5-day BOD may not.  Therefore a comment is inserted which states "This BOD5 value is estimated from the chemical's ThOD and may be overly optimistic of its biodegradability."

ThOD is the theoretical oxygen demand, or the amount of oxygen stoichiometrically required to oxidize an organic chemical to end products such as CO2 and H2O.  Calculation of ThOD is explained in Section 8.3.

- 8.2 Chemical Oxygen Demand

Chemical Oxygen Demand (COD) is a measure of the oxygen consumed by oxidizing a sample under harsh oxidizing conditions of the COD test.  Under these conditions COD was estimated for the following classes of compounds by equating it to the ThOD

Aromatic compounds:  phenols, chloroquinones, nitroquinones, aromatic acids, aromatic amines, aromatic thiols, polycyclic aromatic hydrocarbons.  The user can have 90% statistical confidence that the COD for these compounds will be within the range of (0.91·ThOD) to (1.0·ThOD).

Nonaromatic compounds:  esters, ethers, nitriles, aldehydes, ketones, acids, glycerine esters.  The user can have 90% statistical confidence that the COD for these compounds will be within the range of (0.92·ThOD) to (1.0·ThOD).

If the COD for a chemical, not included in the above mentioned classes, is required, measure the value if the accuracy is crucial.  The COD test is relatively quick (several hours) and inexpensive.  Many companies have the facilities to perform this test in the labs associated with their wastewater treatment plants. If the correct oxidation end products are assumed, the experimental COD should never be greater than the ThOD.  Therefore a COD equal to ThOD will always be a conservative estimate of oxygen demand.

- 8.3 Theoretical Oxygen Demand

Theoretical Oxygen Demand (ThOD) is estimated for specific organic compounds based on the stoichiometry of a balanced chemical reaction which describes the reaction of the organic chemical with oxygen to produce theoretical inorganic end products.  The following guidelines were utilized to determine the reaction's inorganic end products.  Organic chemicals which contain only carbon and hydrogen were assumed to be converted to CO2 and H2O; organic chemicals which contain carbon, hydrogen, and sulfur were assumed to be converted to CO2, H2O, and SO42-; organic chemicals which contain carbon, hydrogen, and phosphorus were assumed to be converted to CO2, H2O and orthophosphate forms (i.e., H2PO4- or HPO42-); organic chemicals which contain carbon, hydrogen, and halogens (e.g., chlorine, bromine) were assumed to be converted to CO2, H2O and the respective halogen ion (e.g., Cl-, Br-); and, organic chemicals which contain carbon, hydrogen, and nitrogen were assumed to be converted to CO2, H2O, and ammonia.  In the later case the oxygen demand associated with nitrogenous theoretical oxygen demand was found by showing the stoichiometric amount of oxygen required to oxidize the ammonia to nitrate.

An exception to these rules is chemicals with nitro or nitroso groups.  The nitrogen in nitro or nitroso groups will be converted to ammonia under anaerobic, reducing conditions.  Under aerobic conditions the nitro or nitroso groups will go directly to nitrite and subsequently to nitrate.  In order to calculate the carbonaceous ThOD for chemicals with nitro or nitroso groups, the stoichiometry was drawn out by hand assuming nitro or nitroso groups go to nitrate and all other nitrogen goes to ammonia.  For combined carbonaceous and nitrogenous oxygen demand all the nitrogen was assumed to go to nitrate.

- 8.4 Partitioning/Equilibrium Parameters

To fill data gaps in Blocks 2 and 4, the UNIFAC activity coefficient model is being used to provide estimates for those properties involving nonideal liquids.  From fundamental principles, all the partitioning and equilibrium properties are functions of liquid-phase activity coefficients.  Under DIPPR® Project 912, a software algorithm has been developed to implement phase equilibrium and partitioning calculations based on UNIFAC activity coefficients.  Three UNIFAC parameter sets are available in the software: (1) a published UNIFAC-VLE set, derived primarily from vapor-liquid equilibrium data; (2) a published UNIFAC-LLE set recommended for liquid-liquid equilibrium calculations; and (3) the UNIFAC-ENV, a new parameter set developed at MTU and optimized for environmental systems, e.g. dilute aqueous mixtures.   Testing UNIFAC against data compiled under Project ESP has led to a preferred choice of parameter sets for each property.  This ranking is described below.

- 8.4.1 Solubility of a Chemical in Water

Mutual miscibility can be calculated rigorously for any partially miscible binary mixture by assuming solute-rich and solvent-rich phases in contact at equilibrium.  In such a case, the solute is defined as the dissolved organic and the solvent as water.  For a binary system, equal solute and solvent chemical potentials in the two phases at equilibrium give rise to two coupled nonlinear algebraic equations. The two unknowns are the organic solubility in the water-rich phase and the water solubility in the organic-rich phase.  These quantities are found numerically by a computer program based on the UNIFAC liquid-phase activity coefficient model.

- 8.4.2 Solubility of Water in a Chemical

Most models of aqueous solubility focus on the dissolved solute and ignore the mutual miscibility of the solvent in a solute-rich phase at equilibrium.  However, the phase equilibrium (activity coefficient) approach outlined earlier has the advantage of predicting both ends of the binary miscibility tie-line.  The endpoints of the tie-line are referred to as the solubility of the chemical in water and of water in the chemical, respectively.

Recommended UNIFAC Parameters for Solubility: The UNIFAC-LLE parameter set is recommended as the first choice and UNIFAC-ENV as the second choice.  Test calculations have shown comparable accuracy for the two sets.  The UNIFAC-VLE set is used when both LLE and ENV parameters are missing.

- 8.4.3 1-Octanol/Water Partition Coefficient (Kow)

A UNIFAC-based model of 1-octanol/water partitioning is an extension of the general procedure for liquid-liquid equilibrium calculations.  UNIFAC activity coefficients permit phase equilibrium calculations for the binary 1-octanol/water system.  Because the two components are almost immiscible in each other, two phases will form: an 1-octanol-rich phase containing dissolved water, and a water-rich phase containing dissolved 1-octanol.  Once the phase compositions are known, infinite dilution activity coefficients for a third component in each phase can be calculated.  The 1-octanol/water partition coefficient is directly proportional to the ratio of the infinite dilution activity coefficients for a third component distributed between the water-rich and 1-octanol-rich phases.

Summarizing, the 1-octanol/water partition coefficient is calculated from an activity coefficient model (e.g., UNIFAC) by a series of steps:

(1) Perform a binary 1-octanol/water equilibrium calculation at the temperature of interest.  Component 1 is 1-octanol, component 2 is water, and component 3 is the distributing chemical.  The 1-octanol-rich phase is designated Phase I, and the water-rich phase is designated Phase II.

(2) Define Kow as the ratio of the concentrations of a third component distributed between the 1-octanol-rich and water-rich phases.  As the mole fraction of the distributed chemical in the two phases approaches zero (i.e., approaches infinite dilution of component 3), the Kow concentration ratio approaches a constant value.

(3) Determine the total molar densities of Phases I and II from the calculated phase compositions.  The concentration of component 3 in each phase is the product of its mole fraction and the total molar density of that phase.

(4) Use the calculated compositions of Phases I and II to estimate the activity coefficients of component 3 at infinite dilution in each phase.  At equilibrium, component 3 must have equal fugacities in the two phases, so the mole fraction ratio of component 3 in Phases I and II is equal to the activity coefficient ratio in Phases II and I.

(5) Calculate Kow as the ratio of total molar densities of Phases I and II multiplied by the ratio of infinite dilution activity coefficients of component 3 in Phases II and I.  The CLOGP software was also used as a source of estimated values.

- 8.4.4 Soil/Water Partitioning

Soil water partition coefficients normalized to organic carbon, Koc, were estimated from the following relationship which relates Koc to Kow.  This method is valid for chemicals which follow:  1.7  log Kow < 7.0.

log Koc = 0.903 log Kow + 0.094; r2=0.91				Eqn 5

- 8.4.5 Bioconcentration Factor

Estimates are taken from an empirical correlation to log10Kow presented by G. Briggs, Journal of Agricultural and Food Chemistry 29:1050-1059 (1981), or if molecular connectivity indices are available, then R. Kock, pp.207-222 in K.L.E. Kaiser (ed.) QSAR in Environmental Toxicology, D. Reidel Publishing Co., Dordrecht, Netherlands (1984), or if solubility is available, then E.E. Kenaga and C.A.I. Goring, pp. 78-115 in J.G. Eaton, P.R. Parrish, and A.C. Hendricks (eds.) Aquatic Toxicology:  Proceedings of the Third Annual Symposium on Aquatic Toxicology.  ASTM STP 707, ASTM, Philadelphia, PA (1980).

- 8.4.6 Activity Coefficients

To fill data gaps, the UNIFAC algorithm is being used to calculate activity coefficient estimates from chemical structure.  The DIPPR® Project ESP compilation contains infinite dilution activity coefficients for the chemical in water and for water in the chemical.  Because the Henry's constant data upon which UNIFAC-ENV was derived is considered to be of high quality, the infinite dilution activity coefficients for the chemical in water come from the UNIFAC-ENV parameter set.  The UNIFAC-VLE and UNIFAC-LLE parameters are used as second and third selections, respectively.  Generally, we lack validated data for water solubility in chemicals, so the activity coefficient of water in a chemical is computed from the standard UNIFAC-LLE model.  The UNIFAC-VLE parameters are used as a second estimation source, while the UNIFAC-ENV model is used as a last choice.

- 8.4.7 Henry's Law Constant

This is the product of the infinite dilution activity coefficient and the pure component vapor pressure.  The latter is obtained from the temperature-dependent equation in DIPPR® Project 801 or from data compiled under DIPPR® Project ESP.  The UNIFAC-ENV parameter set is the first choice for calculating the infinite dilution activity coefficient, followed by UNIFAC-VLE and UNIFAC-LLE.

- 9.0 Database Software (Sponsor Version and ENVIRON 2001)

- 9.1 Description

A completed objective of Project ESP was to provide a DOS software product incorporating features such as CAS number indexing, literature citations, quality codes, and temperature dependent graphing capabilities.  The first version of this software was developed during the 1992 effort.  The software was written in C language, based on Paradox™ engine, with Hiscreen Pro by Softway screen package, and Quinn-Curtis Science and Engineering graphic package.  The product was designed to run on any 286 or higher IBM compatible computer with 640 RAM and a VGA or EGA monitor.  The software was mouse compatible.  The final DOS software product (Sponsor Version 6.0) was released to the Project ESP Steering Committee in July 1998.

Future public dissemination of Project ESP data in electronic form will be done through a third party vendor, EPCON International (www.epcon.com) within a product entitled ENVIRON2001. ENVIRON2001 utilizes a MS Visual Basic™ Version 5.0 user interface.   The Paradox™ format database structure that has been in use since 1992 by Project ESP was retained for purposes of data entry, data manipulation, and overall database structure.  The VALUES, CITATION, and CHEMICAL Paradox™ format files was translated into MS Access™ format for easier use with both Visual Basic™ interfaces;  DIADEM and the product being developed by EPCON.  The user help file for ENVIRON 2001 gives details of use which should be examined before using this product.

- 9.2 Software Tables

Project ESP sponsors continue to receive the Project ESP database prior to public release.  At present, the sponsors receive all data in Access™ format.  The primary data table, VALUES, contains all data collected within the Project ESP effort.

- Table 1: Property List

| Property Code | Project ESP Description | Technical Steering Committee Description |
| --- | --- | --- |
| Block 1 | Oxygen Demand | Oxygen Demand |
| 1a | Biochemical O2 Demand (BOD) | Biochemical Oxygen Demand (BOD) |
| 1b | Dichromate Chemical O2 Demand (COD) | Chemical Oxygen Demand (COD) |
| 1bp | Permanganate Chemical O2 Demand (COD) | Chemical Oxygen Demand (COD) |
| 1cc | Theoretical O2 Demand, Carbonaceous | Theoretical Oxygen Demand |
| 1cn | Theoretical O2 Demand, Combined (C+N) |  |
| Block 2 | Partitioning Parameters | Partitioning Parameters |
| 2a | 1-Octanol/Water Partitioning | 1-Octanol/Water Partition Coefficient |
| 2b | Soil/Water Partitioning (Kp) | Soil/Water Partition Coefficient |
| 2c | Organic Carbon/Water Partitioning (Koc) | Organic Carbon/Water Partition Coefficient |
| 2d | Bioconcentration Factor | Bioconcentration Factor |
| Block 3 | General Physical Properties | General Physical Properties |
| 3a | Molecular Weight | Molecular Weight |
| B | Liquid Density at 298.15 K | Liquid Density at 298.15 K |
| 3bt | Liquid Density | Liquid Density vs. Temperature |
| 3c | Solubility in Water | Solubility in Water |
| 3d | Melting Point | Melting Point |
| 3e | Normal Boiling Point | Normal Boiling Point (Normal BP) |
| 3f | Vapor Pressure at 298.15 K | Vapor Pressure at 298.15 K |
| 3g | Vapor Pressure | Vapor Pressure vs. Temperature |
| 3h | Molecular Diffusivity in Air | Molecular Diffusivity in Air |
| 3i | Molecular Diffusivity in Water | Molecular Diffusivity in Water |
| 3j | Vapor Viscosity | Vapor Viscosity vs. Temperature |
| 3k | Liquid Viscosity | Liquid Viscosity vs. Temperature |
| 3l | Surface Tension at 298.15 K | Surface Tension at 298.15 K |
| 3lt | Surface Tension | Surface Tension vs. Temperature |
| 3ml | Thermal conductivity, liquid | Thermal Conductivity, liquid vs. Temperature |

- Table 1:  Property List (cont.)

| Property Code | Project ESP Description | Technical Steering Committee Description |
| --- | --- | --- |
| 3mv | Thermal conductivity, vapor | Thermal Conductivity, vapor vs. Temperature |
| 3n | Enthalpy of Formation at 298.15 K | Enthalpy of Formation at 298.15 K |
| 3o | Liquid Heat Capacity | Liquid Heat Capacity vs. Temperature |
| 3p | Vapor Heat Capacity | Vapor Heat Capacity vs. Temperature |
| 3q | Critical Temperature | Critical Temperature |
| 3r | Critical Pressure | Critical Pressure |
| 3s | Critical Volume | Critical Volume |
| 3t | Enthalpy of Vaporization at 298.15 K | Enthalpy of Vaporization at 298.15 K |
| 3tt | Enthalpy of Vaporization | Enthalpy of Vaporization vs. Temperature |
| 3tz | Enthalpy of Vaporization at Normal BP | Enthalpy of Vaporization at Normal BP |
| Block 4 | Vapor-Liquid Equilibrium | Vapor-Liquid Equilibrium |
| 4a | Activity Coefficient of Chemical | Activity Coefficient in Water |
| 4b | Activity Coefficient of Water | Activity Coefficient of Water in Chemical |
| 4c | Aqueous Henry's Law Constant | Henry's Law Constant for Water Mixtures |
| Block 5 | Fire and Explosion Parameters | Fire and Explosion Parameters |
| 5al | Lower Flammability Limit in Air | Flammability Limits in Air vs. Temperature |
| 5au | Upper Flammability Limit in Air |  |
| 5b | Flash Point | Flash Point |
| 5c | Autoignition Temperature | Autoignition Temperature |
| 5d | Enthalpy of Combustion at 298.15 K | Enthalpy of Combustion at 298.15 K |

- Table 1:  Property List (cont.)

| Property Code | Project ESP Description | Technical Steering Committee Description |
| --- | --- | --- |
| Block 6 | Sensory, Health and Toxicity Impacts | Sensory, Health, and Toxicity Impacts |
| 6a-6i | Acute Aquatic Toxicity | Aquatic Toxicity |
| 6ab | Fathead minnow, 48h, EC50 |  |
| 6ac | Fathead minnow, 96h, EC50 |  |
| 6ba | Fathead minnow, 24h, LC50 |  |
| 6bb | Fathead minnow, 48h, LC50 |  |
| 6bc | Fathead minnow, 96h, LC50 |  |
| 6ca | Daphnia magna, 24h, EC50 |  |
| 6cb | Daphnia magna, 48h, EC50 |  |
| 6da | Daphnia magna, 24h, LC50 |  |
| 6db | Daphnia magna, 48h, LC50 |  |
| 6fa | Salmonidae, 24h, LC50 |  |
| 6fb | Salmonidae, 48h, LC50 |  |
| 6fc | Salmonidae, 96h, LC50 |  |
| 6hc | Mysid, 96h, LC50 |  |
| 6i | Other |  |

Table 2:  Default Units and Mathematical Expressions to Convert to Unit Options

| Property Code | Project ESP Description | Default Units | Unit Options |
| --- | --- | --- | --- |
| Block 1 | Oxygen Demand |  |  |
| 1a | Biochemical O2 Demand (BOD) | g O2/g chem | x 1000 = mg O2/g chem |
| 1b | Dichromate Chemical O2 Demand (COD) | g O2/g chem | x 1000 = mg O2/g chem |
| 1bp | Permanganate Chemical O2 Demand (COD) | g O2/g chem | x 1000 = mg O2/g chem |
| 1cc | Theoretical O2 Demand, Carbonaceous | g O2/g chem | x 1000 = mg O2/g chem |
| 1cn | Theoretical O2 Demand, Combined | g O2/g chem | x 1000 = mg O2/g chem |
| Block 2 | Partitioning Parameters |  |  |
| 2a | 1-Octanol/Water Partitioning | Log Kow, Kow is unit-less |  |
| 2b | Soil/Water Partitioning | cm3/g soil |  |
| 2c | Organic Carbon/Water Partitioning | cm3/g OC cm3/g OM |  |
| 2d | Bioconcentration Factor | unit-less |  |
| Block 3 | General Physical Properties |  |  |
| 3a | Molecular Weight | kg/kmol | = g/mol = lb/lbmol |
| 3b | Liquid Density at 298.15 K | kg/m3 | x 0.001 = g/cm3 x 0.001 = g/mL ÷ 3a = kmol/m3 x 0.062435 = lb/ft3 |
| 3bt | Liquid Density | kmol/m3 | x 3a x 0.001 = g/cm3 x 3a x 0.001 = g/mL x 3a = kg/m3 x 3a x 0.062435 = lb/ft3 |
| 3c | Solubility in Water | ppm(wt) | = mg/L ÷ 3a x 1.802 x 10-5 = mol frac. x 10-6 = mass frac. |
| 3d | Melting Point | K | C x 1.8 - 459.67 = F |
| 3e | Normal Boiling Point | K | C x 1.8 - 459.67 = F |

- Table 2: Default Units and Mathematical Expressions to Convert to Unit Options (cont.)

| Property Code | Project ESP Description | Default Units | Unit Options |
| --- | --- | --- | --- |
| 3f | Vapor Pressure at 298.15 K | Pa | ÷ 101325 = atm x 10-5 = bar ÷ 1000 = kPa x 0.02089 = lbf/ft2 x 7.5 x 10-3 = mm Hg x 1.45 x 10-4 = psia x 7.5 x 10-3 = torr |
| 3g | Vapor Pressure | Pa | ÷ 101325 = atm x 10-5 = bar ÷ 1000 = kPa x 0.02089 = lbf/ft2 x 7.5 x 10-3 = mm Hg x 1.45 x 10-4 = psia x 7.5 x 10-3 = torr |
| 3h | Molecular Diffusivity in Air | cm2/s | x 0.155 = in2/s x 1.076 x 10-3 = ft2/s x 10-4 = m2/s |
| 3i | Molecular Diffusivity in Water | cm2/s | x 0.155 = in2/s x 1.076 x 10-3 = ft2/s x 10-4 = m2/s |
| 3j | Vapor Viscosity | Pas | x 1000 = cp x 3600 = kg/mhr x 2419 = lb/fthr x 0.6719 = lb/fts x 0.02089 = slug/fts |
| 3k | Liquid Viscosity | Pas | x 1000 = cp x 3600 = kg/mhr x 2419 = lb/fthr x 0.6719 = lb/fts x 0.02089 = slug/fts |
| 3l | Surface Tension at 298.15 K | N/m | x 1000 = dynes/cm x 0.06852 = lbf/ft x 5.7 x 10-3 = lbf/in |
| 3lt | Surface Tension | N/m | x 1000 = dynes/cm x 0.06852 = lbf/ft x 5.7 x 10-3 = lbf/in |
| 3ml | Thermal Conductivity, liquid | W/mK | x 0.5781 = BTU/fthrF x 2.39 x 10-3 = cal/cmsC x 0.8604 = kcal/mhrC |
| 3mv | Thermal Conductivity, vapor | W/mK | x 0.5781 = BTU/fthrF x 2.39 x 10-3 = cal/cmsC x 0.8604 = kcal/mhrC |

- Table 2: Default Units and Mathematical Expressions to Convert to Unit Options (cont.)

| Property Code | Project ESP Description | DefaultUnits | UnitOptions |
| --- | --- | --- | --- |
| 3n | Enthalpy of Formation at 298.15 K | J/kmol | x 4.299 x 10-4 = BTU/lb x 0.001 = J/mol ÷ 3a x 2.39 x 10-4 = cal/g x 0.1084 = cal/lbmol ÷ 3a x 0.001 = kJ/kg x 0.001 = kJ/kmol |
| 3o | Liquid Heat Capacity | J/kmolK | ÷ 3a x 2.389 x 10-4 = BTU/lbF ÷ 3a x 2.39 x 10-7 = cal/gC ÷ 3a x 0.001 = kJ/kgK x 0.001 = kJ/kmolK |
| 3p | Vapor Heat Capacity | J/kmolK | ÷ 3a x 2.389 x 10-4 = BTU/lbF ÷ 3a x 2.39 x 10-7 = cal/gC ÷ 3a x 0.001 = kJ/kgK x 0.001 = kJ/kmolK |
| 3q | Critical Temperature | K | C x 1.8 - 460 = F |
| 3r | Critical Pressure | Pa | ÷ 101325 = atm x 10-5 = bar ÷ 1000 = kPa x 0.02089 = lbf/ft2 x 7.5 x 10-3 = mm Hg x 1.45 x 10-4 = psia x 7.5 x 10-3 = torr |
| 3s | Critical Volume | m3/kmol | x 1000 = L/kmol x 16.02 = ft3/lbmol |
| 3t | Enthalpy of Vaporization at 298.15 K | J/kmol | x 4.299 x 10-4 = BTU/lb x 0.001 = J/mol ÷ 3a x 2.39 x 10-4 = cal/g x 0.1084 = cal/lbmol ÷ 3a x 0.001 = kJ/kg x 0.001 = kJ/kmol |
| 3tt | Enthalpy of Vaporization vs Temperature | J/kmol | x 4.299 x 10-4 = BTU/lb x 0.001 = J/mol ÷ 3a x 2.39 x 10-4 = cal/g x 0.1084 = cal/lbmol ÷ 3a x 0.001 = kJ/kg x 0.001 = kJ/kmol |
| 3tz | Enthalpy of Vaporization at Normal Boiling Point | J/kmol | x 4.299 x 10-4 = BTU/lb x 0.001 = J/mol ÷ 3a x 2.39 x 10-4 = cal/g x 0.1084 = cal/lbmol ÷ 3a x 0.001 = kJ/kg x 0.001 = kJ/kmol |

- Table 2: Default Units and Mathematical Expressions to Convert to Unit Options (cont.)

| Property Code | Project ESP Description | Default Units | Unit Options |
| --- | --- | --- | --- |
| Block 4 | Vapor-Liquid Equilibrium |  |  |
| 4a | Activity Coefficient of Chemical | unit-less |  |
| 4b | Activity Coefficient of Water | unit-less |  |
| 4c | Aqueous Henry's Law Constant 25°C assumed (Air-water Partition Coeff.) | kPamol/mol | x 1.78 x 10-4 = atmL/mol x 1.78 x 10-7 = atmm3/mol x 9.87 x 10-3 = atmmol/mol x 0.01802 = kPam3/kmol x 0.001 = MPamol/mol x 1000 = Pamol/mol x 7.5 = Torrmol/mol x 1.38 x 105 = unit-less |
| Block 5 | Fire and Explosion Parameters |  |  |
| 5al | Lower Flammability Limit in Air | vol% in air |  |
| 5au | Upper Flammability Limit in Air | vol% in air |  |
| 5b | Flash Point | K | C x 1.8 - 460 = F |
| 5c | Autoignition Temperature | K | C x 1.8 - 460 = F |
| 5d | Enthalpy of Combustion at 298.15 K | J/kmol | x 4.299 x 10-4 = BTU/lb x 0.001 = J/mol ÷ 3a x 2.39 x 10-4 = cal/g x 0.1084 = cal/lbmol ÷ 3a x 0.001 = kJ/kg x 0.001 = kJ/kmol |
| Block 6 | Sensory, Health and Toxicity Impacts |  |  |
| 6a-i | Acute Aquatic Toxicity | mg/L |  |

- Table 3 :Chemical List

C#	CAS #	Description	Molecular Formula (Hill ordering system)

---	---------	-----------------------------------	------------------------

1.0	        75-07-0	Acetaldehyde	C2H4O

2.0	        60-35-5	Acetamide	C2H5NO

3.0	        75-05-8	Acetonitrile	C2H3N

4.0	        98-86-2	Acetophenone	C8H8O

5.0	        53-96-3	2-Acetylaminofluorene	C15H13NO

6.0	      107-02-8	Acrolein	C3H4O

7.0	        79-06-1	Acrylamide	C3H5NO

8.0	        79-10-7	Acrylic Acid	C3H4O2

9.0	      107-13-1	Acrylonitrile	C3H3N

10.0	      107-05-1	Allyl Chloride	C3H5Cl

11.0	        92-67-1	4-Aminobiphenyl	C12H11N

12.0	        62-53-3	Aniline	C6H7N

13.0	        90-04-0	o-Anisidine	C7H9NO

14.0	    1332-21-4	Asbestos

15.0	        71-43-2	Benzene 	C6H6

16.0	        92-87-5	Benzidine	C12H12N2

17.0	        98-07-7	Benzotrichloride	C7H5Cl3

18.0	      100-44-7	Benzyl Chloride	C7H7Cl

19.0	        92-52-4	Biphenyl	C12H10

20.0	      117-81-7	Bis(2-ethylhexyl) Phthalate (DEHP)	C24H38O4

21.0	      542-88-1	Bis(chloromethyl) Ether	C2H4Cl2O

22.0	        75-25-2	Bromoform	CHBr3

23.0	      106-99-0	1,3-Butadiene	C4H6

24.0	      156-62-7	Calcium Cyanamide	CCaN2

25.0	      105-60-2	Caprolactam, 	C6H11NO

26.0	      133-06-2	Captan	C9H8Cl3NO2S

27.0	        63-25-2	Carbaryl	C12H11NO2

28.0	        75-15-0	Carbon Disulfide	CS2

29.0	        56-23-5	Carbon Tetrachloride	CCl4

30.0	      463-58-1	Carbonyl Sulfide	COS

31.0	      120-80-9	Catechol	C6H6O2

32.0	      133-90-4	Chloramben	C7H5Cl2NO2

33.0	        57-74-9	Chlordane	C10H6Cl8

34.0	    7782-50-5	Chlorine	Cl2

35.0	        79-11-8	Chloroacetic Acid	C2H3ClO2

36.0	      532-27-4	2-Chloroacetophenone	C8H7ClO

37.0	      108-90-7	Chlorobenzene	C6H5Cl

38.0	      510-15-6	Chlorobenzilate	C16H14Cl2O3

39.0	        67-66-3	Chloroform	CHCl3

40.0	      107-30-2	Chloromethyl Methyl Ether	C2H5ClO

41.0	      126-99-8	Chloroprene	C4H5Cl

42.0	    1319-77-3	Cresols/Cresylic Acid (isomers & mixture)	C7H8O

43.0	        95-48-7	o-Cresol	C7H8O

44.0	      108-39-4	m-Cresol	C7H8O

45.0	      106-44-5	p-Cresol	C7H8O

46.0	        98-82-8	Cumene	C9H12

47.0	        94-75-7	2,4-D, salts and esters	C8H6Cl2O3

48.0	        72-55-9	DDE	C14H8Cl4

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	--------------------------------- 	------------------------

49.0	      334-88-3	Diazomethane	CH2N2

50.0	      132-64-9	Dibenzofuran	C12H8O

51.0	        96-12-8	1,2-Dibromo-3-chloropropane	C3H5Br2Cl

52.0	        84-74-2	Dibutyl Phthalate	C16H22O4

53.0	      106-46-7	1,4-Dichlorobenzene	C6H4Cl2

54.0	        91-94-1	3,3-Dichlorobenzidene	C12H10Cl2N2

55.0	      111-44-4	Dichloroethyl Ether (Bis(2-chloroethyl) Ether)	C4H8Cl2O

56.0	      542-75-6	1,3-Dichloropropene, mixture	C3H4Cl2

56.1	  10061-01-5	1,3-Dichloropropene, Z (cis)	C3H4Cl2

56.2	  10061-02-6	1,3-Dichloropropene, E (trans)	C3H4Cl2

57.0	        62-73-7	Dichlorvos	C4H7Cl2O4P

58.0	      111-42-2	Diethanolamine	C4H11NO2

59.0	      121-69-7	N,N-Dimethylaniline	C8H11N

59.1	        91-66-7	N,N-Diethylaniline	C10H15N

60.0	        64-67-5	Diethyl Sulfate	C4H10O4S

61.0	      119-90-4	3,3’-Dimethoxybenzidine	C14H16N2O2

62.0              60-11-7	4-(Dimethylamino)azobenzene	C14H15N3

63.0	      119-93-7	3,3'-Dimethylbenzidine	C14H16N2

64.0	        79-44-7	Dimethylcarbamyl Chloride	C3H6ClNO

65.0	        68-12-2	Dimethyl Formamide	C3H7NO

66.0	        57-14-7	1,1-Dimethylhydrazine	C2H8N2

67.0	      131-11-3	Dimethyl Phthalate	C10H10O4

68.0	        77-78-1	Dimethyl Sulfate	C2H6O4S

69.0	      534-52-1	4,6-Dinitro-o-cresol, and salts	C7H6N2O5

70.0	        51-28-5	2,4-Dinitrophenol	C6H4N2O5

71.0	      121-14-2	2,4-Dinitrotoluene	C7H6N2O4

72.0	      123-91-1	1,4-Dioxane (1,4-Diethyleneoxide)	C4H8O2

73.0	      122-66-7	1,2-Diphenylhydrazine	C12H12N2

74.0	      106-89-8	Epichlorohydrin	C3H5ClO

75.0	      106-88-7	1,2-Epoxybutane	C4H8O

76.0	      140-88-5	Ethyl Acrylate	C5H8O2

77.0	      100-41-4	Ethylbenzene	C8H10

78.0	        51-79-6	Ethyl Carbamate (Urethane)	C3H7NO2

79.0	        75-00-3	Ethyl Chloride (Chloroethane)	C2H5Cl

80.0	      106-93-4	Ethylene Dibromide (Dibromoethane)	C2H4Br2

81.0	      107-06-2	Ethylene Dichloride (1,2-Dichloroethane)	C2H4Cl2

82.0	      107-21-1	Ethylene Glycol	C2H6O2

83.0	      151-56-4	Ethylene Imine (Aziridine)	C2H5N

84.0	        75-21-8	Ethylene Oxide	C2H4O

85.0	        96-45-7	Ethylene Thiourea	C3H6N2S

86.0	        75-34-3	Ethylidene Dichloride (1,1-Dichloroethane)	C2H4Cl2

87.0	        50-00-0	Formaldehyde	CH2O

88.0	        76-44-8	Heptachlor	C10H5Cl7

89.0	      118-74-1	Hexachlorobenzene	C6Cl6

90.0	        87-68-3	Hexachlorobutadiene	C4Cl6

91.0	        77-47-4	Hexachlorocyclopentadiene	C5Cl6

92.0	        67-72-1	Hexachloroethane	C2Cl6

93.0	      822-06-0	Hexamethylene-1,6-diisocyanate	C8H12N2O2

94.0	      680-31-9	Hexamethylphosphoramide	C6H18N3OP

95.0	      110-54-3	Hexane	C6H14

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	--------------------------------- 	------------------------

96.0	      302-01-2	Hydrazine	H4N2

97.0	    7647-01-0	Hydrochloric Acid	ClH

98.0	    7664-39-3	Hydrogen Fluoride (Hydrofluoric acid)	FH

99.0	      123-31-9	Hydroquinone	C6H6O2

100.0	        78-59-1	Isophorone	C9H14O

101.0	        58-89-9	Lindane (all isomers)	C6H6Cl6

102.0	      108-31-6	Maleic Anhydride	C4H2O3

103.0	        67-56-1	Methanol	CH4O

104.0	        72-43-5	Methoxychlor	C16H15Cl3O2

105.0	        74-83-9	Methyl Bromide (Bromomethane)	CH3Br

106.0	        74-87-3	Methyl Chloride (Chloromethane)	CH3Cl

107.0	        71-55-6	Methyl Chloroform (1,1,1-trichloroethane)	C2H3Cl3

108.0	        78-93-3	Methyl Ethyl Ketone (2-Butanone)	C4H8O

109.0	        60-34-4	Methylhydrazine	CH6N2

110.0	        74-88-4	Methyl Iodide (Iodomethane)	CH3I

111.0	      108-10-1	Methyl Isobutyl Ketone (Hexone)	C6H12O

112.0	      624-83-9	Methyl Isocyanate	C2H3NO

113.0	        80-62-6	Methyl Methacrylate	C5H8O2

114.0	    1634-04-4	tert-butyl Methyl Ether	C5H12O

115.0	      101-14-4	4,4’-Methylenebis(2-chloroaniline)	C13H12Cl2N2

116.0	        75-09-2	Methylene Chloride (Dichloromethane)	CH2Cl2

117.0	      101-68-8	4,4’-Methylenebis(phenyl isocyanate) (MDI)	C15H10N2O2

118.0	      101-77-9	4,4'-Methylenedianiline	C13H14N2

119.0	        91-20-3	Naphthalene	C10H8

120.0	        98-95-3	Nitrobenzene	C6H5NO2

121.0	        92-93-3	4-Nitrobiphenyl	C12H9NO2

122.0	      100-02-7	4-Nitrophenol	C6H5NO3

123.0	        79-46-9	2-Nitropropane	C3H7NO2

124.0	      684-93-5	N-Nitroso-N-methylurea	C2H5N3O2

125.0	        62-75-9	N-Nitrosodimethylamine	C2H6N2O

126.0	        59-89-2	N-Nitrosomorpholine	C4H8N2O2

127.0	        56-38-2	Parathion	C10H14NO5PS

128.0	        82-68-8	Pentachloronitrobenzene (Quintobenzene)	C6Cl5NO2

129.0	        87-86-5	Pentachlorophenol	C6HCl5O

130.0	      108-95-2	Phenol	C6H6O

131.0	      106-50-3	p-Phenylenediamine	C6H8N2

132.0	        75-44-5	Phosgene	CCl2O

133.0	    7803-51-2	Phosphine	H3P

134.0         7723-14-0	Phosphorus, white	P4

135.0           085-44-9	Phthalic Anhydride	C8H4O3

136.0	    1336-36-3	Polychlorinated Biphenyls (Aroclors)

137.0	    1120-71-4	1,3-Propane Sultone	C3H6O3S

138.0	        57-57-8	beta-Propiolactone	C3H4O2

139.0	      123-38-6	Propionaldehyde	C3H6O

140.0	      114-26-1	Propoxur (Baygon)	C11H15NO3

141.0	        78-87-5	Propylene Dichloride (1,2-Dichloropropane)	C3H6Cl2

142.0	        75-56-9	Propylene Oxide	C3H6O

143.0	        75-55-8	1,2-Propylene Imine	C3H7N

144.0	        91-22-5	Quinoline	C9H7N

145.0	      106-51-4	Quinone	C6H4O2

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

146.0	      100-42-5	Styrene	C8H8

147.0	        96-09-3	Styrene Oxide	C8H8O

148.0	    1746-01-6	2,3,7,8-Tetrachlorodibenzo-p-dioxin	C12H4Cl4O2

149.0	        79-34-5	1,1,2,2-Tetrachloroethane	C2H2Cl4

150.0	      127-18-4	Tetrachloroethylene (Perchloroethylene)	C2Cl4

151.0	    7550-45-0	Titanium Tetrachloride	Cl4Ti

152.0	      108-88-3	Toluene	C7H8

153.0	        95-80-7	2,4-Toluenediamine	C7H10N2

154.0	      584-84-9	Tolylene 2,4-Diisocyanate	C9H6N2O2

155.0	        95-53-4	o-Toluidine	C7H9N

156.0	    8001-35-2	Toxaphene (chlorinated camphene)

157.0	      120-82-1	1,2,4-Trichlorobenzene	C6H3Cl3

158.0	        79-00-5	1,1,2-Trichloroethane	C2H3Cl3

159.0	        79-01-6	Trichloroethylene	C2HCl3

160.0	        95-95-4	2,4,5-Trichlorophenol	C6H3Cl3O

161.0	        88-06-2	2,4,6-Trichlorophenol	C6H3Cl3O

162.0	      121-44-8	Triethylamine	C6H15N

163.0	    1582-09-8	Trifluralin	C13H16F3N3O4

164.0	      540-84-1	2,2,4-Trimethylpentane	C8H18

165.0	      108-05-4	Vinyl Acetate	C4H6O2

166.0	      593-60-2	Vinyl Bromide	C2H3Br

167.0	        75-01-4	Vinyl Chloride	C2H3Cl

168.0	        75-35-4	Vinylidene Chloride (1,1-dichloroethylene)	C2H2Cl2

169.0	    1330-20-7	Xylenes (isomers & mixture)	C8H10

170.0	        95-47-6	o-Xylene	C8H10

171.0	      108-38-3	m-Xylene	C8H10

172.0	      106-42-3	p-Xylene	C8H10

173.0	      627-13-4	Propyl Nitrate	C3H7NO3

174.0	        74-89-5	Methylamine	CH5N

175.0	        74-90-8	Hydrogen Cyanide	CHN

176.0	        74-93-1	Methyl Mercaptan	CH4S

177.0	        75-04-7	Ethylamine	C2H7N

178.0	        75-31-0	Isopropylamine	C3H9N

179.0	        75-52-5	Nitromethane	CH3NO2

180.0	        75-74-1	Tetramethyl Lead	C4H12Pb

181.0	        75-78-5	Dimethyldichlorosilane	C2H6Cl2Si

182.0	        75-79-6	Methyltrichlorosilane	CH3Cl3Si

183.0	        75-91-2	tert-Butyl Hydroperoxide	C4H10O2

184.0	        76-06-2	Chloropicrin	CCl3NO2

185.0	        78-85-3	Methacrolein	C4H6O

186.0	        79-21-0	Peracetic Acid	C2H4O3

187.0	        79-22-1	Methyl Chloroformate	C2H3ClO2

188.0	        79-38-9	Trifluorochloroethylene	C2ClF3

189.0	        79-84-4	Methyl Vinyl Ketone	C4H6O

190.0	        80-15-9	Cumene Hydroperoxide	C9H12O2

191.0	        94-36-0	Dibenzoyl Peroxide	C14H10O4

192.0	        96-10-6	Chlorodiethylaluminum	C4H10AlCl

193.0	        97-00-7	1-Chloro-2,4-dinitrobenzene	C6H3ClN2O4

194.0	        97-02-9	2,4-Dinitroaniline	C6H5N3O4

195.0	      100-01-6	4-Nitroaniline	C6H6N2O2

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

196.0	      105-64-6	Diisopropyl Peroxydicarbonate	C8H14O6

197.0	      105-74-8	Dilauroyl Peroxide	C24H46O4

198.0	      106-96-7	3-Bromopropyne	C3H3Br

199.0	      107-11-9	Allylamine	C3H7N

200.0	      107-44-8	Sarin	C4H10FO2P

201.0	      109-95-5	Ethyl Nitrite	C2H5NO2

202.0	      110-00-9	Furan	C4H4O

203.0	      110-05-4	Di-tert-butyl Peroxide 	C8H18O2

204.0	      110-22-5	Diacetyl Peroxide (Conc>70%)	C4H6O4

205.0	      116-14-3	Tetrafluoroethylene	C2F4

206.0	      124-40-3	Dimethylamine	C2H7N

207.0	      126-98-7	Methacrylonitrile	C4H5N

208.0	      353-50-4	Carbonyl Fluoride	CF2O

209.0	      371-62-0	Ethylene Fluorhydrin	C2H5FO

210.0	      421-20-5	Methyl Fluorosulfate	CH3FO3S

211.0	      453-18-9	Methyl Fluoroacetate	C3H5FO2

212.0	      460-19-5	Cyanogen	C2N2

213.0	      463-51-4	Ketene	C2H2O

214.0	      506-77-4	Cyanogen Chloride	CClN

215.0	      557-20-0	Diethylzinc	C4H10Zn

216.0	      594-42-3	Perchloromethyl Mercaptan	CCl4S

217.0	      614-45-9	tert-Butyl Perbenzoate	C11H14O3

218.0	      675-14-9	Cyanuric Fluoride	C3F3N3

219.0	      684-16-2	Hexafluoroacetone	C3F6O

220.0	    7787-71-5	Bromine Trifluoride	BrF3

221.0	      814-68-6	Acryloyl Chloride	C3H3ClO

222.0	      920-46-7	Methacryloyl Chloride	C4H5ClO

223.0	  33817-91-3	Ethyl Methyl Ketone Peroxide Z (cis)	C8H16O4

223.1	    1338-23-4	Ethyl Methyl Ketone Peroxide (Conc>60%)	C8H16O4

223.2	  33817-92-4 	Ethyl Methyl Ketone Peroxide E (trans)	C8H16O4

224.0	    1558-25-4	Trichloro(chloromethyl)silane	CH2Cl4Si

225.0	    2487-90-3	Trimethoxysilane	C3H10O3Si

226.0	    5714-22-7	Disulfur Decafluoride	F10S2

227.0	    4109-96-0	Dichlorosilane	Cl2H2Si

228.0	    7446-09-5	Sulfur Dioxide	O2S

229.0	    7446-11-9	Sulfur Trioxide, alpha	O3S

229.1	  13771-24-9	Sulfur Trioxide, gamma	O3S

229.2	  27118-91-8	Sulfur Trioxide, beta	O3S

230.0	    7572-29-4	Dichloroacetylene	C2Cl2

231.0	    7601-90-3	Perchloric Acid (Conc.>60%)	ClHO4

232.0	    7616-94-6	Perchloryl Fluoride	ClFO3

233.0	    7637-07-2	Boron Trifluoride	BF3

234.0	    7664-41-7	Ammonia solutions (>44 wt% Ammonia)

234.1	    7664-41-7	Ammonia	H3N

235.0	    7697-37-2	Nitric Acid	HNO3

236.0	    7719-09-7	Thionyl Chloride	Cl2OS

237.0	    7719-12-2	Phosphorus Trichloride	Cl3P

238.0	    7722-84-1	Hydrogen Peroxide (52 wt%)	H2O2

239.0	    7726-95-6	Bromine	Br2

240.0	    7782-41-4	Fluorine	F2

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	-------------------------

241.0	    7783-07-5	Hydrogen Selenide	H2Se

242.0	    7783-41-7	Oxygen Difluoride	F2O

243.0	    7783-54-2	Nitrogen Trifluoride	F3N

244.0	    7783-60-0	Sulfur Tetrafluoride	F4S

245.0	    7783-79-1	Selenium Hexafluoride	F6Se

246.0	    7783-80-4	Tellurium Hexafluoride	F6Te

247.0	    7784-42-1	Arsine	AsH3

248.0	  13446-10-1	Ammonium Permanganate	H4MnNO4

249.0	    7789-30-2	Bromine Pentafluoride	BrF5

250.0	    7790-91-2	Chlorine Trifluoride	ClF3

251.0	    7790-98-9	Ammonium Perchlorate	ClH4NO4

252.0	    7803-49-8	Hydroxylamine	H3NO

253.0	    7803-52-3	Stibine	H3Sb

254.0	    8014-95-7	Oleum (65-85 wt%)	H2O7S2

255.0	    9004-70-0	Cellulose Nitrate (@>12.5% nitrogen)

256.0	  10025-78-2	Trichlorosilane	Cl3HSi

257.0	  10025-87-3	Phosphorus Oxychloride	Cl3OP

258.0	  10028-15-6	Ozone	O3

259.0	  10035-10-6	Hydrogen Bromide	BrH

260.0	  10036-47-2	Tetrafluorohydrazine	F4N2

261.0	  10049-04-4	Chlorine Dioxide	ClO2

262.0	  10102-43-9	Nitric Oxide	NO

263.0	  10102-44-0	Nitrogen Dioxide	NO2

264.0	  10294-34-5	Boron Trichloride	BCl3

265.0	  10544-72-6	Dinitrogen Tetroxide	N2O4

266.0	  10544-73-7	Dinitrogen Trioxide	N2O3

267.0	  13463-39-3	Nickel Carbonyl	C4NiO4

268.0	  13463-40-6	Iron Pentacarbonyl	C5FeO5

269.0	  13637-63-3	Chlorine Pentafluoride	ClF5

270.0	  13863-41-7	Bromine Chloride	BrCl

271.0	  19287-45-7	Diborane	B2H6

272.0	  19624-22-7	Pentaborane	B5H9

273.0	  20816-12-0	Osmium Tetroxide	O4Os

274.0	  27137-85-5	Trichloro(dichlorophenyl)silane	C6H3Cl5Si

275.0	  30674-80-7	Methacroyloxyethyl Isocyanate	C7H9NO3

276.0	        64-19-7	Acetic Acid	C2H4O2

277.0	        75-86-5	Acetone Cyanohydrin	C4H7NO

278.0	        74-86-2	Acetylene	C2H2

279.0	      107-18-6	Allyl Alcohol	C3H6O

280.0	    5332-73-0	3-Methoxypropylamine	C4H11NO

281.0	    7783-70-2	Antimony Pentafluoride	F5Sb

282.0	    7784-34-1	Arsenic Trichloride	AsCl3

283.0	        98-87-3	Benzal Chloride	C7H6Cl2

284.0	        98-16-8	Benzenamine, 3-(Trifluoromethyl)	C7H6F3N

285.0	      140-29-4	Benzyl Cyanide	C8H7N

286.0	      353-42-4	Boron Trifluoride with Methyl Ether (1:1)	C2H6BF3O

287.0	      598-73-2	Bromotrifluorethylene	C2BrF3

288.0	      106-97-8	Butane	C4H10

289.0	  25167-67-3	Butene	C4H8

290.0           106-98-9	1-Butene	C4H8

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

291.0	      107-01-7	2-Butene	C4H8

291.1	      624-64-6	2-Butene, E (trans)	C4H8

291.2	      590-18-1	2-Butene, Z (cis)	C4H8

292.0	        85-68-7	Butyl Benzyl Phthalate	C19H20O4

293.0	    7791-21-1	Dichlorine Monoxide	Cl2O

294.0	        79-04-9	Chloroacetyl Chloride	C2H2Cl2O

295.0	      107-07-3	Chloroethanol	C2H5ClO

296.0	      590-21-6	1-Chloropropylene	C3H5Cl

296.1	  16136-85-9	1-Chloropropylene, E (trans)	C3H5Cl

296.2	  16136-84-8	1-Chloropropylene, Z (cis)	C3H5Cl

297.0	      557-98-2	2-Chloropropylene	C3H5Cl

298.0	    4170-30-3	Crotonaldehyde	C4H6O

298.1	      123-73-9	Crotonaldehyde, E (trans)	C4H6O

298.2	  15798-64-8	Crotonaldehyde, Z (cis)	C4H6O

299.0	      108-91-8	Cyclohexylamine	C6H13N

300.0	        75-19-4	Cyclopropane	C3H6

301.0	        75-37-6	1,1-Difluoroethane	C2H4F2

302.0	    2524-03-0	Dimethyl Chlorothiophosphate	C2H6ClO2PS

303.0	      463-82-1	2,2-Dimethylpropane	C5H12

304.0	      101-84-8	Diphenyl Oxide	C12H10O

305.0	        74-84-0	Ethane	C2H6

306.0	      141-78-6	Ethyl Acetate	C4H8O2

307.0	      107-00-6	Ethyl Acetylene	C4H6

308.0	        60-29-7	Ethyl Ether	C4H10O

309.0	        75-08-1	Ethyl Mercaptan	C2H6S

310.0	        74-85-1	Ethylene	C2H4

311.0	      107-15-3	Ethylenediamine	C2H8N2

312.0	      107-16-4	Formaldehyde Cyanohydrin	C2H3NO

313.0	      764-42-1	Fumaronitrile	C4H2N2

314.0	      142-82-5	Heptane	C7H16

315.0	    1333-74-0	Hydrogen (equilibrium)	H2

316.0	    7783-06-4	Hydrogen Sulfide	H2S

317.0	      103-90-2	4-Hydroxyacetanilide	C8H9NO2

318.0	        99-93-4	4-Hydroxyacetophenone	C8H8O2

319.0	        75-28-5	Isobutane	C4H10

320.0	        78-82-0	Isobutyronitrile	C4H7N

321.0	        78-78-4	Isopentane	C5H12

322.0	        78-79-5	Isoprene	C5H8

323.0	        75-29-6	Isopropyl Chloride	C3H7Cl

324.0	      108-23-6	Isopropyl Chloroformate	C4H7ClO2

325.0	        78-97-7	Lactonitrile	C3H5NO

326.0	      928-53-0	Maleonitrile	C4H2N2

327.0	        79-41-4	Methacrylic Acid	C4H6O2

328.0	        74-82-8	Methane	CH4

329.0	      115-10-6	Methyl Ether	C2H6O

330.0	      107-31-3	Methyl Formate	C2H4O2

331.0	      556-64-9	Methyl Thiocyanate	C2H3NS

332.0	      563-46-2	2-Methyl-1-butene	C5H10

333.0	      563-45-1	3-Methyl-1-butene	C5H10

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

334.0	      109-87-5	Methylal	C3H8O2

335.0	      115-11-7	2-Methylpropene	C4H8

336.0	      100-54-9	Nicotinonitrile	C6H4N2

337.0	      100-00-5	p-Nitrochlorobenzene	C6H4ClNO2

338.0	      836-30-9	p-Nitrodiphenylamine	C12H10NO2

339.0	      504-60-9	1,3-Pentadiene	C5H8

339.1	    2004-70-8	1,3-Pentadiene, E (trans)	C5H8

339.2	    1574-41-0	1,3-Pentadiene, Z (cis)	C5H8

340.0	      109-66-0	Pentane	C5H12

341.0	      109-67-1	1-Pentene	C5H10

341.1	      646-04-8	2-Pentene, E (trans)	C5H10

341.2	      627-20-3	2-Pentene, Z (cis)	C5H10

342.0	      122-79-2	Phenyl acetate	C8H8O2

343.0	    1314-80-3	Phosphorus Pentasulfide	P4S10

344.0	    1314-56-3	Phosphorus Pentoxide	O10P4

345.0	      110-89-4	Piperidine	C5H11N

346.0	      463-49-0	Propadiene	C3H4

347.0	        74-98-6	Propane	C3H8

348.0	      107-12-0	Propionitrile	C3H5N

349.0	      109-61-5	Propyl Chloroformate	C4H7ClO2

350.0	      115-07-1	Propylene	C3H6

351.0	        74-99-7	Propyne	C3H4

352.0	      140-76-1	2-Methyl-5-vinylpyridine	C8H9N

353.0	      289-95-2	Pyrimidine	C4H4N2

354.0	        69-72-7	Salicylic Acid	C7H6O3

355.0	    7803-62-5	Silane	H4Si

356.0	      497-19-8	Sodium Carbonate	CNa2O3

357.0	      110-61-2	Succinonitrile	C4H4N2

358.0	  10545-99-0	Sulfur Dichloride	Cl2S

359.0	  10025-67-9	Sulfur Monochloride	Cl2S2

360.0	    7664-93-9	Sulfuric Acid	H2O4S

361.0	        75-64-9	Tert-Butylamine	C4H11N

362.0	        75-76-3	Tetramethylsilane	C4H12Si

363.0	      509-14-8	Tetranitromethane	CN4O8

364.0	      108-98-5	Thiophenol	C6H6S

365.0	        91-08-7	Toluene 2,6-Diisocyanate	C9H6N2O2

366.0	  26471-62-5	Toluene Diisocyanate (unspecified isomer)	C9H6N2O2

367.0	      110-57-6	trans-1,4-Dichloro-2-butene	C4H6Cl2

368.0	      115-21-9	Trichloroethylsilane	C2H5Cl3Si

369.0	        75-50-3	Trimethylamine	C3H9N

370.0	        75-77-4	Trimethylchlorosilane	C3H9ClSi

371.0	        77-99-6	Trimethylolpropane	C6H14O3

372.0	      689-97-4	Vinyl Acetylene	C4H4

373.0	      109-92-2	Vinyl Ethyl Ether	C4H8O

374.0	        75-02-5	Vinyl Fluoride	C2H3F

375.0	      107-25-5	Vinyl Methyl Ether	C3H6O

376.0	        75-38-7	Vinylidene Fluoride	C2H2F2

377.0	  18600-41-4	3-Methoxypropylamine Hydrochloride	C4H11NOHCl

378.0	      141-43-5	Monoethanolamine	C2H7NO

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

379.0	    2002-24-6	Monoethanolamine Hydrochloride	C2H7NOHCl

380.0	      108-01-0	N,N-Dimethylethanolamine	C4H11NO

381.0	    2498-25-1	N,N-Dimethylethanolamine Hydrochloride	C4H11NOHCl

382.0	    7440-36-0	Antimony	Sb

383.0	    1309-64-4	Antimony(III) Oxide	O3Sb2

384.0	  28300-74-5	Antimony Potassium Tartarate (Hydrate)	C8H4K2O12Sb23H2O

385.0	    7440-38-2	Arsenic	As

386.0	    1327-53-3	Arsenic Trioxide	As2O3

387.0	    7784-46-5	Sodium Arsenite	AsNaO2

388.0	    7440-41-7	Beryllium	Be

389.0	    1304-56-9	Beryllium Oxide	BeO

390.0	    7787-56-6	Beryllium Sulfate Tetrahydrate	BeO4S4H2O

391.0	    7440-43-9	Cadmium	Cd

392.0	  10108-64-2	Cadmium Chloride	CdCl2

393.0	    1306-19-0	Cadmium Oxide	CdO

394.0	    7440-47-3	Chromium	Cr

395.0	    1333-82-0	Chromium Trioxide	CrO3

396.0	    7778-50-9	Potassium Dichromate	Cr2K2O7

397.0	    7440-48-4	Cobalt	Co

398.0	  10026-24-1	Cobalt(II) Sulfate Heptahydrate	CoO4S7H2O

399.0	      109-16-8	2-Picoline	C6H7N

400.0	      108-99-6	3-Picoline	C6H7N

401.0	      108-89-4	4-Picoline	C6H7N

402.0	      110-86-1	Pyridine	C5H5N

403.0	      151-50-8	Potassium Cyanide	CKN

404.0	      506-64-9	Silver Cyanide	AgCN

405.0	      506-61-6	Silver Potassium Cyanide	C2AgKN2

406.0	      143-33-9	Sodium Cyanide	CNNa

407.0	      629-14-1	Ethylene Glycol Diethyl Ether	C6H10O2

408.0	      110-71-4	Ethylene Glycol Dimethyl Ether	C4H10O2

409.0	      111-76-2	Ethylene Glycol Monobutyl Ether	C6H14O2

410.0	      122-99-6	Ethylene Glycol Monophenyl Ether	C8H10O2

411.0	    7439-92-1	Lead	Pb

412.0	    6080-56-4	Lead(II) Acetate, Trihydrate	C4H6O4Pb3H20

413.0	    1309-60-0	Lead Dioxide	O2Pb

414.0	    1335-32-6	Lead(II) Subacetate	C4H10O8Pb3

415.0	    1314-41-6	Red Lead	O4Pb3

416.0	    7439-96-5	Manganese	Mn

417.0	  10034-96-5	Manganese(II) Sulfate, Monohydrate	MnO4SH2O

418.0	    7439-97-6	Mercury	Hg

419.0	    7487-94-7	Mercuric Chloride	HgCl2

420.0	    7440-02-1	Nickel	Ni

421.0	    7791-20-0	Nickel Chloride Hexahydrate	Cl2Ni6H2O

422.0	    1313-99-1	Nickel(II) Oxide	NiO

423.0	    7786-81-4	Nickel Sulfate	NiSO4

424.0	  10101-97-0	Nickel Sulfate Hexahydrate	NiO4S6H2O

425.0	  12674-11-2	Aroclor 1016

426.0	  11104-28-2	Aroclor 1221

427.0	  11141-16-5	Aroclor 1232

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

428.0	  53469-21-9	Aroclor 1242

429.0	  12672-29-6	Aroclor 1248

430.0	  11096-82-5	Aroclor 1260

431.0	  37324-23-5	Aroclor 1262

432.0	  11100-14-4	Aroclor 1268

433.0	      120-12-7	Anthracene	C14H10

434.0	        50-32-8	Benzo[a]pyrene	C20H12

435.0	      218-01-9	Chrysene	C18H12

436.0	        85-01-8	Phenanthrene	C14H10

437.0	      129-00-0	Pyrene	C16H10

438.0	    7440-14-4	Radium	Ra

439.0	  14859-67-7	Radon	Rn

440.0	    7782-49-2	Selenium	Se

441.0	    7783-00-8	Selenious Acid	H2O3Se

442.0	      112-07-2	Ethylene Glycol Monobutyl Ether Acetate	C8H16O3

443.0	      111-15-9	Ethylene Glycol Monoethyl Ether Acetate	C6H12O3

444.0	      112-36-7	Diethylene Glycol Diethyl Ether	C8H18O3

445.0	      111-96-6	Diethylene Glycol Dimethyl Ether	C6H14O3

446.0	    2807-30-9	Ethylene Glycol Monopropyl Ether	C5H12O2

447.0	      110-80-5	Ethylene Glycol Monoethyl Ether	C4H10O2

448.0	      109-86-4	Ethylene Glycol Monomethyl Ether	C3H8O2

449.0	      112-15-2	Diethylene Glycol Monoethyl Ether Acetate	C8H16O4

450.0	      112-34-5	Diethylene Glycol Monobutyl Ether	C8H18O3

451.0	      112-49-2	Triethylene Glycol Dimethyl Ether	C8H18O4

452.0	      111-77-3	Diethylene Glycol Monomethyl Ether	C5H12O3

453.0	      111-90-0	Diethylene Glycol Monoethyl Ether	C6H14O3

454.0	        84-15-1	o-Terphenyl	C18H14

455.0	        92-94-4	p-Terphenyl	C18H14

456.0	        92-06-8	m-Terphenyl	C18H14

457.0	      135-70-6	p-Quaterphenyl	C24H18

458.0	      128-37-0	BHT, 2,6-Di-tert-butyl-p-cresol	C15H24O

459.0	    6484-52-2	Ammonium Nitrate	H4N2O3

460.0	        96-24-2	alpha-Chlorohydrin	C3H7ClO2

461.0	        96-23-1	alpha-Dichlorohydrin	C3H6Cl2O

462.0	      616-23-9	beta-Dichlorohydrin	C3H6Cl2O

463.0	      637-92-3	Ethyl tert-butyl Ether	C6H14O

464.0	      556-52-5	Glycidol	C3H6O2

465.0	        56-55-3	Benz[a]anthracene	C18H12

466.0	      110-49-6	Ethylene Glycol Monomethyl Ether Acetate                 C5H10O3

467.0	      542-92-7	Cyclopentadiene	C5H6

468.0	        77-73-6	Dicyclopentadiene	C10H12

469.0	      617-94-7	Dimethylbenzyl Alcohol	C9H12O

470.0	      141-79-7	Mesityl Oxide	C6H10O

471.0	      116-09-6	Hydroxyacetone	C3H6O2

472.0	    7732-18-5	Water 	H2O

473.0	    3037-72-7	(4-Aminobutyl)diethoxymethylsilane	C9H23NO2Si

474.0	      956-36-1	2,4-Trimethylbenzene	C9H12

475.0	        95-50-1	1,2-Dichlorobenzene	C6H4Cl2

476.0	      540-59-0	1,2-Dichloroethylene (unspecified cis- or trans-)	C2H2Cl2

477.0	        88-05-1	2,4,6-Trimethylaniline	C9H13N

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

478.0	      120-83-2	2,4-Dichlorophenol	C6H4Cl2O

479.0	      606-20-2	2,6-Dinitrotoluene	C7H6N2O4

480.0	        95-57-8	2-Chlorophenol	C6H5ClO

481.0	        90-43-7	2-Phenylphenol	C12H10O

482.0	      504-24-5	4-Aminopyridine	C5H6N2

483.0	        95-83-0	4-Chloro-o-phenylenediamine	C6H7ClN2

484.0	    3697-24-3	5-Methylchrysene	C19H14

485.0	      134-32-7	alpha-Naphthylamine	C10H9N

486.0	        83-32-9	Acenaphthene	C12H10

487.0	      208-96-8	Acenaphthylene	C12H8

488.0	        67-64-1	Acetone	C3H6O

489.0	      111-69-3	Adiponitrile	C6H8N2

490.0	      116-06-3	Aldicarb	C7H14N2O2S

491.0	      309-00-2	Aldrin	C12H8Cl6

492.0	    1344-28-1	Aluminum Oxide	Al2O3

493.0	    7787-36-2	Barium Permanganate	BaMn2O8

494.0	  7783-20-2	Ammonium Sulfate	H8N2O4S

495.0	      71-41-0	Amyl Alcohol	C5H12O

496.0	  1303-28-2	Arsenic Pentoxide	As2O5

497.0	  1912-24-9	Atrazine	C8H14ClN5

498.0	      91-59-8	beta-Naphthylamine	C10H9N

499.0	  7440-39-3	Barium	Ba

500.0	      55-21-0	Benzamide	C7H7NO

501.0	    598-31-2	Bromoacetone	C3H5BrO

502.0	    141-32-2	Butyl Acrylate	C7H12O2

503.0	      78-92-2	Butyl Alcohol, sec-	C4H10O

504.0	    123-72-8	Butyraldehyde	C4H8O

505.0	  7778-44-1	Calcium Arsenate  	As2Ca3O8

506.0	16887-00-6	Chloride ion	Cl-

507.0	18540-29-9	Chromium ion (hexavalent)	Cr6+

508.0	10025-73-7	Chromium(III) Chloride 	Cl3Cr

509.0	  7440-50-8	Copper	Cu

510.0	      57-12-5	Cyanide ion	CN-

511.0	    110-82-7	Cyclohexane	C6H12

512.0	    111-92-2	Dibutylamine	C8H19N

513.0	  2050-92-2	Dipentylamine	C10H23N

514.0	    541-73-1	1,3-Dichlorobenzene	C6H4Cl2

515.0	      75-27-4	Bromodichloromethane	CHBrCl2

516.0	    149-74-6	Dichloromethylphenylsilane	C7H8Cl2Si

517.0	      75-71-8	Dichlorodifluoromethane	CCl2F2

518.0	    156-59-2	cis-1,2-Dichloroethylene	C2H2Cl2

518.1	    156-60-5	trans-1,2-Dichloroethylene	C2H2Cl2

519.0	      60-57-1	Dieldrin	C12H8Cl6O

520.0	      84-66-2	Diethyl Phthalate	C12H14O4

521.0	    109-89-7	Diethylamine	C4H11N

522.0	  1615-80-1	1,2-Diethylhydrazine	C4H12N2

523.0	    674-82-8	Diketene	C4H4O2

524.0	      75-18-3	Dimethyl Sulfide	C2H6S

525.0	      67-68-5	Dimethyl Sulfoxide	C2H6OS

526.0	      64-17-5	Ethanol	C2H6O

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

527.0	      86-73-7	Fluorene	C13H10

528.0	    144-49-0	Fluoroacetic Acid	C2H3FO2

529.0	      75-12-7	Formamide	CH3NO

530.0	      98-01-1	Furfural	C5H4O2

531.0	    124-09-4	Hexamethylenediamine	C6H16N2

532.0	  7439-89-6	Iron	Fe

533.0	      78-84-2	Isobutyraldehyde	C4H8O

534.0	      67-63-0	Isopropyl Alcohol	C3H8O

535.0	    301-04-2	Lead(II) Acetate	C4H6O4Pb

536.0	  7446-27-7	Lead Phosphate	O8P2Pb3

537.0	    121-75-5	Malathion	C10H19O6PS2

538.0	21908-53-2	Mercuric Oxide	HgO

539.0	01600-27-7	Mercuric Acetate	C4H6HgO4

540.0	16752-77-5	Methomyl	C5H10N2O2S

541.0	      96-33-3	Methyl Acrylate	C4H6O2

542.0	    624-92-0	Methyl Disulfide	C2H6S2

543.0	      75-54-7	Dichloromethylsilane	CH4Cl2Si

544.0	      74-95-3	Methylene Bromide	CH2Br2

545.0	  1313-27-5	Molybdenum Trioxide	MoO3

546.0	    123-86-4	Butyl Acetate	C6H12O2

547.0	      71-36-3	Butyl Alcohol	C4H10O

548.0       1116-54-7	N-Nitrosodiethanolamine	C4H10N2O3

549.0	      86-30-6	N-Nitrosodiphenylamine	C12H10N2O

550.0	      54-11-5	Nicotine	C10H14N2

551.0	      55-63-0	Nitroglycerin	C3H5N3O9

552.0	      88-75-5	2-Nitrophenol	C6H5NO3

553.0	    108-03-2	1-Nitropropane	C3H7NO2

554.0	    111-36-4	Butyl Isocyanate	C5H9NO

555.0	  4685-14-7	Paraquat ion	C12H14N2

556.0	  2074-50-2	Paraquat Methylsulfate	C14H20N2O8S2

557.0	  8030-30-6	Petroleum Naphtha

558.0	    103-71-9	Phenyl Isocyanate	C7H5NO

559.0	    103-85-5	Phenylthiourea	C7H8N2S

560.0	  7664-38-2	Phosphoric Acid	H3O4P

561.0	10026-13-8	Phosphorus Pentachloride	Cl5P

562.0	      88-89-1	Picric Acid	C6H3N3O7

563.0	  7758-01-2	Potassium Bromate	BrKO3

564.0	    625-55-8	Isopropyl Formate	C4H8O2

565.0	    109-90-0	Ethyl Isocyanate	C3H5NO

566.0	    108-20-3	Isopropyl Ether	C6H14O

567.0	      81-07-2	Saccharin	C7H5NO3S

568.0	  7631-89-2	Sodium Arsenate (mixture mono,di,tri basic salts)

569.0	  1310-73-2	Sodium Hydroxide	HNaO

570.0	      57-24-9	Strychnine	C21H22N2O2

571.0	    100-21-0	Terephthalic Acid	C8H6O4

572.0	      75-65-0	tert-Butyl Alcohol	C4H10O

573.0	      78-00-2	Tetraethyllead	C8H20Pb

574.0	    109-99-9	Tetrahydrofuran	C4H8O

575.0	      62-56-6	Thiourea	CH4N2S

576.0	13463-67-7	Titanium Dioxide	O2Ti

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

577.0	      98-13-5	Trichlorophenylsilane	C6H5Cl3Si

578.0	    998-30-1	Triethoxysilane	C6H16O3Si

579.0	  7440-62-2	Vanadium (fume or dust)	V

580.0	  1314-62-1	Vanadium Pentoxide	O5V2

581.0	    628-63-7	Pentyl Acetate	C7H14O2

582.0	      71-23-8	1-Propanol	C3H8O

583.0	      70-69-9	Aminopropiophenone, 4’-	C9H11NO

584.0	  1341-49-7	Ammonium Bifluoride	F2H5N2

585.0	12125-02-9	Ammonium Chloride	ClH4N

586.0	12125-01-8	Ammonium Fluoride	FH4N

587.0	10022-31-8	Barium Nitrate	BaN2O6

588.0	13813-79-1	Boric Acid	BH3O3

589.0	    109-72-8	Butyllithium	C4H9Li

590.0	  7778-54-3	Calcium Hypochlorite	CaCl2O2

591.0	    124-38-9	Carbon Dioxide	CO2

592.0	    630-08-0	Carbon Monoxide	CO

593.0	  9004-36-8	Cellulose Acetate Butyrate

594.0	  7738-94-5	Chromic(VI) Acid	CrH2O4

595.0	13530-68-2	Dichromic Acid	Cr2H2O7

596.0	    544-92-3	Copper(I) Cyanide	CCuN

597.0	    108-93-0	Cyclohexanol	C6H12O

598.0	    124-18-5	Decane	C10H22

599.0	    621-64-7	Dipropylnitrosamine	C6H14N2O

600.0	39156-41-7	Diaminoanisole Sulfate, 2,4-	C7H12N2O5S

601.0	    101-80-4	Diaminodiphenyl Ether, 4,4’-	C12H12N2O

602.0	    111-46-6	Diethylene Glycol	C4H10O3

603.0	    105-10-2	N,N’-Dimethyl-p-phenylenediamine	C8H12N2

604.0	33204-76-1	Diphenylhexamethylcyclotetrasiloxane, 2,6-cis	C18H28O4Si4

605.0	      72-20-8	Endrin	C12H8Cl6O

606.0	    379-79-3	Ergotamine Tartrate	C35H38N5O8

607.0	  9004-57-3	Ethyl Cellulose

608.0	    538-07-8	Ethylbis(2-chloroethyl)amine	C6H13Cl2N

609.0	      64-18-6	Formic Acid	CH202

610.0	  7782-42-5	Graphite	C

611.0	18406-41-2	Hexakis(methoxy)disilylethane	C8H22O6Si2

612.0	    109-49-9	Hexen-2-one, 5-	C6H10O

613.0	    118-93-4	Hydroxyacetophenone, 2-	C8H8O2

614.0	15687-27-1	Ibuprofen	C13H18O2

615.0	  7553-56-2	Iodine	I2

616.0	    110-19-0	Isobutyl Acetate	C6H12O2

617.0	      78-83-1	Isobutyl Alcohol	C4H10O

618.0	  8008-20-6	Kerosene

619.0	      50-21-5	Lactic Acid	C3H6O3

619.1	    598-82-3	dl-Lactic Acid	C3H6O3

619.2           79-33-4	L, S, & d-Lactic Acid	C3H6O3

619.3         326-41-7	D & l-Lactic Acid	C3H6O3

620.0	12141-20-7	Lead Oxide Phosphate	HO5PPb3

621.0	68476-85-7	Liquefied Petroleum Gas

622.0	10377-66-9	Manganous Nitrate	MnN2O6

623.0	    872-50-4	Methyl-2-pyrrolidinone, 1-	C5H9NO

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

624.0	    108-87-2	Methylcyclohexane	C7H14

625.0	    101-61-1	Methylenebis(N,N-dimethyl) benzeneamine, 4,4’-	C17H22N2

626.0	64091-91-4	Methylnitrosoamino-1-(3-pyridyl)-1-butanone	C10H13N3O2

627.0	  1317-33-5	Molybdenum Disulfide	MoS2

628.0	  4835-11-4	N,N’-Dibutylhexamethylenediamine	C14H32N2

629.0	  3710-84-7	N,N-Diethylhydroxylamine	C4H11NO

630.0	04062-60-6	N,N’-Di-tert-butylethylenediamine	C10H24N2

631.0	18328-90-0	N-Ethyl-2-methylallylamine	C6H13N

632.0	  2424-01-3	N-Methyldiallylamine	C7H13N

633.0	    924-16-3	N-Nitrosodibutylamine	C8h18N2O

634.0	      55-18-5	N-Nitrosodiethylamine	C4H10N2O

635.0	10595-95-6	N-Nitrosomethylethylamine	C3H8N2O

636.0	  4549-40-0	N-Nitrosomethylvinylamine	C3H6N2O

637.0	    373-02-4	Nickel(II) Acetate	C4H6NiO4

638.0	  7718-54-9	Nickel(II) Chloride	Cl2Ni

639.0	13770-89-3	Nickel(II) Sulfamate	H4N2NiO6S2

640.0	    111-84-2	Nonane	C9H20

641.0	    111-65-9	Octane	C8H18

642.0	30525-89-4	Paraformaldehyde	(CH2O)x

643.0	  2570-26-5	Pentadecylamine	C15H33N

644.0	  9002-84-0	Polytetrafluoroethylene	(C2F4)x

645.0	    877-24-7	Potassium Acid Phthalate	C8H5KO4

646.0	    584-08-7	Potassium Carbonate	CK2O3

647.0	13746-66-2	Potassium Ferricyanide	C6FeK3N6

648.0	16923-95-8	Potassium Fluorozirconate	F6K2Zr

649.0	13967-50-5	Potassium Gold(I) Cyanide	C2AuKN2

650.0	  1310-58-3	Potassium Hydroxide	HKO

651.0	12142-33-5	Potassium Stannate	K2O3Sn

652.0	    107-19-7	Propargyl Alcohol	C3H4O

653.0	    123-75-1	Pyrrolidine	C4H9N

654.0	35574-23-3	Pyrrolidium Acetate	C6H13NO2

655.0   112945-52-5	Silica (fumed)	O2Sn

656.0	    127-09-3	Sodium Acetate	C2H3O2Na

657.0	    144-55-8	Sodium Bicarbonate	CHNaO3

658.0	  7647-14-5	Sodium Chloride	ClNa

659.0	  6132-04-3	Sodium Citrate (dihydrate)	C6H5Na3O7

660.0	10588-01-9	Sodium Dichromate	Cr2Na2O7

661.0	13755-29-8	Sodium Fluoroborate	BF4Na

662.0	16721-80-5	Sodium Hydrosulfide	HNaS

663.0	  7681-52-9	Sodium Hypochlorite	ClNaO

664.0	  7681-53-0	Sodium Hypophosphite	H2NaO2P

665.0	  7632-00-0	Sodium Nitrite	NNaO2

666.0	  1344-08-7	Sodium Polysulfide	Na2Sx

667.0	16893-85-9	Sodium Silicofluoride	F6Na2Si

668.0	12201-54-6	Sodium Stannate	Na4O4Sn

669.0	  7757-82-6	Sodium Sulfate	Na2O4S

670.0	  1313-82-2	Sodium Sulfide	Na2S

671.0	    126-33-0	Sulfolane	C4H8O2S

672.0	  7704-24-9	Sulfur	S

673.0	14807-96-6	Talc	H2Mg3O12Si4

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

674.0	    112-60-7	Tetraethylene  Glycol	C8H18O5

675.0	    112-27-6	Triethylene Glycol	C6H14HO4

676.0	    555-77-1	Tris(2-Chloroethyl)amine	C6H12Cl3N

677.0	  7646-85-7	Zinc Chloride	Cl2Zn

678.0	13530-65-9	Zinc Chromate	CrO4Zn

679.0	    557-21-1	Zinc Cyanide	C2N2Zn

680.0	  1314-13-2	Zinc Oxide	OZn

681.0	10026-11-6	Zirconium (IV) Chloride	Cl4Zr

682.0	    111-87-5	1-Octanol	C8H18O

683.0	      75-98-9	Propanoic Acid, 2, 2-Dimethyl-	C5H10O2

684.0	    156-10-5	p-Nitrosodiphenylamine	C12H10N2O

704.0	    108-24-7	Acetic Anhydride	C4H6O3

705.0	    124-04-9	Adipic Acid	C6H10O4

706.0	11097-69-1	Aroclor 1254

707.0	      65-85-0	Benzoic Acid	C7H6O2

708.0	  2467-02-9	Bis(2-Hydroxyphenyl)methane	C13H12O2

709.0	    107-88-0	1,3-Butanediol	C4H10O2

710.0	    110-63-4	1,4-Butanediol	C4H10O2

711.0	  4435-50-1	1,2,3-Butanetriol	C4H10O3

712.0	    592-84-7	Butyl Formate	C5H10O2

713.0	    590-01-2	Butyl Propionate	C7H14O2

714.0	    107-92-6	Butyric Acid	C4H8O2

715.0	      77-92-9	Citric Acid	C6H8O7

716.0	    592-57-4	1,3-Cyclohexadiene	C6H8

717.0	    110-83-8	Cyclohexane	C6H10

718.0	    292-64-8	Cyclooctane	C8H16

719.0	    287-92-3	Cyclopentane	C5H10

720.0	      50-29-3	DDT	C14H9Cl5

721.0	    872-05-9	1-Decene	C10H20

722.0	  5989-27-5	d-Limonene	C10H16

723.0	13029-08-8	2,2’-Dichlorobiphenyl	C12H8Cl2

724.0	  2050-68-2	4,4’-Dichlorobiphenyl	C12H8Cl2

725.0	    100-37-8	N,N-Diethylethanolamine	C6H15NO

726.0	    624-72-6	1,2-Difluoroethane	C2H4F2

727.0	      75-83-2	2,2-Dimethylbutane	C6H14

728.0	  2207-01-4	cis-1,2- Dimethylcyclohexane	C8H16

729.0	    638-04-0	cis-1,3- Dimethylcyclohexane	C8H16

730.0	    624-29-3	cis-1,4- Dimethylcyclohexane	C8H16

731.0	    581-42-0	2,6-Dimethylnaphthalene	C12H12

732.0	    582-16-1	2,7-Dimethylnaphthalene	C12H12

733.0	14426-20-1	2-Diethylaminoethanol hydrochloride	C6H15NO·ClH

734.0	  1678-91-7	Ethylcyclohexane	C8H16

735.0	  3553-80-8	Ethyl Diethylcarbamate	C7H15NO2

736.0	    109-94-4	Ethyl Formate	C3H6O2

737.0	    110-17-8	Fumaric Acid	C4H4O4

738.0	      96-48-0	Gamma-Butyrolactone	C4H6O2

739.0	    591-50-4	Iodobenzene	C6H5I

740.0	    107-08-4	1-Iodopropane	C3H7I

741.0	    110-16-7	Maleic Acid	C4H4O4

742.0	    141-82-2	Malonic Acid	C3H4O4

- Table 3: Chemical List (cont.)

C#	CAS #	Description	Molecular Formula

---	----------	---------------------------------	------------------------

743.0	      79-20-9	Methyl Acetate	C3H6O2

744.0	    547-63-7	Methyl Isobutyrate	C5H10O2

745.0	    623-42-7	Methyl Butyrate	C5H10O2

746.0	    590-67-0	1-Methylcyclohexanol	C7H14O

747.0	      90-12-0	1-Methylnaphthalene	C11H10

748.0	      91-57-6	2-Methylnaphthalene	C11H10

749.0	    691-38-3	4-Methyl-2-pentene	C6H12

750.0	    100-80-1	m-Methylstyrene	C9H10

751.0	    622-97-9	p-Methylstyrene	C9H10

752.0	    616-44-4	3-Methylthiophene	C5H6S

753.0	      88-74-4	o-Nitroaniline	C6H6N2O2

754.0	      79-24-3	Nitroethane	C2H5NO2

755.0	    111-66-0	1-Octene	C8H16

756.0	    144-62-7	Oxalic Acid	C2H2O4

757.0	    110-85-0	Piperazine	C4H10N2

758.0	10430-90-7	1-Piperazinecarboxylic Acid	C5H10N2O2

759.0	  2762-32-5	2-Piperazinecarboxylic Acid	C5H10N2O2

760.0	  7709-80-0	1,4-Piperazinedicarboxylic Acid	C8H15NO

761.0	    142-64-3	Piperazine Dihydrochloride	C4H10N2·2ClH

762.0	  6091-62-9	Piperazine Dihydrochloride Monohydrate	C4H10N2·2ClH·H2O

763.0	      57-55-6	1,2-Propanediol	C3H8O2

764.0	    504-63-2	1,3-Propanediol	C3H8O2

765.0	      79-09-4	Propionic Acid	C3H6O2

766.0	    106-36-5	Propyl Propionate	C6H12O2

767.0	    108-30-5	Succinic Anhydride	C4H4O3

768.0	    586-62-9	Terpinolene	C10H16

769.0	12408-10-5	Tetrachlorobenzene (general)	C6H2Cl4

769.1	    634-66-2	1,2,3,4- Tetrachlorobenzene	C6H2Cl4

769.2	      95-94-3	1,2,4,5- Tetrachlorobenzene	C6H2Cl4

769.3	    634-90-2	1,2,3,5- Tetrachlorobenzene	C6H2Cl4

770.0	26914-33-0	Tetrachlorobiphenyl (general)	C12H6Cl4

771.0	    811-97-2	1,1,1,2- Tetrafluoroethane	C2H2F4

772.0	    359-35-3	1,1,2,2- Tetrafluoroethane	C2H2F4

773.0	    119-64-2	Tetralin	C10H12

774.0	    108-70-3	1,3,5- Trichlorobenzene	C6H3Cl3

775.0	25323-68-6	Trichlorobiphenyl (general)	C12H7Cl3

776.0	16747-26-5	2,2,4- Trimethylhexane	C9H20

777.0	16747-28-7	2,3,3- Trimethylhexane	C9H20

778.0	16747-31-2	3,3,4- Trimethylhexane	C9H20

779.0	    106-43-4	Benzene, 1-chloro-4-methyl	C7H7Cl

780.0	    620-14-4	Benzene, 1-ethyl-3-methyl	C9H12

781.0	25619-60-7	Benzene, tetramethyl- (general)	C10H14

781.1	    527-53-7	1,2,3,5- Tetramethylbenzene	C10H14

781.2	    488-23-3	1,2,3,4- Tetramethylbenzene	C10H14

782.0	      98-83-9	Benzene, (1-methylethenyl)-	C9H10

783.0	    108-67-8	Benzene, 1,3,5-trimethyl-	C9H12

784.0	      99-87-6	Benzene, 1-methyl-4-(1-methylethyl)-	C10H14

785.0	      95-93-2	Benzene, 1,2,4,5-tetramethyl-	C10H14

786.0	    526-73-8	Benzene, 1,2,3,-trimethyl-	C9H12

787.0	55880-77-8	Pentachlor-1,3-butadiene (general)	C4HCl5

- Table 3: Chemical List (cont.)

C#	CAS#	Description	Molecular Formula

----	----------	----------------------------------------------	--------------------------

788.0	    464-06-2	Butane, 2,2,3-trimethyl-	C7H16

789.0	  1502-38-1	Cyclooctane, methyl-	C9H18

790.0	      96-37-7	Cyclopentane, methyl-	C6H12

791.0	    630-20-6	Ethane, 1,1,1,2- tetrachloro-	C2H2Cl4

792.0	    589-34-4	Hexane, 3-methyl-	C7H16

792.1	  6131-24-4	Hexane, 3-Methyl-, (S)-	C7H16

792.2	78918-91-9	Hexane, 3-Methyl-, (R)-  	C7H16

793.0	28699-88-9	Naphthalene, dichloro- (general)	C10H6Cl2

794.0	    593-45-3	Octadecane	C18H38

795.0	  2216-33-3	Octane, 3-methyl-	C9H20

796.0	    107-83-5	Pentane, 2-methyl-	C6H14

797.0	    576-26-1	Phenol, 2,6-dimethyl-	C8H10O

798.0	34652-54-5	Propane, bromochloro- (general)	C3H6BrCl

799.0	67708-83-2	Propane, dibromochloro- (general)	C3H5Br2Cl

800.0	    554-14-3	Thiophene, 2-methyl-	C5H6S

801.0	    110-15-6	Succinic Acid	C4H6O4

802.0	    629-73-2	1-Hexadecene	C16H32

803.0	  2532-58-3	cis-1,3-Dimethylcyclopentane	C7H14

804.0	    822-50-4	trans-1,2-Dimethylcyclopentane	C7H14

805.0	  1759-58-6	trans-1,3-Dimethylcyclopentane	C7H14

806.0	  1638-26-2	1,1-Dimethylcyclopentane	C7H14

807.0	    112-47-0	1,10-Decanediol	C10H22O2

808.0	  7440-21-3	Silicon	Si

809.0	    590-19-2	1,2-Butadiene	C4H6

811.0	    591-95-7	1,2-Pentadiene	C5H8

812.0	    646-06-0	1,3-Dioxolane	C3H6O2

813.0	26519-91-5	Methyl-1,3-cyclopentadiene	C6H8

814.0	    628-41-1	1,4-Cyclohexadiene	C6H8

815.0	    591-93-5	1,4-Pentadiene	C5H8

816.0	13048-33-4	1,6-Hexanediol Diacrylate	C12H18O4

817.0	    109-79-5	1-Butanethiol	C4H10S

818.0	10026-04-7	Silicon Tetrachloride	Cl4Si

821.0	    592-76-7	1-Heptene	C7H14

823.0	36653-82-4	1-Hexadecanol	C16H34O

825.0	    592-41-6	1-Hexene	C6H12

826.0	    947-19-3	1-Hydroxycyclohexyl Phenyl Ketone	C13H16O2

829.0	  2885-00-9	1-Octadecanethiol	C18H38S

832.0	    622-96-8	1-Ethyl-4-methylbenzene	C9H12

838.0	24650-42-8	2,2-Dimethoxy-2-phenylacetophenone	C16H16O3

839.0	  4032-93-3	2,3,6-Trimethylheptane	C10H22

840.0	    513-85-9	2,3-Butanediol	C4H10O2

841.0	  5341-95-7	meso-2,3-Butanediol	C4H10O2

842.0           79-29-8	2,3-Dimethylbutane	C6H14

843.0	    565-59-3	2,3-Dimethylpentane	C7H16

844.0	    591-96-8	2,3-Pentadiene	C5H8

845.0	    108-08-7	2,4-Dimethylpentane	C7H16

846.0	      79-74-3	2,5-Di-tert-pentylhydroquinone	C16H26O2

847.0	      75-26-3	2-Bromopropane	C3H7Br

849.0	  3404-71-5	2-Ethyl-1-pentene	C7H14

850.0	    543-49-7	2-Heptanol	C7H16O

- Table 3: Chemical List (cont.)

C#	CAS#	Description	Molecular Formula

----	----------	----------------------------------------------	--------------------------

851.0	    110-43-0	2-Heptanone	C7H14O

852.0	    611-14-3	1-Ethyl-2-methylbenzene	C9H12

853.0	26741-29-7	2-Hexadecene	C16H32

854.0	    626-93-7	2-Hexanol	C6H14O

855.0	      75-30-9	2-Iodopropane	C3H7I

856.0	    763-29-1	2-Methyl-1-pentene	C6H12

857.0	    591-76-4	2-Methylhexane	C7H16

858.0	    611-15-4	2-Methylstyrene	C9H10

860.0	    123-96-6	2-Octanol	C8H18O

861.0	  6032-29-7	2-Pentanol	C5H12O

863.0	  4032-92-2	2,4,4-Trimethylheptane	C10H22

864.0	  4032-86-4	3,3-Dimethylheptane	C9H20

865.0	    562-49-2	3,3-Dimethylpentane	C7H16

866.0	  4038-04-4	3-Ethyl-1-pentene	C7H14

867.0	15869-80-4	3-Ethylheptane	C9H20

868.0	    619-99-8	3-Ethylhexane	C8H18

869.0	    617-78-7	3-Ethylpentane	C7H16

870.0	    589-38-8	3-Hexanone	C6H12O

871.0	      96-14-0	3-Methylpentane	C6H14

872.0	  3404-61-3	3-Methyl-1-hexene	C7H14

873.0	    760-20-3	3-Methyl-1-pentene	C6H12

874.0	  7784-13-6	Aluminum Chloride (6-Hydrate)	AlCl3·6H2O

875.0	    584-02-1	3-Pentanol	C5H12O

876.0	    123-19-3	4-Heptanone	C7H14O

877.0	    691-38-3	cis-4-Methyl-2-pentene	C6H12

878.0	    691-37-2	4-Methyl-1-pentene	C6H12

879.0	  3178-29-8	4-Propylheptane	C10H22

880.0	      98-29-3	4-tert-Butylcatechol	C10H14O2

881.0	    103-84-4	Acetanilide	C8H9NO

882.0	    123-54-6	Acetylacetone	C5H8O2

883.0	    260-94-6	Acridine	C13H9N

884.0	  7446-70-0	Aluminum Chloride (Anhydrous)	AlCl3

885.0	21645-51-2	Aluminum Hydroxide	AlH3O3

886.0	    145-08-8	1-Nonanol	C9H20O

887.0	    100-66-3	Anisole	C7H8O

888.0	      84-65-1	Anthraquinone	C14H8O2

889.0	    225-11-6	Benz(a)acridine	C14H11N

890.0	    100-52-7	Benzaldehyde	C7H6O

891.0	    207-08-9	Benzo(k)fluoranthene	C20H12

892.0	    100-47-0	Benzonitrile	C7H5N

893.0	    140-11-4	Benzyl Acetate	C9H10O2

894.0	    100-51-6	Benzyl Alcohol	C7H8O

895.0	    539-30-0	Benzyl Ethyl Ether	C9H12O

896.0	    104-57-4	Benzyl Formate	C8H8O2

897.0	    100-46-9	Benzylamine	C7H9N

899.0	      75-62-7	Bromotrichloromethane	CBrCl3

900.0	      75-63-8	Bromotrifluoromethane	CBrF3

901.0	    111-34-2	Butyl Vinyl Ether	C6H12O

902.0	    109-73-9	Butylamine	C4H11N

903.0	    104-51-8	Butylbenzene	C10H14

- Table 3: Chemical List (cont.)

C#	CAS#	Description	Molecular Formula

----	----------	----------------------------------------------	--------------------------

904.0	    471-34-1	Calcium Carbonate	CCaO3

905.0	  1305-62-0	Calcium Hydroxide	CaH2O2

906.0	  7789-82-4	Calcium Molybdate	CaMoO4

907.0	      75-73-0	Carbon Tetrafluoride	CF4

908.0	    610-31-1	1,2,4-Trinitrobenzene	C6H3N3O6

909.0	  7688-21-3	cis-2-Hexene	C6H12

910.0	12192-29-9	Cobalt Carbide	Co2C

911.0	  7447-39-4	Cupric Chloride (Anhydrous)	Cl2Cu

912.0	    287-23-0	Cyclobutane	C4H8

913.0	    291-64-5	Cycloheptane	C7H14

914.0	    108-94-1	Cyclohexanone	C6H10O

915.0	    931-87-3	(z)-Cyclooctene	C8H14

916.0	    120-92-3	Cyclopentanone	C5H8O

917.0	    142-29-0	Cyclopentene	C5H8

918.0	    112-30-1	1-Decanol	C10H22O

919.0	      96-22-0	Diethyl Ketone	C5H10O

920.0	    142-96-1	Butyl Ether	C8H18O

921.0	    142-84-7	Dipropylamine	C6H15N

922.0	  7783-28-0	Ammonium Hydrogenphosphate	H9N2O4P

923.0	      53-70-3	Dibenz(a,h)anthracene	C22H14

924.0	    189-64-0	Dibenzo(a,h)pyrene	C24H14

925.0	    103-50-4	Benzyl Ether	C14H14O

926.0	  1067-33-0	Dibutyltin Diacetate	C12H24O4Sn

927.0	    352-9-32	Ethyl Sulfide	C4H10S

928.0	    623-81-4	Diethyl Sulfite	C4H10O3S

929.0	      75-10-5	Difluoromethane	CH2F2

930.0	      75-11-6	Diiodomethane	CH2I2

931.0	    628-55-7	Isobutyl Ether	C8H18O

932.0	51363-64-5	Diisodecyl Phenyl Phosphate	C26H47O4P

933.0	26761-40-0	Diisodecyl Phthalate	C28H46O4

934.0	    123-28-4	Didodecyl 3,3’-Thiodipropionate	C30H58O4S

935.0	    624-48-6	Dimethyl Maleate	C6H8O

936.0	    971-15-3	Dipentamethylenethiuram Hexasulfide	C12H20N2S8

937.0	    120-54-7	Dipentamethylenethiuram Tetrasulfide	C12H20N2S6

938.0	25265-71-8	Dipropylene Glycol	C6H14O3

939.0	    112-40-3	Dodecane	C12H26

940.0	    565-69-5	Ethyl Isopropyl Ketone	C6H12O

941.0	    540-67-0	Ethyl Methyl Ether	C3H8O

942.0	    105-37-3	Ethyl Propionate	C5H10O2

943.0	    628-32-0	Ethyl Propyl Ether	C5H12O

944.0	    103-69-5	N-Ethylaniline	C8H11N

945.0	    542-10-9	Ethylidene Diacetate	C6H10O4

947.0	    206-44-0	Fluoranthene	C16H10

948.0	    462-06-6	Fluorobenzene	C6H5F

949.0	    353-36-6	Fluoroethane	C2H5F

950.0	    593-53-3	Fluoromethane	CH3F

951.0	    103-70-8	Formanilide	C7H7NO

952.0	      56-81-5	Glycerine	C3H8O3

953.0	    111-70-6	Heptyl Alcohol	C7H16O

- Table 3: Chemical List (cont.)

C#	CAS#	Description	Molecular Formula

----	----------	----------------------------------------------	--------------------------

954.0	      87-82-1	Hexabromobenzene	C6Br6

955.0	    544-76-3	Hexadecane	C16H34

956.0	    392-56-3	Hexafluorobenzene	C6F6

957.0	    142-62-1	Hexanoic Acid	C6H12O2

958.0	    111-27-3	Hexyl Alcohol	C6H14O

959.0	    288-32-4	Imidazole	C3H4N2

960.0	  5888-33-5	Isobornyl Acrylate	C13H20O2

961.0	      78-81-9	Isobutylamine	C4H11N

962.0	    538-93-2	Isobutylbenzene	C10H14

963.0	      79-31-2	Isobutyric Acid	C4H8O2

964.0	29590-42-9	Isooctyl Acrylate	C11H20O2

965.0	    108-21-4	Isopropyl Acetate	C5H10O2

966.0	78448-33-6	Isopropyl Isobutyl Ether	C7H16O2

967.0	    112-53-8	1-Dodecanol	C12H26O

968.0	  7758-97-6	Lead Chromate	CrO4Pb

969.0	  7447-41-8	Lithium Chloride	ClLi

970.0	    546-93-0	Magnesium Carbonate	CMgO3

971.0	    298-00-0	Methyl Parathion	C8H10NO5PS

- 628-28-4	Butyl Methyl Ether	C5H12O

- 142-47-2	Monosodium Glutamate	C5H8NNaO4

974.0	    924-42-5	N-(Hydroxymethyl)acrylamide	C4H7NO

975.0	    109-74-0	Butyronitrile	C4H7N

976.0	    142-92-7	Hexyl Acetate	C8H16O2

977.0	    105-59-9	N-Methyldiethanolamine	C5H13NO2

978.0	    109-60-4	Propyl Acetate	C5H10O2

979.0	      88-12-0	N-Vinyl-2-pyrrolidone	C6H9NO

980.0	    554-84-7	3-Nitrophenol	C6H5NO3

981.0	    124-19-6	Nonanal	C9H18O

982.0	    108-82-7	2,6-Dimethyl-4-heptanol	C9H20O

983.0	    279-23-2	Norbornane	C7H12

984.0	    498-66-8	Norbornene	C7H10

985.0	    528-29-0	o-Dinitrobenzene	C6H4N2O4

986.0	    112-96-9	Octadecycl Isocyanate	C19H37NO

987.0	    124-13-0	Octanal	C8H16O

988.0	    124-07-2	Octanoic Acid	C8H16O2

989.0	    538-68-1	Pentylbenzene	C11H16

990.0	    122-60-1	Phenyl Glycidyl Ether	C9H10O2

991.0	    536-74-3	Phenylacetylene	C8H6

992.0	      88-99-3	Phthalic Acid	C8H6O4

993.0	  7447-40-7	Potassium Chloride	ClK

994.0	    111-43-3	Propyl Ether	C6H14O

995.0	    103-65-1	Propylbenzene	C9H12

996.0	    151-21-3	Sodium Docecyl Sulfate	C12H25NaO4S

997.0	  7631-99-4	Sodium Nitrate	NNaO3

998.0	  1313-60-6	Sodium Peroxide	Na2O2

999.0	  7772-99-8	Stannous Chloride	Cl2Sn

1000.0	      57-11-4	Stearic Acid	C18H36O2

1001.0	    121-57-3	Sulfanilic Acid	C6H7NO3S

1002.0	    540-88-5	tert-Butyl Acetate	C6H12O2

- Table 3: Chemical List (cont.)

C#	CAS#	Description	Molecular Formula

----	----------	----------------------------------------------	--------------------------

1003.0	762-75-4	tert-Butyl Formate	C5H10O2

1004.0	    631-36-7	Tetraethylsilane	C8H20Si

1005.0	  3333-52-6	Tetramethylsuccinonitrile	C8H12N2

1006.0	  7705-07-9	Titanium Trichloride	Cl3Ti

1007.0	      76-03-9	Trichloroacetic Acid	C2HCl3O2

1008.0	    102-71-6	Triethanolamine	C6H15NO3

1009.0	    143-22-6	Triethylene Glycol Monobutyl Ether	C10H22O4

1011.0	      76-05-1	Trifluoroacetic Acid	C2HF3O2

1012.0	    122-20-3	Triisopropanolamine	C9H21NO3

1013.0	      99-35-4	1,3,5-Trinitrobenzene	C6H3N3O6

1014.0	    115-86-6	Triphenyl Phosphate	C18H15O4P

1015.0	    102-69-2	Tripropylamine	C9H21N

1016.0	42978-66-5	Tripropylene Glycol Diacrylate	C15H24O6

1017.0	      57-13-6	Urea	CH4N2O

1018.0	    692-45-5	Vinyl Formate	C3H4O2

1019.0	    105-38-4	Vinyl Propionate	C5H8O2

1020.0	      78-78-4	Methylbutane, 2-	C5H12

1021.0	19132-06-0	2,3-Butanediol, (2S,3S)-(+)-	C4H10O2

1022.0	    104-76-7	2-Ethylhexanol	C8H18O

1023.0	    100-74-3	4-Ethylmorpholine	C6H13NO

1024.0	    109-02-4	4-Methylmorpholine	C5H11NO

1025.0	    108-83-8	Diisobutyl ketone	C9H18O

1026.0	    111-14-8	Heptanoic acid	CH3(CH2)5CO2H

1027.0	    110-12-3	Methyl isoamyl ketone	C7H14O

1028.0	    107-87-9	Methyl n-propyl ketone	C5H10O

1029.0	    110-91-8	Morpholine	C4H9O

1030.0	   7727-37-9	Nitrogen	N2

1031.0	 10024-97-2	Nitrous oxide	N2O

1032.0	  7782-44-7	Oxygen	O2

1033.0	    109-52-4	Pentanoic acid	C5H10O2

1034.0	    107-98-2	Propylene glycol monomethyl ether	C4H10O2

1035.0	   108-65-6	Propylene glycol monomethyl ether acetate	C6H12O3

1036.0	     99-94-5	p-Toluic acid	C8H8O2

1037.0	21282-97-3	2-(acetoacetoxy)ethyl methacrylate	C10H14O5

1038.0	42946-19-0	2,6-dicarboxyanthraquinone	C16H8O6

1039.0	60505-55-7	2,6-dicarboxyfluorenone	C15H8O5

1040.0	2051-24-3	decachlorobiphenyl	C12Cl10

1041.0	1115-20-4	hydroxypivalyl hydroxypivalate	C10H20O4

1042.0	97-85-8	Isobutyl isobutyrate	C8H16O2

1043.0	1075-38-3	m-Butyl Toluene	C11H16

1044.0	1344-48-5	Mercury Sulfide	HgS

1045.0	99-75-2	methyl p-toluate	C9H10O2

1046.0	11113-74-9	Nickel Hydroxide	NiO2H2

1047.0	39001-02-0	octachlorodibenzofuran	C12Cl8O

1048.0	29082-74-4	octachlorostyrene	C8Cl8

1049.0	608-90-2	Pentabromobenzene	C6HBr5

1050.0	608-93-5	pentachlorobenzene	C6HCl5

1051.0	335-67-1	perfluorooctanoic acid	C8HF15O2

1052.0	77-68-9	Texanol	C12H24O3

- Table 3: Chemical List (cont.)

C#	CAS#	Description	Molecular Formula

----	----------	----------------------------------------------	--------------------------

1053.0	629-20-9	1,3,5,7-cyclooctatetraene	C8H18

1054.0	1191-99-7	2,3-dihydrofuran	C4H6O

1055.0	149-57-5	2-ethylhexanoic acid	C8H16O2

1056.0	3825-26-1	Ammonium pentadecafluorooctanote	C8F15O2H4N

1057.0	12135-76-1	Ammonium sulfide	H8N2S

1058.0	10294-33-4	Boron tribromide	BBr3

1059.0	1317-39-1	Copper (I) oxide	Cu2O

1060.0	1317-38-0	Copper (II) oxide	CuO

1061.0	7758-98-7	Copper sulfate	CuO4S

1062.0	120-61-6	Dimethyl terephthalate	C10H10O4

1063.0	37143-54-7	Methoxyisopropylamine	C4H11NO

1064.0	115-25-3	Octafluorocyclobutane	C4F8

1065.0	78-40-0	Triethylphosphate	C6H15O4P

1066.0	512-56-1	Trimethylphosphate	C3H9O4P

1067.0	      121-45-9	Trimethylphosphite	C3H9O3P

1068.0	    99-96-7	4-Hydroxybenzoic Acid	C7H6O3

1069.0	491-35-0	4-Methylquinoline	C10H9N

1070.0	55079-83-9	Acitretin	C21H26O3

1071.0	7647-17-8	Cesium Chloride	CsCl

1072.0	929-06-6	Diglycolamine	C4H11NO2

1073.0	108-18-9	Diisopropyl Amine	C6H15N

1074.0	593-74-8	Dimethyl Mercury	C2H6Hg

1075.0	999-97-3	Hexamethyldisilazane	C6H19NSi2

1076.0	66-25-1	Hexanal	C6H12O

1077.0	133-32-4	Indole-3-Butyric Acid	C12H13NO2

1078.0	108-11-2	Methyl Isobutyl Carbinol	C6H14O

1079.0	3268-87-9	Octachlorodibenxo-p-Dioxin	C12Cl8O2

1080.0	2234-13-1	Octachloronapthalene	C10Cl8

1081.0	40487-42-1	Pendimethalin	C13H19N3O4

1082.0	110-62-3	Pentanal	C5H10O

1083.0	57465-28-8	3,3',4,4',5-Pentachlorobiphenyl	C12H5Cl5

1084.0	103426-96-6	1,2,3,4,6,7-Hexachloronapthalene (PCN-66)

1085.0	108-45-2	m-phenylenediamine (1,3-phenylenediamine)	C6H8N2

1086.0	8007-18-9	C.I. Pigment Yellow 53

1087.0	77-58-7	Dibutyl tin dilaurate	C32H64O4Sn

1088.0	1672-46-4	Digoxigenin	C23H34O5

1089.0	64742-95-6	Light aromatic naphtha

1090.0	32588-76-4	N,N'-ethylenebis(tetrabromophthalimide)	C18H4Br8N2O4

1091.0	497-25-6	Oxazolidone	C3H5NO2

1092.0	2451-62-9	Tris(2,3-epoxypropyl) isocyanurate	C12H15N3O6

1093.0	539-82-2	Ethyl-valerate	C7H14O2

1094.0	132259-10-0	Air

1095.0	105-08-8	1,4-Cyclohexanedimethanol	C8H16O2

1096.0	13556-58-6	1-Ethyltetralin      	C12H16

1097.0	1559-81-5	1-Methyltetralin	C11H14

1098.0	1679-09-0	2-Methyl-2-butanethiol	C5H12S

1099.0	6846-50-0	2,2,4-Trimethyl-1,3-Pentanediol Diisobutyrate	C16H30O4

1100.0	135989-69-4	2,4,5-Tricarboxybenzophenone	C16H10O7

1101.0		2,4,5-Tricarboxybiphenyl	C15H10O6

- Table 3: Chemical List (cont.)

C#	CAS#	Description	Molecular Formula

----	----------	----------------------------------------------	--------------------------

1102.0	964-68-1	4,4-Dicarboxybenzophenone	C15H10O5

1103.0	787-70-2	4,4Dicarboxybiphenyl	C14H10O4

1104.0	6422-86-2	Di(2-Ethylhexyl)terephthalate	C24H38O4

1105.0	627-44-1	Diethyl mercury    	C4H10Hg

1106.0	539-88-8	Ethyl levulinate	C7H12O3

1107.0	115-09-3	Methyl mercury chloride	CH3ClHg

1108.0	126-30-7	Neopentyl glycol	C5H12O2

1109.0	107-03-9	n-Propyl mercaptan  	C3H8S

1110.0	13188-60-8	Nylon 612 salt

1111.0	1885-14-9	Phenyl chloro formate	C7H5ClO2

1112.0	110-02-1	Thiophene  	C4H4S

1113.0	217-59-4	Triphenylene	C18H12

1114.0	7440-66-6	Zinc	Zn

- Table 4: Principal Literature Sources

- BLOCK 1 - Oxygen Demand

- Cooper,W.J., Chemistry in Water Reuse, Ann Arbor Science, Ann Arbor, MI, 1981

Howard, P.H., R.S. Boethling, W.F. Jarvis, W.M. Meylan, and E.M. Michalenko, Handbook of Environmental Degradation Rates, Lewis Publishers, Chelsea, MI 1991

Janicke, W., Chemishe Oxidierbarkeit organischer Wasserinhaltsstoffe, Berichte Institut für Wasser-, Boden-und 	Lufthygiene des Bundesgesundheitsamtes, Detrich Reimer Verlag, Berlin, 1983

Jenkins, D., V.L. Snoeyink, J.F. Ferguson, and J.O. Leckie, Water Chemistry Laboratory Manual 3rd Edition, Wiley, New York, 1980

Lund, H.F., Industrial Pollution Control Handbook, McGraw-Hill, New York, 1971

Metcalf & Eddy, Inc., Wastewater Engineering, 2nd Edition, McGraw-Hill, New York, 1979

Pitter, P. and J. Chudoba, Biodegradability of Organic Substances in the Aquateic Environment, CRC, Boca, 	Raton, FL, 1990

Verschueren, K., Handbook of Environmental Data on Organic Compounds, 3rd Edition, Van Nostrand

Reinhold, New York, 1996

JOURNAL		ABBREVIATION

Acta Hydrochimica Hydrobiologica	Acta Hydrochim. Hydrobiol.

American Dyestuff Reporter	Amer. Dyestuff Reporter

American Public Health Association (APHA)	Am. Public Health Assoc. Yearb.

Applied Microbiology		Appl. Microbiol.

Annual Book of ASTM Standards	Ann. Book ASTM Stand.

Chemosphere		(same)

Industrial and Engineering Chemistry	Ind. Eng. Chem.

Industrial Pollution Control Handbook	(same)

Journal of Industrial Microbiology	J. Ind. Microbiol.

Journal of the Water Pollution Control Federation	J. Water Poll. Cont. Fed.

Plant Engineering Handbook

Proceedings of the Purdue Industrial Waste Conference

Proceedings of the American Association of Textile 	Proc. Amer. Assoc. Text. Chem &  Chemists and Colorests	Colorests

Sewage and Industrial Wastes	Sew. Ind. Wastes

Vodosnabzhenie i Sanitarnaya Tekhnika	Vodosnabzh. Sanit. Tekh.

Vom Wasser

Water Research	Water Res.

- Table 4: Principal Literature Sources (cont.)

- BLOCK 2 - Soil/Water Partitioning

Abdul, A.S., T.L. Gibson, and D.N. Rai, “Statistical Correlations for Predicting the Partition Coefficient for 	Nonpolar Organic Contaminants Between Aquifer Organic Carbon and Water.”  Hazard. Waste Hazard. 	Mater. 4, 211, 1987

Abdul, A.S., T.L. Gibson, and D.N. Rai, “The Effect of Organic Carbon on the Adsorption of Fluorene by Aquifer 	Materials.”  Waste Hazard. Mater. 3,429, 1986

Baker, J.R.; J.R. Mihelcic; D.C. Luehrs; J.P. Hickey; “Evaluation of Estimation Method for Organic Carbon 	Normalized Sorption Coefficients,”  Water Environment Research 69, 136, 1997

Barber,L.B., Thurman,E.M., and Runnells,D.D. “Geochemical Heterogeneity in a Sand and Gravel Aquifer: Effect 	of Sediment Minerology and Particle Size on the Sorption of Chlorobenzenes.”  J. Contam. Hydrol. 9,35, 	1992

Brown, D.S., and E.W. Flagg, “Empirical Prediction of  Pollutant Sorption in Natural Sediments. J. Environ. Qual. 	10, 382, 1981

Hansch, C., A. Leo, and D. Hoekman,  Exploring QSAR, American Chemical Society, Washington, DC, 1995

Hassett, J.J., J.C. Means, W.L. Banwart, and S.G. Wood,  “Sorption Properties of Sediments and Energy Related 	Pollutants,” Environmental Protection Agency, Springfield, VA, 1980

Karickhoff, S.W., D.S. Brown, and T.A. Scott, “Sorption of Hydrophobic Pollutants on Natural Sediments.” Water 	Res. 13,241, 1979

Lyman, W.J., W.F. Reehl, and D.H. Rosenblatt, Handbook of Chemical Property Estimation Methods: Environmental Behavior of Organic Compounds, McGraw-Hill, New York, 1982

Mackay, D., W.Y. Shiu, and K.C. Ma, Illustrated Handbook of Physical-Chemical Properties and Environmental Fate for Organic Chemicals, Vol. I and II, Lewis, Baca Raton, FL, 1992

Means, J.C., S.G. Wood, J.J. Hassett, and W.L. Banwart, “Sorption of Polynuclear Aromatic Hydrocarbons by 	Sediments and Soils.” Environ. Sci. Technol., 14, 1524, 1980

Montgomery, J.H. Groundwater Chemicals Desk Reference, Vol. 2. Lewis Publishers, Chelsea, MI, 1991

Montgomery, J.H. Agrochemicals Desk Reference,  Lewis Publishers, Chelsea, MI, 1993

Nelson, D.W. and L.E. Summers, Total Carbon, Organic Carbon, and Organic Matter, American Society 	Agronomy, Madison, WI, 1982

Oliver, B.G.  “Partitioning Relationships for Chlorinated Organics Between Water and Particulates in the St. Clair, 	Detroit, and Niagara rivers.”  in QSAR in Environmental Toxicology-II (Ed., Kaiser,K.L.E.) D. Reidel 	Publishing,  Norwell, MA,1987

Paya-Perez, B., M. Raiz, and B.R. Larsen, “Soil Sorption of 20 PCB Congeners and Six Chlorobenzenes.” 	Ecotoxicol. Environ. Safety, 21, 1, 1991

Rippen, G., M. Ilgestein,  W. Klopffer, “Screening of the Adsorption Behavior of New Chemicals: Natural Soils 	and Model Adsorbents.” Ecotoxicol. Environ. Safety, 6, 236, 1982

Russell, D.J., and B. McDuffie, “Chemodynamic Properties of Phthalate Esters: Partitioning and Soil Migration.” 	Chemosphere, 15, 1003, 1986

Schellenberg, K., C. Leuenberger,  and R.P. Schwarzenbach,. “Sorption of Chlorinated Phenols by Natural 	Sediments and Aquifer Materials.” Environ. Sci. Technol. 18, 652, 1984

Stuart, B.J., G.F. Bowlen, and D.S. Kosson, “Competitive Sorption of Benzene, Toluene, and the Xylenes onto 	Soil.” Env. Progress, 10, 109, 1991

USEPA  Superfund Public Health Evaluation Manual, EPA/540/1-86 1-175, United States Environmental 	Protection Agency, 1986

Verschueren, K., Handbook of Environmental Data on Organic Compounds, 3rd Edition, Van Nostrand Reinhold, New York, 1996

von Oepen, B., W. Kordel, W. Klein, and G. Schuurmann,  “Sorption of Nonpolar and Polar Compounds to Soils: 	Process, Measurements and Experience with the Applicability of the Modified OECD-Guideline 106.” 	Sci. Total Environ. 109-110, 343, 1991.

- Table 4: Principal Literature Sources (cont.)

- BLOCK 2 - Soil/Water Partitioning (cont.)

Walters, R.W., S.A. Ostazeski, A. Guiseppi-Elie, “Sorption of 2,3,7,8-Trichlorodibenzo-p-dioxin from Water by 	Surface Soils.” Environ. Sci. Technol. 23, 480, 1989

Ware, G.W., Reviews of Environmental Contamination and Toxicology, Springer, New York, 1992

JOURNAL		ABBREVIATION

American Society of Agronomy	Am. Soc. Agron., Sp. Publ.

Bulletin of the Chemical Society of Japan	Bull. Chem. Soc. Jpn.

Ecotoxicology and Environmental Safety	Ecotoxicol. Environ. Saf.

Environmental Health Perspectives	Environ. Health Perspect.

Environmental Science and Technology	Environ. Sci. Technol.

Hazardous Waste and Hazardous Materials	Hazard. Waste Hazard. Mater.

Illinois Institute of Natural Resources, St. Geological Survey	Bull.-Ill. State Water Surv.

Journal of Agricultural and Food Chemistry	J. Agric. Food Chem.

Journal of Chemical and Engineering Data	J. Chem. Eng. Data

Journal of Environmental Quality	J. Environ. Qual.

Journal of Hazardous Materials	J. Hazard. Mater.

Reviews of Environmental Contamination and Toxicology	Rev. Environ. Contam. Toxicol.

Soil Science	Soil Sci.

Water Research	Water Res.

- Table 4: Principal Literature Sources (cont.)

- BLOCK 3 - General Physical Properties

- Aldrich Chemical Co., Catalog/Handbook of Fine Chemicals, Aldrich Chemical Co., Milwaukee, WI, 1992-2000

- Barin, I., Thermochemical Data of Pure Substances, Parts I and II, 2nd Edition, VCH, Weinheim, 1993

Beaton, C.F. and G.F. Hewiltt, Physical Property Data for the Design Engineer, Hemisphere, New York, 1989

Beilstein's Handbuch der Organischen Chemie, 4th Edition, Springer, New York (1918-)

- Boublik, T., V. Fried, and E. Hala, The Vapour Pressures of Pure Substances, Elsevier, New York, 1973

Braker, W., A.L. Mossman, Matheson Gas Data Book, 5th edition, Matheson Gas Products, E. Rutherford, NJ, 1971

Cox, J.D., and G. Pilcher, Thermochemistry of Organic and Organometallic Compounds, Academic Press, New York, 1970

- Danner, R.P., and T.E. Daubert, Manual for Predicting Chemical Process Design Data, AIChE, New York, 1987

Dean, J.A., Lange's Handbook of Chemistry, 14th Edition, McGraw-Hill, New York, 1992

Flick, E.W., Industrial Solvents Handbook, 3rd Edition, Noyes Data, Park Ridge, NJ, 1985

Freier, R.K., Aqueous Solutions Data for Inorganic and Organic Compounds, de Gruyler, New York, 1975

Gammon, B.E., K.N. Marsh, and A.K.R. Dewan, Transport Properties and Related Thermodynamic Data of 	Binary Mixtures, Parts 1, 2, AIChE, New York, 1994

Gerhartz, W., Ullmann's Encyclopedia of Industrial Chemistry, 5th Edition, VCH, Weinheim, 1985

Gmelin's Handbook of Inorganic Chemistry, 8th Edition, Springer, New York (1924-)

Grayson, M., and D. Eckroth, Kirk-Othmer Encyclopedia of Chemical Technology, 3rd Edition, Vol. 1-24, Wiley, New York, 1978

Hartley, D., and H. Kidd, The Agrochemicals Handbook, 2nd Edition, Royal Society of Chemistry, Nottingham, 1987

Hellwege, K., and O. Madelung, Landolt-Börnstein, Numerical Data and Functional Relationships in Science and 	Technology, 6th Edition and New Series, Springer, New York (1950-)

Ho, C.Y., Properties of Selected Ferrous Alloying Elements, Hemisphere, New York, 1989

Jamieson, D.T., J.B. Irving, and J.S. Tudhope, Liquid Thermal Conductivity, Her Majesty's Stationery Office, 	Edinburgh, 1975

Johnson Matthey, Alfa Catalog, Johnson Matthey, Ward Hill, MA, 1990

Keith, L.H. and M.M. Walker, Handbook of Air Toxics, Lewis, Baca Raton, FL, 1995

Knacke, O., O. Kubaschewski, and K. Hesselmann, Thermochemical Properties of Inorganic Substances, 2nd 	Edition, Springer, New York, 1991

Lagowski, J.J., The Chemistry of Nonaqueous Solvents, Vol. I-VB, Academic Press, New York, 1967-1978

Lide, D.R., Handbook of Chemistry and Physics, 71st Edition, CRC Press, Boca Raton, FL, 1990

Mackay, D., W.Y. Shiu, and K.C. Ma, Illustrated Handbook of Physical-Chemical Properties and Environmental Fate, Vol I-III, Lewis, Boca Raton, FL, 1992

- McKetta, J.J., Encyclopedia of Chemical Processing and Design, Vol. 1-68, Dekker, New York, 1976-1999

Montgomery, J.H., Agrochemicals Desk Reference, Lewis, Baca Raton, FL, 1993

Pankratz, L.B., Thermodynamic Properties of Halides, Bureau of Mines Bulletin, U.S. Government Printing

Office, Washington, DC, 1984

Pedley, J.B., R.D. Naylor, and S.P. Kirby, Thermochemical Data of Organic Compounds, 2nd Edition, Chapman 	and Hall, New York, 1986

Perry, R.H., and D. Green, Perry's Chemical Engineers' Handbook, 7th Edition, McGraw-Hill, New York, 1997

Pfaltz & Bauer, Catalog, 12th Edition, Pfaltz & Bauer, Waterbury, CT, 1991

Riddick, J.A., Organic Solvents, 4th Edition, Wiley, New York, 1986

Rivkin, S.L., Thermodynamic Properties of Gases, 4th Edition, Hemisphere, New York, 1988

Schilt, A.A., Perchloric Acid and Perchlorates, G.F. Smith Chemical Co., Columbus, OH, 1979

Stull, D.R., E.F. Westrum, Jr., and G.C. Sinke, The Chemical Thermodynamics of Organic Compounds, Wiley, 	New York, 1970

Timmermans, J., Physico-Chemical Constants of Pure Organic Compounds, Elsevier, Amsterdam, 1965

Worthing, C.R., Pesticide Manual, 6th Edition, British Crop Protection Council, Croydon, 1979

Yaws, C.L., Handbook of Viscosity, Gulf Publishing, Houston, 1995

- Table 4: Principal Literature Sources (cont.)

- BLOCK 3 - General Physical Properties (cont.)

- Yaws, C.L., Handbook of Thermal Conductivity, Vol. 1-3, Gulf Publishing, Houston, 1995

Yaws, C.L., Thermodynamic & Physical Property Data, Gulf Publishing, Houston, 1992

JOURNALS		ABBREVIATION

AIChE Journal		AIChE J.

Analytica Chimica Acta		Anal. Chim. Acta

Analytical Chemistry		Anal. Chem.

Angewante Chemie		Angew. Chem.

Canadian Journal of Chemistry	Can. J. Chem.

Chemical Abstracts		Chem. Abstr.

Chemical Engineering		Chem. Eng.

Chemical Reviews		Chem. Rev.

Chemosphere		(same)

Environmental and Toxicological Chemistry	Environ. Toxicol. Chem.

Environmental Science and Technology	Environ. Sci. Technol.

Fluid Phase Equilibria		Fluid Phase Equilib.

Environmental Contamination and Toxicology	Environ. Contam. Toxicol.

Industrial and Engineering Chemistry Research	Ind. Eng. Chem. Res.

J. Chemical Society of Japan (Nippon Kagaku Kaishi)	J. Chem. Soc. Jpn.

J. of Applied Polymer Science	J. Appl. Polym. Sci.

J. of Applied Toxicology		J. Appl. Toxicol.

J. of Chemical and Engineering Data 	J. Chem. Eng. Data

J. of Chemical Engineering of Japan	J. Chem. Eng. Jpn.

J. of Chemical Information and Computer Science	J. Chem. Inf. Comput. Sci.

J. of Chemical Physics		J. Chem. Phys.

J. of Chemical Thermodynamics	J. Chem. Thermodyn.

J. of Environmental Engineering	J. Environ. Eng.

J. of Hazardous Materials		J. Hazard. Mater.

J. of Pharmaceutical Science	J. Pharm. Sci.

J. of Physical Chemistry		J. Phys. Chem.

J. of Research of the National Bureau of Standards	J. Res. Nat. Bur. Stand.

J. of Solution Chemistry		J. Solution Chem.

J. of the American Chemical Society	J. Am. Chem. Soc.

J. of the Chemical Society (London)	J. Chem. Soc.

J. Physical and Chemical Reference Data	J. Phys. Chem. Ref. Data

Makromolecular Chemie		Makromol. Chem.

Materials Chemistry and Physics	Mater. Chem. Phys.

Mobay Technical Data Sheet for 2,4-Toluene Diisocyanate

SAR and QSAR in Environmental Research	SAR QSAR Environ. Res.

Science	Sci.

Thermochimica Acta 	Thermochim. Acta

Water Research	Water Res.

- Table 4: Principal Literature Sources (cont.)

- BLOCK 4 - Partitioning/Equilibrium

- Gmehling, J., et al., Activity Coefficients at Infinite Dilution, Parts 1-4, DECHEMA, Frankfort, 1986-1994

Hansch, C. and A. Leo, Substituent Constants for Correlation Analysis in Chemistry and Biology, Wiley, New York, 1979

Howe, G.B., M.E. Mullins, and T.N. Rogers, Evaluation and Prediction of Henry's Law Constants and Aqueous Solubilities for Solvents and Hydrocarbon Fuel Components, Vol. I-III, Research Triangle Institute, Research Triangle Park, NC, 1987

Yaws, C.L., Thermodynamics and Physical Property Data-Predicting Henry's Law Constant, Gulf Publishing, 	Houston, 1992

JOURNAL		ABBREVIATION

AIChE Journal		AIChE J.

Analytical Chemistry		Anal. Chem.

Bulletin of the Chemical Society of Japan	Bull. Chem. Soc. Jpn.

Chemical Engineering		Chem. Eng.

Chemical Engineering Progress	Chem. Eng. Prog.

Chemosphere	(same)

Environmental Science and Technology	Environ. Sci. Technol.

Environmental Toxicology and Chemistry	Environ. Toxicol. Chem.

European Journal of Medicinal Chemistry	Eur. J. Med. Chem.

J. American Water Works Association	J. Am. Water Works Assoc.

J. Chemical Engineering Data	J. Chem. Eng. Data

J. Chemical Engineering of Japan	J. Chem. Eng. Jpn.

J. Chemical Society Japan (Nippon Kagaku Kaishi)	J. Chem. Soc. Jpn.

J. Physical Chemical Reference Data	J. Phys. Chem. Ref. Data

J. Medicinal Chemistry		J. Med. Chem.

J. of Hazardous Materials		J. Hazard. Mater.

J. Solution Chemistry	J. Solution Chem.

- Table 4: Principal Literature Sources (cont.)

- BLOCK 5-Fire & Explosion

- Aldrich Chemical Co., Catalog/Handbook of Fine Chemicals, Aldrich Chemical Co., Milwaukee, WI, 1992-2000

Baratov, A.N., A.Ya. Korol'chenko, G.I. Kravchukh, et al., The Handbook for Flammability and Explosability of 	Chemicals and Substances, and Means of Extinguishing, Khimia, Moscow, 1990

Bond, J., Sources of Ignition-Flammability Characteristics of Chemicals and Products, Butterworth-Heinemann, Boston (1991)

Braker, W., A.L. Mossman, Matheson Gas Data Book, 5th edition, Matheson Gas Products, E. Rutherford, NJ, 	1971

Bretherick, L., Bretherick's Handbook of Reactive Chemical Hazards, 4th Edition, Butterworths, New York, 1990

Chatrathi, K., Report on Flammability Testing for Hoechst Celanese, 1991

Coward, H.F., and G.W. Jones, Limits of Flammability of Gases and Vapors, U.S. Bureau of Mines Bulletin 503, U.S. Government Printing Office, Washington, DC, 1952

Fire Protection Guide on Hazardous Materials,  National Fire Protection Association, Quincy, MA, 1984

Hazard Evaluation and Risk Control Services, Hercules Inc., Data Guide, 1991

Lide, D.R., Handbook of Chemistry and Physics, 71st Edition, CRC, Boca Raton, FL, 1990

Kuchta, J.M., Investigation of Fire and Explosion Accidents in the Chemical, Mining, and Fuel-Related Industries-A Manual, U.S. Bureau of Mines Bulletin 680, U.S. Government Printing Office, Washington, DC, 1985

Manufacturing Chemists' Association, Inc., Chemical Safety Data Sheets, Washington, DC, 1985-

Nabert, K., and G. Schön, Sicherheitstechnische Kennzahlen brennbarer Gas und Dämpfe, Deutscher Eichuerlag 	Gmbtl., Berlin, 1963

Perry, J.H., and D. Green, Perry's Chemical Engineers' Handbook, 6th Edition, McGraw-Hill, New York, 1984

Sax, N.I. Dangerous Properties of Industrial Materials, 7th Edition, Van Nostrand Reinhold Co., New York, 1989

Stephenson, R.M., Flash Points of Organic and Organometallic Compounds, Elsevier, Amsterdam, 1987

Shell Chemical Co., Methyl Ethyl Ketone

Shell Chemical Co., Methyl Isobutyl Ketone

Stull, D.R., Fundamentals of Fire and Explosion, AIChE Monograph Series, No. 73, AIChE, New York, 1977

Zabetakis, M.G., Flammability Characteristics of Combustible Gases and Vapors, U.S. Bureau of Mines Bulletin 627, U.S. Government Printing Office, Washington, DC, 1965

Zabetakis, M.G., J.C. Cooper, and A.L. Furno, Flammability in Air of Solvent Mixures Containing Methyl Ethyl Ketone and Tetrahydrofuran, U.S. Bureau of Mines Report of Investigations 6048, U.S. Government Printing Office, Washington, DC, 1962

Zabetakis, M.G., G.S. Scott, and G.W. Jones, The Flammability Characteristics of the CnH2n-6 Aromatic Series, U.S. Bureau of Mines Report of Investigations 4824, U.S. Government Printing Office, Washington, DC, 1951

JOURNAL		ABBREVIATION

Chemical Engineering		Chem. Eng.

Chemical Reviews		Chem. Rev.

Chemicals and Products	Chem. Prod.

Fire and Materials		Fire Mater.

Industrial and Engineering Chemistry (Research)	Ind. Eng. Chem. Res.

J. Chemical Engineering Data	J. Chem. Eng. Data

J. of Hazardous Materials	J. Hazard. Mater.

J. of Research of the National Bureau of Standards	J. Res. Nat. Bur. Stand.

Soviet Chemical Industry	Sov. Chem. Ind.

- Table 4: Principal Literature Sources (cont.)

- BLOCK 6 - Aquatic Toxicity

JOURNAL	ABBREVIATION

Ambio	(same)

Aquatic Toxicology	Aquat. Toxicol.

Archives of Environmental Contamination 	Arch. Environ. Contam. Toxicol.

and Toxicology.

Archives of Environmental Health	Arch. Environ. Health

Bulletin of Environmental Contamination and Toxicology	Bull. Environ. Contam. Toxicol.

Canadian Journal of Fisheries and Aquatic Science	Can. J. Fish. Aquat. Sci.

Comparative Biochemistry and Physiology, C	Comp. Biochem. Physiol. C

Current Contents (A)	(same)

Ecotoxicology and Environmental Safety	Ecotoxicol. Environ. Saf.

Environment and Technology	Environ. Technol.

Environment and Ecology		Environ. Ecol.

Environmental Health Perspectives	Environ. Health Perspect.

Environmental Pollution, Series A	Environ. Pollut., Ser. A

Environmental Science and Technology	Environ. Sci. Technol.

Environmental Toxicology and Chemistry	Environ. Toxicol. Chem.

Freshwater Biology	Freshwater Biol.

Health	(same)

Hydrobiologia	(same)

Journal of Agricultural and Food Chemistry	J. Agric. Food Chem.

Journal of Environmental Quality	J. Environ. Qual.

Journal of Experimental Biology	J. Exp. Biol.

Journal of Experimental Marine Biology and Ecology	J. Exp. Mar. Biol. Ecol.

Journal of Fish Biology	J. Fish Biol.

Journal of Marine Research	J. Mar. Res.

Journal of the Fisheries Research Board of Canada	J. Fish. Res. Board Can.

(changed to Can. J. Fish. Aquat. Sci.)

Journal of the Water Pollution Control Federation	J. Water Pollut. Control Fed.

Journal of Toxicology and Environmental Health	J. Toxicol. Environ. Health

Marine Biology	Mar. Biol.

Marine Ecology Progress Series	Mar. Ecol. Progr. Ser.

Marine Fisheries Review	Mar. Fish. Rev.

NAPCA Abstract Bulletin		(same)

Oikologia	(same)

Oikos	(same)

Progressive Fish Culturist		Prog. Fish Cult.

Reviews in Fish Biology and Fisheries	Rev. Fish Biol. and Fish.

Science	Science

Science and the Total Environment	Sci. Total Environ.

Sewage Works Journal (changed to J. Water Pollut. Control Fed.)	Sewage Works J.

Sewage and Industrial Wastes (changed to J. Water Pollut. Control Fed.)	Sewage Ind. Wastes

Transactions of the American Fisheries Society	Trans. Am. Fish. Soc.

Water Environment Research	Water Env. Res.

Water Research	Water Res.

Water and Sewage Works	Water Sewage Works

Water, Air, and Soil Pollution	Water Air Soil Poll.

- Table 5:  BOD Rating Criteria

- PAPER INFORMATION

- Title_________________________________________________________________________________________

____________________________________________________________________________________________

- Author(s)_____________________________________________________________________________________

____________________________________________________________________________________________

- Comments____________________________________________________________________________________

____________________________________________________________________________________________

____________________________________________________________________________________________

=================================================================================

- Rating Criteria

____________________________________________________________________________________________

- Followed Standard Methods (temperature not included)

Yes			(2)

No or cannot tell from information provided 	(0)

____________________________________________________________________________________________

- Temperature Reported?

Yes			(2)

No			(0)

____________________________________________________________________________________________

- Acclimation

Acclimated seed was used	(3)

Non-Acclimated seed was used	(1)

Acclimation not reported		(0)

____________________________________________________________________________________________

- Concentration Reported?

Yes			(1)

No			(0)

____________________________________________________________________________________________

- Internal Consistency

Compared results to ThoD	(2)

No comparison to ThoD		(1)

____________________________________________________________________________________________

- TOTAL

Table 6:  Evaluation Criteria for Soil/Water Partition Coefficient

- Score

| Criteria | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Experimental Conditions | Data not from primary source | Paper does not report that the sample was sterilized/measured for byproducts. | Paper reports that the samples was sterilized/measured for byproducts. |
| Purity of Chemicals | Data not from primary source | All others | Supplier reported |
| Experimental Techniques | 1 soil tested | 2 - 4 soils tested | ³ 4 soils tested |
| Reported Accuracy/Internal Precision* | No consideration given to relevant conditions | Some relevant conditions monitored and reported | All relevant conditions monitored and reported |
| Agreement with other work | Low scores for previous evaluation criteria and no discussion of comparison to other work and/or poor comparability | High scores for previous evaluation criteria and no discussion of comparison to other work and good comparability of data | High scores for previous evaluation criteria and discusses work done by others with good comparability of data |

* Important considerations include: equilibration, removal of solids from dilution water, soil pH, temperature and general quality of work and journal.

NOTE: The relevance of these conditions varies for different chemicals and different types of tests.  Designations are determined by the reviewer and explained in the comment section specific to the value.

Table 7:  Some Reliable References and Criteria for Soil/Water Partitioning

| Reference | Equil time | Sterilization | Water source | # of soils | pH | OC% | Method |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ASTM, 1991 | ³ 48 hours1 | Mass balance2 | Source or Reagent Grade3 | ³4 | 6 | - | BP1 |
| Barber, 1992 | 48 hours | None | NS | 4 | NS | 0.8-5.7 | BP2 |
| Lee, 1991 | NA CS | MW radiation | NS | 1 | NS | 0.39 | MD |
| Ball, 1991 | 9-100 days | Autoclaved | NS | 8 | NS | 0.015-0.099 | BP2 |
| Boyd, 1982 | 24 hours4 | Anaerobic | NS | 1 | NS | 5.10(OM) | BP3 |
| Schellenberg, 1984 | 15-20 hours4 | None | NS | 3 | NS | 0.84-9.4 | BP2 |
| Brown, 1981 | 24-72 hours4 | None | NS | 1 | NS | 3.27 | BP1 |
| Chin, 1988 | 120 hours4 | None | NS5 | 2 | NS | 0.13-1.42 | BP4 |
| Russell, 1986 | 25 hours | sodium azide | NS | 2 | NS | 0.26-1.59 | BP2 |
| Walters, 1989 | 2 days | sodium azide | NS5 | 1 | 7 | 0.66 | BP3 |
| Hassett, 1980 | 20-24 hours | None6 | NS | 14 | NS | 0.11-2.38 | BP2 |
| Karickhoff, 1979 | 24-48 hours4 | None | NS5 | 3 | NS | 0.086-3.29 | BP2 |
| Abdul, 1987 | 24 hours4 | None | Milli-Q | 3 | NS | 0.04-2.2 | BP2 |
| Rippen, 1982 | 24 hours4 | None | NS | 3 | NS | 0.69-2.24 | BP2 |
| Garbini, 1985 | ~24 hours | None | NS | 2 | NS | 0.23-0.41 | BP5 |

Abbreviations: NS=Not stated in article, BP1=Bottle point with varying sorbent amount, BP2=Bottle point with more than one sorbent of varying organic carbon concentration, BP3=Bottle point with varying chemical concentration in solution, BP4=Bottle point with one soil and one point, BP5=Bottle point with different sorbents of varying sorbent organic carbon concentration where the gas phase was sampled, NA=Not applicable, CS=Column study, GP=Gas purge, MD=Miscible displacement, MW=Microwave

Footnotes: 1=Preliminary tests should be done to determine appropriate equilibrium time, 2=Used to determine if degradation has occurred, 3=Source water should be treated to remove particulate matter, 4=Equilibrium was confirmed with kinetic studies or by sequential sampling, 5=Particulate matter removed by washing, 6=Byproducts of degradation were looked for in the form of unidentified peaks in the gas chromatograph recording.

- References:

Abdul, A.S.; T.L. Gibson, and D.N. Tai, Hazard. Waste Hazard. Mater. 4, 211 (1987)

ASTM, Annual Book of ASTM Standards, Vol. 11.04, Pesticides; Resource Recovery; Hazardous Substances and  Oil Spill Responses; Waste Disposal; Biological Effects, ASTM, Philadelphia, 1991

Ball, W.P. and P.V. Roberts, Environ. Sci. Technol. 25, 1223 (1991)

Barber, L.B., II, E.M. Thurman, and D.D. Runnells, J. Contamin. Hydrology, 9, 35 (1992)

Boyd, S.A., Soil Sci. 134, 337 (1982)

Brown, D.S. and E.W. Flagg, J. Environ. Qual. 10, 382 (1981)

Chin, Y.P; C.S. Peven, and W.J. Weber, Jr., Water Res. 22, 873 (1988)

Garbini, D.R. and L.W. Lion, Environ. Sci. Technol. 19, 1122 (1985)

Hassett, J.P. and M.A. Anderson, Water Res. 16, 681 (1982)

Karickhoff, S.W.; D.S. Brown, and T.A. Scott, Water Res. 13, 241 (1979)

Lee, L.S., P.S.C. Rao, and M.L. Brusseau, Environ. Sci. Technol. 25, 722 (1991)

Rippen, G.M., W. Ilgestein, W. Klopffer, and H.J. Poremski, Ecotoxicol. Environ. Saf. 6, 236 (1982)

Russell, D.J. and B. McDuffe, Chemosphere, 15, 1003 (1986)

Walters, R.W., S.A. Ostazewski, and A. Guiseppi, Environ. Sci. Technol. 23 480 (1989)

Table 8:  Criteria for Flagging Data Entry Errors and Significant Outliers

| Property Code | Project ESP Description | Error Analysis |
| --- | --- | --- |
| Block 1 | Oxygen Demand |  |
| 1a | Biochemical O2 Demand (BOD) | 1a <= 1cn |
| 1b & 1bp | Dichromate Chemical O2 Demand (COD) Permanganate Chemical O2 Demand (COD) | 1b <= 1cc 1bp <= 1b |
| 1cc & 1cn | Theoretical O2 Demand, Carbonaceous Theoretical O2 Demand, Combined | 1cc <= 1cn |
| Block 2 | Partitioning Parameters |  |
| 2a | 1-Octanol/Water Partitioning | Log Kow vs. Log Koc Scatter Plot |
| 2c | Organic Carbon/Water Partitioning | Log Koc vs. Log Kow Scatter Plot |
| 2d | Bioconcentration Factor | > ±1 logarithmic unit |
| Block 3 | General Physical Properties |  |
| 3a | Molecular Weight | > 5% |
| 3b | Liquid Density at 298.15 K (±5 K) | > 10% |
| 3d | Melting Point | > 10% |
| 3e | Normal Boiling Point | > 10% |
| 3f | Vapor Pressure at 298.15 K (±5 K) | > 50% |
| 3h | Molecular Diffusivity in Air | > 30% |
| 3i | Molecular Diffusivity in Water | > 30% |
| 3l | Surface Tension at 298.15 K (±5 K) | > 10% |
| 3n | Enthalpy of Formation at 298.15 K (±5 K) | > 20% |
| 3q | Critical Temperature | > 10% |
| 3r | Critical Pressure | > 10% |
| 3s | Critical Volume | > 10% |
| 3t | Enthalpy of Vaporization at 298.15 K | > 20% |
| Block 5 | Fire and Explosion Parameters |  |
| 5al | Lower Flammability Limit in Air | > 25% |
| 5au | Upper Flammability Limit in Air | > 50% |
| 5b | Flash Point | > 10% |
| 5c | Autoignition Temperature | > 15% |
| 5d | Enthalpy of Combustion at 298.15 K | > 10% |
| Block 6 | Sensory, Health and Toxicity Impacts |  |
| 6.. | Acute Aquatic Toxicity | > 50% |

Table 9:  Evaluation Criteria for Block 3

- Score

| Criteria | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Experimental Conditions | Not stated | Stated without range | Stated with range |
| Purity of Chemicals | Not stated | Stated with no range or as received with range | Stated with range, purified, or calculated |
| Experimental Technique | Not stated or Not acceptable | Stated briefly | Described in detail |
| Reported Accuracy and/or Internal Precision- QA/QC Procedures | Not stated or >20% | Stated with 5-20% | Stated with 1-5% |
| Qualitative Agreement with Other Work | Does not agree | Agrees | Agrees well with other work |

Table 10:  Evaluation Criteria for Block 6

* Essential Information

Score

| Criteria | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Experimental Conditions Temperature and DT* Animal Loading* Volume Additions* Water Chemistry* Water Source* Photo-period Food before experiment | Essentials not all reported or secondary source. | Reported as given elsewhere (other source); standard techniques named but few essentials reported; non-standard technique with all conditions reported. | Standard technique with all essentials listed. |
| Purity of Chemicals Source of chemical* Verification of experimental conc.* Purity of chemical from source reported | Not stated or secondary source. | Source listed or concentration verified. | Sources reported and concentrations verified. |
| Experimental Technique (Standard) technique reported or described* Number of replicates reported* Random assignment* Age, size, and supplier of organism rptd* Concentration of chemical verified throughout experiment (24 intervals)* Control used when chemical is dissolved in solvent. | Not reported or standard method without details or secondary source. | Standard method listed with incomplete report of essentials; non-standard technique with full report of essentials. | Standard method with full report. |
| Accuracy/Precision Statistical tests named & appropriate* No pseudo-replication* Confidence limits reported* | 1 significant figure with 20% confidence or secondary source. | 2 significant figures and 5-20% confidence with essentials. | 2 significant figures and 1-5% confidence with essentials. |
| Agreement with others |  | Agrees with own work. | Agrees with other's work or reports comparisons. |

Table 11:  Translation of DIPPR® 801 Evaluation Codes to DIPPR® ESP Evaluation Codes

| Type of Data | Definition | Points Allocated |
| --- | --- | --- |
| X | Experimental | 1 |
| P | Predicted | Rating will be blank |
| D | Defined | Rating will be blank |
| S | Smoothed | Rating will be blank |
| ? | Unknown | Rating will be blank |
| T | Extrapolated | Rating will be blank |
| Nature of Data Source |  |  |
| E | Evaluated Source | 1 |
| U | Unevaluated Source | 0 |
| "blank" | Predicted, Interpolated, or derived from the definition by the project staff | 0 |
| Reliability of Data |  |  |
| "blank" | Predicted or derived at PSU | 0 |
| 0 | Unknown Accuracy | 0 |
| 1 | < 0.2% Error | 2 |
| 2 | < 1% Error | 2 |
| 3 | < 3% Error | 2 |
| 4 | < 5% Error | 1 |
| 5 | < 10% Error | 1 |
| 6 | < 25% Error | 0 |
| 7 | < 50% Error | 0 |
| 8 | < 100% Error | 0 |
| 9 | > 100% Error | 0 |

- Table 12: Results of SQC Review

| Physical Property | Number of Values Evaluated | Number and Type of Error AF / DE / AD | Level 1 Tolerance Value | Quality Code |
| --- | --- | --- | --- | --- |
| Molecular Weight | 5669 | 5 / 16 / 1 | + 5% | Q1 |
| Melting Point | 3891 | 88 / 20 / 5 | + 10% | Q1 |
| Normal Boiling Point | 5628 | 20 / 34 / 10 | + 10% | Q1 |
| Diffusivity in Air | 1282 | 8 / 1 / 0 | + 30% | Q1 |
| Diffusivity in Water | 1141 | 63 / 4 / 1 | + 30% | Q1 |
| Heat of Formation | 1510 | 173/ 50 / 21 | + 20% | Q1 |
| Critical Temperature | 3469 | 22 / 5 / 0 | + 10% | Q1 |
| Flash Point | 3256 | 73 / 121/ 25 | + 10% | Q1 |
| Heat of Combustion | 1285 | 76 / 10 / 9 | + 10% | Q1 |
| TOTAL | 27,131 | 508 / 261 / 72 |  |  |
|  |  | AF2 / DE / AD |  |  |
| Melting Point less than Normal Boiling Point | 3635 | 56 / 7 / 1 |  | Q2 |

- AF:	Anomaly Flagged, Level 1; value is correct as transcribed from literature source

AF2:	Anomaly Flagged, Level 2; value is correct as transcribed from literature source

- DE:	Data Entry Error

AD:	Anomaly, Rating Dropped; highest rated value is transcribed correctly from literature source, but does not agree with multiple other literature values.

- Table 13:  Structure for Values Table

| Field Name | Purpose | Field Type and Size |
| --- | --- | --- |
| C# | Chemical Number | N* |
| Property Code | Property Code | A3* |
| Article # | Citation Number | N* |
| Rating | Quality Rating | N |
| Desc/Method | Experimental Method or 801 Quality Code | A30 |
| Keywords | Keywords | A30 |
| Value | Value or Lower Temperature Limit | N |
| Units | Units | A11 |
| Temperature | Experimental Conditions or Upper Temperature Limit | A13 |
| Equation | Correlation Equation Number | A3 |
| Coef1 | Coefficient for Equation or Lower Confidence Limit (Block 6) | N |
| Coef2 | Coefficient for Equation or Upper Confidence Limit (Block 6) | N |
| Coef3 | Coefficient for Equation or Hardness (Block 6) | N |
| Coef4 | Coefficient for Equation or pH (Block 6) | N |
| Coef5 | Coefficient for Equation | N |
| Ref2 | Project 801 Second Reference | N |
| Ref3 | Project 801 Third Reference | N |
| FootN | Project 801 Footnote Code | A2 |
| Pressure | Experimental Conditions or Life stage of Organism (Block 6)*** | A13 |
| Comment | Comment | A120 |
| Fixed |  |  |
| Anomaly |  |  |
| Field1 |  |  |

* Key Field

Table 14:  Structure of Citation Table

| Field Name | Purpose | Field Type and Size |
| --- | --- | --- |
| Paper # | Data Source Number | N* |
| Title | Title | A120 |
| Author | Author | A120 |
| Journal | Journal | A120 |
| Date | Date of Publication | A15 |
| Volume | Volume | A15 |
| Number | Journal Number | A15 |
| Pages | Page Number | A15 |
| Pcode1 | Property code for tracking | A3 |
| Pcode2 | Property code for tracking | A3 |
| Pcode3 | Property code for tracking | A3 |
| Pcode4 | Property code for tracking | A3 |
| Pcode5 | Property code for tracking | A3 |
| Pcode6 | Property code for tracking | A3 |
| Date Assigned | Date for tracking | D |
| Comments |  |  |

* Key Field

Table 15:  Structure of Property Table

| Field Name | Purpose | Field Type and Size |
| --- | --- | --- |
| Property Code | Property Code | A3 |
| Description | Property Name | A45 |
| Reviewer | Technical Reviewer | A30 |
| Default Units | Default Units for the Property | A11 |
| T. Dep. | Temperature Dependent? | A1 |
| Comment |  |  |

Table 16:  Structure of Chemical Table

| Field Name | Purpose | Field Type and Size |
| --- | --- | --- |
| C# | Project ESP Chemical Number | N* |
| CAS# | Chemical Abstract Number | N |
| Description | Clean Air Act Name | A50 |
| Molecular Formula | Molecular Formula | A25 |
| Structural Formula | Structural Formula | A25 |
| SMILES Strings |  |  |
| Common Name | Common Name | A50 |
| IUPAC Name |  |  |
| Chem. Abstracts Name | CAS Name | A50 |
| Family Group (see Table 19) | Chemical Family Group | A2 |

* Key Field

Table 17:  Structure of Convunit Table

| Field Name | Purpose | Field Type and Size |
| --- | --- | --- |
| From Units | Units from the database (VALUES Table) | A11* |
| To Units | Default Units or Unit Options | A11* |
| Multiplier Property | Multiplier Property | A3 |
| Multiplier | Multiplier | N |
| Addend | Addend | N |
| Mult Property Flag | Multiplier Property Flag | A1 |
| Comment | States for which properties units are used | A20 |

* Key Field

Table 18:  Project 912 Estimation Methods Included in Project ESP Software Product

| PROPERTY (CODE) | ESTIMATION TECHNIQUE |
| --- | --- |
| Biochemical Oxygen Demand (BOD) (1a) | None |
| Chemical Oxygen Demand (COD) (1b) | COD = ThOD for 14 chemical classes. |
| Theoretical Oxygen Demand, Carbonaceous (ThOD) (1cc) | Baker (1994; corrected, 1999) Baker (1994) |
| Theoretical Oxygen Demand, Combined (ThOD) (1cn) | Baker (1994) |
| Octanol Water Partitioning (Log Kow ) (2a) | UNIFAC (Fredenslund, 1975) with choice of interaction parameter set ENV- Environmental Parameter Set (Rogers, 1994) GBL- Global (Rogers, 1994) VLE- Vapor Liquid Parameter Set (Hansen, 1991) LLE- Liquid Liquid Parameter Set (Gemhling, 1982) Kenga-Goring (1980) Hansch (1968) |
| Organic Carbon Water Partitioning Log(Koc) (2c) | Baker et al. (1997) |
| Bioconcentration Factor (2d) | Kobayashi (1981) Kenaga and Goring (1980) |
| Molecular Weight (3a) | Estimate from addition of UNIFAC chemical fragment groups, if necessary |
| Liquid Density (3b) | Bhirud (Bhirud, 1978) Modified Racket equation (Spencer, 1972), using critical properties. |
| Solubility in Water (3c) | Yalkowsky and Mishra (1990) UNIFAC (See above) |
| Solubility of Water in Chemical | UNIFAC (See above) |
| Melting Point (3d) | Taft and Starek (1930: Horvath, 1992) Lorenz and Hertz (1922: Horvath 1992) |
| Molecular Diffusivity in Air (3h) | Wilke-Lee (1955) |
| Molecular Diffusivity in Water (3i) | Hayduk and Minhas (1982). Hayduk and Laudie (1974). Polson (1950) Wilkie and Chang (1955). |
| Surface Tension (3l) | Brock and Bird (Danner, 1983) |
| Heat of Vaporization at 25 (3t) | Watson (1931; Reid, et al., 1986) |
| Heat of Vaporization at NBP | Klein (1949: Lyman, 1982) Chen (1965) Watson (1931; Reid, et al., 1986) |
| Activity Coeff. Of Water in Chemical (4b) | UNIFAC (See above) |
| Henry's Law Constant (4c) | UNIFAC (See above) |
| Upper and Lower Flammability Limit in Air (5a and 5b) | MTU Group Contribution Method (Pintar, 1999) MTU Combustion Reaction Method (Pintar, 1999) Penn State Group Contribution Method (Danner, 1986) |

- Table 19:  Family Grouping Codes

AA  Normal Alkanes

MA   Methylalkanes

DA   Dimethylalkanes

OA   Other alkanes

CA   Cycloalkanes

GA   Alkylcyclopentanes

HA   Alkylcylohexanes

RA   Multiring cycloalkanes

AE   1-Alkenes

BE   2, 3, or 4-Alkenes

ME   Methylalkenes

EE   Ethyl and higher alkenes

CE   Cycloalkenes

DE   Dialkenes

YY   Alkynes

AR   Normal alkylbenzenes

BR   Other alkylbenzenes

MR   Other monoaromatics

DR   Naphthalenes

OR   Other condensed rings

PR   Diphenyl/polyaromatics

TR   Terpenes

HR   Other hydrocarbon rings

IG   Inorganic gases

LD   Aldehydes

KK   Ketones

AL   Normal alcohols

OL   Other aliphatic alcohols

CL   Cycloaliphatic alcohols

RL   Aromatic alcohols

PL   Polyols

AC   Normal aliphatic acids

OC   Other aliphatic acids

DC   Dicarboxylic acids

RC   Aromatic carboxylic acids

HD   Anhydrides

FS   Formates

ES   Acetates

BS   Propionates and butyrates

SS   Other saturated aliphatic esters

US   Unsaturated aliphatic esters

RS   Aromatic esters

AT   Aliphatic ethers

OT   Other ethers/diethers

CT   Epoxides

TT   Peroxides

AH   C1/C2 Aliphatic chlorides

HH   C3 and higher aliphatic chloride

RH   Aromatic chlorides

VH   C,H,Br Compounds

WH   C,H,I Compounds

FH   C,H,F Compounds

PH   C,H, Multihalogen compounds

AM   Normal aliphatic primary amines

BM   Other aliphatic amines

RM   Aromatic amines

OM   Other amines, imines

NX   Nitriles

TN   C,H,NO2 Compounds

YN   Isocyanates/diisocyanates

DZ   Diazoalkanes

SD   Mercaptans

SF   Sulfides/thiophenes

SU   Sultones

PC   Polyfunctional acids

PS   Polyfunctional esters

PO   Other polyfunctional C,H,O

NP   Polyfunctional nitriles

TM   Nitroamines

PM   Polyfunctional amides/amines

PN   Polyfunctional C,H,O,N

SP   Polyfunctional C,H,O,S

HP   Polyfunctional C,H,O,halide

BP   Polyfunctional C,H,N,halide,(O)

OP   Other polyfunctional organics

LX   Elements

SX   Silanes/siloxanes

GI   Organic/inorganic compounds

IC   Inorganic acids

IB   Inorganic bases

GS   Organic salts

XS   Sodium salts

OS   Other inorganic salts

IH   Inorganic halides

OI   Other inorganics

- Figure 1:  DIPPR® ESP Tracking Form

- Figure 2:  Citation Formats

- Books

(complete - not a compilation)

Author	: Author or Editor(s)

Title	: Book Title

Journal	: Publisher and City, State

Volume	: If given

Number	: Chapter or Subheading

Pages	: Starting Page #

Date	: Year

- Books

(compilation)

Author	: Chapter Author(s)

Title	: Title of Chapter

Journal	: Book Title, Editor(s), Publisher, Place of Publication

Volume	: If given

Number	: Chapter

Pages	: Starting Page #

Date	: Year

- Industrial Reports

Author	: Company and Division

Title	: Title (or subject); if sales literature note such in parentheses

Journal	: Company Address

Volume	:

Number	: Report Number (if given)

Pages	:

Date	: Date

- Figure 3:  Data Entry Form

| Article # |  |
| --- | --- |
| Rating |  |

| C # | P. Code | Value | Units | Temp. | Press. | Comment |
| --- | --- | --- | --- | --- | --- | --- |

Figure 4:  Acute Aquatic Toxicity Entry Format

- Values Table Fields

C #			DIPPR chemical number

Property Code		DIPPR property code

Article #		DIPPR article number

Rating			quality code assigned by reviewer (0-10; 0 if predicted, 1 if secondary)

Desc/Method		experiment method*

Keywords		species tested	For toxicity, note species here if it is a Salmonidae or Mysidacea)

Value			toxicity or BCF value from article

Units					mg/L for toxicity; unit-less for BCF;  If author reports as log of a value but does not

report the original units, then record the log value, leave units blank, and provide a note

in comment.

Temperature		test temp. or temp. range, °C If a temp. range is given, put median in "Temperature;" include 			range as a ±... in "Comment.")

Equation		unused field

Coef1			lower 95% confidence limit

Coef2			upper 95% confidence limit

Coef3			pH (If pH is a range, put median in "Coef3;" include range as a ±... in "Comment.")

Coef4			hardness (mg/L as CaCO3)  (If hardness is a range, put median in "Coef4;" include range as a 			±... in "Comment.")

Coef5			unused field

Ref2			unused field

Ref3			unused field

FootN			unused field

Pressure		life stage of organism**

Comment			additional pertinent details  (For:  < or > EC50, LC50, or BCF values; data which do

not fit in other fields; ±values or ranges of temperature, pH, hardness.  For BCFs,

include exposure duration in "Comment."  Note when data at other test conditions are

available.  Note whether QSARs are available.  For secondary sources, list the reference

for the data).  Example--describe coefficients 1 and 2 if they are not the lower and upper

confidence limits; report hardness as hard or soft if author reports this but no values are given.

* Standard methods are abbreviated:

APHA = American Public Health Association	USEPA = United States Environmental Protection Agency

ASTM = American Society for Testing and Materials	AWWA = American Water Works Association

OECD = Organization for Economic Cooperation and Development 	WPCF = Water Pollution Control Federation

If a standard method was used, follow method abbreviation with a hyphen and an abbreviation of the exposure system:

f = flow-through			fc = flow-through, covered system to limit volatilization

s = static			sc = static, covered vessel to limit volatilization

sr = static with renewal		src = static with renewal, covered vessel to limit volatilization

r = recirculation			rc = recirculation, covered system to limit volatilization

Example: EPA-s  means the author used the U.S. Environmental Protection Agency method in a static water system.

** Pressure field is used for the organism's life stage information (weight, length, age).  Use the following abbreviations:

g = gram		mm = millimeter		y = years

mg = milligram		cm = centimeter		mo = months

ug = microgram					w = weeks

d = days

If the author uses a term rather than an age, note the term in comments (ex: juvenile; larval; smolt).

Conversions to mg/L				              from mol/L, multiply by molecular weight and by 1000

from g/m3, no change					from mmol/L, multiply by molecular weight

from g/L, multiply by 1000				from German dH° hardness to mg/L CaCO3, divide by .056

from ppm (parts per million), no change

from ppb (parts per billion), divide by 1000

Salmonidacea (salmon and trout family)					Mysidacea (Opossum shrimp order)

scientific name			common name				scientific name

Salmo salar			Atlantic salmon				Mysidopsis almyra

Salmo trutta			Brown trout				Mysidopsis bahia

Salmo trutta lacustris		Lake trout				Mysidopsis bigelowi

Salvelinus alpinus		Arctic char (or charr)			Mysis relicta

Salvelinus fontinalis		Brook trout				Neomysis americana

Salvelinus leucomaenis		White spotted char (or charr)		Neomysis awatschensis

Salvelinus malma			Dolly varden				Neomysis integer

Salvelinus namaycush		Lake trout				Neomysis sp.***

Siscowet					Leptomysis lingvura

Salvelinus sp.***			Trout, charr				Leptomysis mediterranea

Oncorhynchus aguabonita		Golden trout

Oncorhynchus clarki		Cutthroat trout

Oncorhynchus gilae		Gila trout

Oncorhynchus gorbuscha		Pink salmon

Oncorhynchus keta		Chum salmon

Oncorhynchus kisutch		Coho salmon

Silver salmon

Oncorhynchus masou		Cherry salmon

Yamame trout

Oncorhynchus mykiss		Rainbow trout

or Salmo Gairdneri	Donaldson trout

Oncorhynchus nerka		Sockeye salmon

Oncorhynchus tshawytscha	Chinook salmon

*** sp. = species, any species of the given genus.